3GPP TSG-RAN WG2 Meeting #108	R2-1915209

USA, 18th – 22th, November 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | draftCR | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TS 38.306 CR on UE capabilities for NR_SON_MDT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT |  |  |  |  |  | Date: |  |  | 2019-11-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the previous RAN meetings, there are many good progresses on MDT, L2 measurements and SON in NR. UE capability is used as one of the criteria for the network to perform UE selection. It is straightforward to define the MDT related UE capabilites as in LTE. This CR captures UE Capability parameters based on the agreements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Capture UE capabilities for SON and MDT: - loggedMeasurements - rach-Report - standaloneGNSS-Location - loggedMeasBT, loggedMeasWLAN, immMeasBT, immMeasWLAN - barometerMeasReport, speedMeasReport, orientationMeasReport - ulPDCP-Delay - Mobility history information storage Impact analysis Impacted functionality: Logged MDT and Immdeidate MDT. Inter-operability: This CR is for UE and network to implement and the proposed parameters are included in a message sent from UE to the network. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The network does not know if the UE supports the features of MDT when the network perform UE selection for MDT. The network may request UEs which do not support the desired features to perform MDT. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2, 5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 4.2	UE Capability Parameters

<partially omitted>

4.2.14	RRC buffer size

The RRC buffer size is defined as the maximum overall RRC configuration size that the UE is required to store. The RRC buffer size is 45Kbytes.

4.2.x	SON parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| rach-Report Indicate whether the UE supports delivery of rachReport upon request from the network. | UE | No | No | No |

4.2.y	UE-based network performance measurement parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| barometerMeasReport This parameter indicates whether UE supports uncompensated Barometeric pressure measurement reporting. | UE | No | No | No |
| immMeasBT This parameter indicates whether the UE supports Bluetooth measurements in RRC_CONNECTED state. | UE | No | No | No |
| immMeasWLAN This parameter indicates whether the UE supports WLAN measurements in RRC_CONNECTED state. | UE | No | No | No |
| loggedMeasBT This parameter indicates whether the UE supports Bluetooth measurements in RRC_IDLE or RRC_INACTIVE state. | UE | No | No | No |
| loggedMeasurements This parameter defines whether the UE supports logged measurements in RRC_IDLE and RRC_INACTIVE upon request from the network. A UE that supports logged measurements shall support both periodical logging and event-triggered logging. And a UE that supports logged measurements in RRC_IDLE or RRC_INACTIVE shall also support a minimum of 64kB memory for log storage. | UE | No | No | No |
| loggedMeasWLAN This parameter indicates whether the UE supports WLAN measurements in RRC_IDLE or RRC_INACTIVE state. | UE | No | No | No |
| orientationMeasReport This parameter indicates whether the UE supports orientation measurements reporting. | UE | No | No | No |
| speedMeasReport This parameter indicates whether the UE supports speed measurements reporting. | UE | No | No | No |
| standaloneGNSS-Location This parameter defines whether the UE is equipped with a standalone GNSS receiver that may be used to provide detailed location information in RRC measurement report and logged measurements in RRC_IDLE or RRC_INACTIVE, RLF report and accessibility measurement report. | UE | No | No | No |
| ulPDCP-Delay This parameter indicates whether the UE supports pdcp delay measurements in RRC_CONNECTED state. | UE | No | No | No |

5	Optional features without UE radio access capability parameters

Definitions for feature

SU-MIMO Interference Mitigation advanced receiver - R-ML (reduced complexity ML) receivers with enhanced inter-stream interference suppression for SU-MIMO transmissions with rank 2 with 2 RX antennas - R-ML (reduced complexity ML) receivers with enhanced inter-stream interference suppression for SU-MIMO transmissions with rank 2, 3, and 4 with 4 RX antennas UE supporting the feature is required to meet the Enhanced Receiver Type requirements in TS 38.101-4 [18].

Mobility history information storage It is optional for UE to support the storage of mobility history information and the reporting in UEInformationResponse message as specified in TS 38.331 [9].