3GPP TSG-RAN WG2 #108	R2-1915372

Reno, USA, 18th – 22nd November 2019	(Revision of R2-1912542)

Agenda Item:	6.10.2

Source:	Ericsson

Title:	NR-DC power control

Document for:	Discussion, Decision

# Introduction

In Rel-15 late drop, only cell group independent power control was supported for NR-DC, since MCG operated only in FR1 and SCG in FR2. For Rel-16, the discussion is still ongoing in RAN1. So far RAN1 has agreed the support for semi-static power sharing for NR-DC [4]. Thus, there will be a need to signal the Pmax values per cell group. Existing signalling supports this for the FR1 range, but not for the FR2 range. Similarly, inter node messages need to be updated to support power coordination across cell groups in the FR2 range. RAN1 furthermore agreed the support for different modes of power control, which may also require signalling support. In this contribution, we study the impact on RAN2 signalling.

# Discussion

## 2.1 RAN1 agreements

In RAN1#98bis, the following agreements were made:

- Adopt Alt.1-2 and Alt.2 for semi-static power sharing for NR-NR DC.

  - Alt.1-2 is only subject to configured maximum transmission power defined by RAN4

  - Configuration between Alt.1-2 and Alt.2 is supported.

  - FFS: add more clarification

  - FFS: applied for synchronous DC only or applied for both synchronous and asynchronous DC (which may be the same or different for Alt.1-2 and Alt. 2)

- For semi-static power sharing for NR-NR DC, Virtual PHR for active CCs of another CG

- For dynamic power sharing for NR-NR DC (if supported), Virtual PHR for active CCs of another CG

- Support dynamic power sharing

  - If there is no overlapping transmission, maximum power on CG i is determined by RAN4 spec without considering P_CG_i.

  - If there is overlapping transmission, maximum power on CG i is limited to P_CG_i.

  - Note: “look-ahead” operation is included as a UE capability below

- In case of power limitation, MCG is prioritized over SCG and reuse CA rule within each CG

- Optional UE capability to indicate the support of dynamic power sharing operation

- Separate optional UE capability to indicate the support of ’look-ahead’ operation on condition that UE indicates support of dynamic power sharing operation.

## 2.2 Power configuration

We start by revisiting the parameters used in Rel-15 to control the transmit power for NR cell groups. In NR RRC, the following fields in PhysicalCellGroupConfig where used to configure the transmit power for FR1:

p-NR-FR1 The maximum total transmit power to be used by the UE in this NR cell group across all serving cells in frequency range 1 (FR1). The maximum transmit power that the UE may use may be additionally limited by p-Max (configured in FrequencyInfoUL) and by p-UE-FR1 (configured total for all serving cells operating on FR1).

p-UE-FR1 The maximum total transmit power to be used by the UE across all serving cells in frequency range 1 (FR1) across all cell groups. The maximum transmit power that the UE may use may be additionally limited by p-Max (configured in FrequencyInfoUL) and by p-NR-FR1 (configured for the cell group).

These were used together with p-MaxEUTRA on the E-UTRA side to limit the max power for NR and E-UTRA cell groups respectively.

- By setting p-NR-FR1 and p-MaxEUTRA such that the sum is smaller or equal to the total allowed UE transmit power for FR1, NR and E-UTRA are allocated fixed shares of the available power within which it can operate. This is sometimes referred to as semi-static power sharing.

- Setting p-NR-FR1 and p-MaxEUTRA such that the sum is larger than the total allowed UE transmit power for FR1 is only supported for UEs supporting dynamic power sharing, based on UE capability. With dynamic power sharing, when the UE tx power exceeds maximum allowed transmit power for FR1, the UE will scale down or drop the transmit power for the SCG.

In EN-DC, the possibility to configure max power for E-UTRA and NR cell groups and per UE were defined.

In EN-DC, semi-static power sharing was mandatory supported. Dynamic power sharing was supported based on UE capability.

For Rel-16, the discussion on NR-DC with regards to the support of FR1 and FR2 in MCG and SCG is still ongoing in RAN1 [1][2][3]. Based on the discussion so far, configurations with FR2 in both MCG and SCG are not excluded. Therefore, there is a need to add corresponding fields to configure the max transmit power across all serving cells operating in FR2 within each cell group, e.g.:

p-NR-FR2 The maximum total transmit power to be used by the UE in this NR cell group across all serving cells in frequency range 2 (FR2). The maximum transmit power that the UE may use may be additionally limited by p-Max (configured in FrequencyInfoUL) and by p-UE-FR2 (configured total for all serving cells operating on FR2).

p-UE-FR2 The maximum total transmit power to be used by the UE across all serving cells in frequency range 2 (FR2) across all cell groups. The maximum transmit power that the UE may use may be additionally limited by p-Max (configured in FrequencyInfoUL) and by p-NR-FR2 (configured for the cell group).

p-NR-FR2 and p-UE-FR2 are added in PhysicalCellGroupConfig to configure the max transmit power for serving cells operating in FR2.

## Power coordination

In addition to the fields used to configure the UE, there are also fields defined in inter-node RRC messages to enable the power coordination between MCG and SCG. In Rel-15, the following fields were used in configRestrictInfoSCG of message CG-ConfigInfo by the MN to inform SN of the maximum transmission power allowed per cell group and per UE on FR1:

powerCoordination-FR1               SEQUENCE {

p-maxNR-FR1                     P-Max                                OPTIONAL,

p-maxEUTRA                      P-Max                                OPTIONAL,

p-maxUE-FR1                     P-Max                                OPTIONAL

}                                                               OPTIONAL,

p-maxEUTRA Indicates the maximum total transmit power to be used by the UE in the E-UTRA cell group (see TS 36.104 [33]).

p-maxNR-FR1 Indicates the maximum total transmit power to be used by the UE in the NR cell group across all serving cells in frequency range 1 (FR1) (see TS 38.104 [12]) the UE can use in NR SCG.

p-maxUE-FR1 Indicates the maximum total transmit power to be used by the UE across all serving cells in frequency range 1 (FR1).

powerCoordination-FR1 Indicates the maximum power that the UE can use in FR1.

If the SN determines a need to increase its transmit power beyond the allocated maximum transmit power, it can request a change using the field requestedP-MaxFR1 in configRestrictModReq of message CG-Config.

requestedP-MaxFR1 Requested value for the maximum power for the serving cells on frequency range 1 (FR1) in this secondary cell group (see TS 38.104 [12]) the UE can use in NR SCG.

For Rel-16 NR-DC where both MCG and SCG operate in FR2, corresponding power control parameters are needed to support power coordination between MCG and SCG for carriers operating in FR2. Since FR2 transmission is not supported for E-UTRA, there is no need for an E-UTRA field.

powerCoordination-FR2               SEQUENCE {

p-maxNR-FR2                     P-Max                                OPTIONAL,

p-maxUE-FR2                     P-Max                                OPTIONAL

}                                                               OPTIONAL,

p-maxNR-FR2 Indicates the maximum total transmit power to be used by the UE in the NR cell group across all serving cells in frequency range 2 (FR2) (see TS 38.104 [12]) the UE can use in NR SCG.

p-maxUE-FR2 Indicates the maximum total transmit power to be used by the UE across all serving cells in frequency range 2 (FR2).

powerCoordination-FR2 Indicates the maximum power that the UE can use in FR2.

p-maxNR-FR2 and p-maxUE-FR2 are added in ConfigRestictInfoSCG to support power coordination between MCG and SCG for FR2 carriers.

Correspondingly, a new field requestedP-MaxFR2 is needed in e.g. ConfigRestricModReqSCG to allow SN to request a different share of the maximum allowed transmit power on FR2.

requestedP-MaxFR2               P-Max                                       OPTIONAL,

requestedP-MaxFR2 Requested value for the maximum power for the serving cells on frequency range 2 (FR2) in this secondary cell group (see TS 38.104 [12]) the UE can use in NR SCG.

requestedP-MaxFR2 is added in ConfigRestictModReqSCG to support the possibility for SN to request a different share of the FR2 transmit power.

# Conclusion

In this contribution we made the following observations:

Observation 1	In EN-DC, the possibility to configure max power for E-UTRA and NR cell groups and per UE were defined.

Observation 2	In EN-DC, semi-static power sharing was mandatory supported. Dynamic power sharing was supported based on UE capability.

Based on the discussion we propose the following:

Proposal 1	p-NR-FR2 and p-UE-FR2 are added in PhysicalCellGroupConfig to configure the max transmit power for serving cells operating in FR2.

Proposal 2	p-maxNR-FR2 and p-maxUE-FR2 are added in ConfigRestictInfoSCG to support power coordination between MCG and SCG for FR2 carriers.

Proposal 3	requestedP-MaxFR2 is added in ConfigRestictModReqSCG to support the possibility for SN to request a different share of the FR2 transmit power.

# References

R1-1907731, Feature lead summary for Uplink power control for supporting NN-DC, Intel, RAN1#97, Reno, USA 13th – 17th May 2019

R1-1909619, Feature lead summary for Uplink Power Control for Supporting NN-DC, Apple, RAN1#98, Prague, Czech Republic, 26th-30th August 2019

R1-1911455, Feature lead summary #1 for uplink power control for NR-NR DC, RAN1#98bis, Chongqing, China, 14th-20th October, 2019

RAN1 chairman’s notes, RAN1#98bis, Chongqing, China, October 14th – 20th, 2019