3GPP TSG-RAN WG2 Meeting #108	R2-1915386

Reno,  USA, 18 – 22 November 2019

Agenda item:	6.18.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on PCI list

WID/SID:	NG_RAN_PRN-Core - Release 16

Document for:	Discussion and Decision

# 1	Introduction

RAN agreed that the Non-Public Networks (NPN) related RAN work will be carried out within NG_RAN_PRN work item [1]. At RAN2#107bis the following agreements were made related PCI lists:

Agreements:

- the UE can optionally implement an autonomous search function of CAG cells. FFS on the relationship with dedicated priorities.

- reserving a PCI range for CAG cells is purely a deployment issue (does not need to be reflected in the spec)

- the PCI list of CAG cells can optionally be signalled to UEs. FFS on details of the list

- There is no autonomous search function of SNPN cells.

- reserving a PCI range for SNPN cells is purely a deployment issue (does not need to be reflected in the spec)

- FFS whether PCI range of SNPN cells can optionally be signalled to UEs.

This paper discusses the details on the PCI list that can be provided for NPNs..

# 2	Discussion

In LTE to assist the autonomous search function on mixed carriers, all CSG cells on mixed carriers broadcast in system information a range of PCI values reserved by the network for use by CSG cells and optionally also non-CSG cells on the mixed carrier can send this information in system information (see clause 10.5.1.1 of TS 36.300 [2]).

During the discussion of R2-1912843 (Report of Email Discussion [107#41] on CSG Aspects) it was discussed if similar functionality is needed for CAG and SNPN cells. It was left open (agreement f)) if PCI range of SNPN cells can be signalled to UEs.

According to the agreement d) no autonomous search function may be implemented UEs for SNPN cells. Therefore, there is no need to signal of PCI range to UEs to assist autonomous search. To help UEs to avoid cells that do not belong to the registered network there is a "blacklist" in SIB3, SIB4 and SIB 5 in Rel-15 NR. Moreover, it was agreed in RAN2#107 during the discussion of unlicensed NR to introduce a "whitelist" to help UEs to avoid cells that do not belong to the registered network. Our conclusion is that there are already mechanisms to signal PCI lists to the UE, and there is no need to introduce any additional mechanism in SNPNs.

Proposal 1: No new signalling is introduced to signal PCI range of SNPN cells to UEs.

# 3	Conclusions

We have made the following proposal in this paper:

Proposal 1: No new signalling is introduced to signal PCI range of SNPN cells to UEs.

# References

[1]	RP-1900729: "New Work Item Proposal on Private Network Support for NG-RAN"

[2]	3GPP TS 23.501, System Architecture for the 5G System; Stage 2 (Release 16)