3GPP TSG-RAN3 Meeting #108	R2-1915849

Reno, Nevada, USA, 18th – 22nd Nov 2019

Title:	[Draft] LS to RAN3 on conditional PSCell addition/change

Response to:

Release:	Rel-16

Work Item:	NR_Mob_enh-Core

Source:	Huawei (to be RAN2)

To:	RAN3

CC:

Contact Person:

Name:	Chen Jun

Email Address:	jun.chen@huawei.com

Attachments:

1. Overall Description:

RAN2 have discussed conditional PSCell addition/change, and agreed to support both MN and SN initiated conditional PSCell addition/change. RAN2 agreements are as below:

Agreements

- Maintain Rel-15 principle that only one PScell is active at a time even with conditional PScell addition/change.

2	For conditional PScell addition, the MN decides on the conditional PScell addition execution condition. The condition is defined by a measurement identity, given by a measurement configuration provided by the MN.

3	For conditional PScell change, execution condition may be decided by MN (MN-initiated) or SN (SN-initiated)

4	For conditional PScell change, A3/A5 execution condition should be supported while for conditional PScell addition, A4/B1 like execution condition should be supported.

5	For conditional SN change, the source SN configuration can be used as the reference in generation of delta signalling for the candidate SNs.

2. Actions:

To RAN3:

ACTION: 	RAN2 respectfully ask RAN3 to take the above agreements into consideration and perform appropriate RAN3 specification work to support conditional PSCell addition/change.

3. Date of Next TSG-RAN WG2 Meetings:

RAN2#109		24th – 28th February 2020			Athens, Greece

RAN2#109b		20th – 24th April 2020			TBD, TBD