3GPP TSG RAN WG2 Meeting #108	R2-1916090

Reno, USA, 18th November – 22th November 2019

Agenda Item:	  6.4.6 L2/3 protocols for QoS support

Source: 	ITRI

Title: 	Remaining issue on Groupcast and Broadcast QoS Report

Document for:	Discussion and Decision

Introduction

The RAN2#107 meeting specified the email discussion [1] on QoS flows in SLRB for the discussion on [2]

- QoS information report by UE in RRC connected state

- The QoS flow to SLRB mapping in dedicated RRC signaling

- The PC5 QoS parameter to SLRB mapping in SIB

- The PC5 QoS parameter to SLRB mapping in pre-configuration

While the QoS parameter details have not been confirmed in the current discussion, this contribution proposed remaining issues for the L2/L3 QoS support in NR.

Discussion

  - QoS report and SLRB configuration

An exemplary procedure figure for the UE reporting PC5 QoS parameters and the corresponding SLRB to QoS mapping is provided in [2]:

While enabling UE reporting the PC5 QoS parameters is necessary for the NG-RAN to know about the QoS parameters of each QoS flow, the way of reporting on a per QoS flow per destination basis may influenced the complexity and overhead of the UE report and RRC configuration message. This is especially crucial for the 1-M transmission such as groupcast and broadcast, because 1 QoS flow may include several packets delivered to several different destinations, and thus the reporting on a per QoS flow per destination basis can be implemented by different methods.

The NG-RAN provides the mapping of PC5 QoS flow to SLRB based on the QoS information reported by the UE. The UE will derive the PC5 QoS parameters for the PC5 QoS flow and assign a PFI for the PC5 QoS flow. Because only per-flow QoS model is supported in NR V2X [3], the unicast, groupcast, and broadcast operation should be supported with such model. The unicast case is much simpler in view of QoS report in view that each unicast link belongs to one QoS flow and it has its own distinct destination. Therefore, the focus of QoS report should be on the groupcast and broadcast. The one-to-many characteristic of transmission enables one source node transmitting data to multiple destination, within only one QoS flow. (It can be arguable if the groupcast or groupcast is configured as multiple unicast links.)

While there is limited time remained for the Release 16 NR V2X standardization, the QoS support for all the casting types should be detailed and tailored to fit the future V2X applications and services. The PC5 QoS parameters reporting, with standardized and non-standardized PQI, can be limited to PQI, PC5 flow bit rates, PC5 LINK-AMBR, range, or default values, but the casting type specific QoS parameters might be considered in the later stage, trying to capture the requirement of next-step V2X or 5G D2D.

- Proposal: 	Consider the PC5 QoS parameter report per QoS flow per destination for the groupcast and broadcast. Include necessary parameters specific for 1-M casting types.

Conclusion

This contribution provides the following and proposal:

- Proposal 	Consider the PC5 QoS parameter report per QoS flow per destination for the groupcast and broadcast. Include necessary parameters specific for 1-M casting types.

References

- Draft RAN2 107 Report v1

- Report of [107#74] [NR/V2X] QoS flows in SLRB

- S2-1904823, LS response to SA2 on unicast, groupcast and broadcast in NR sidelink, SA2