3GPP TSG-RAN WG2 Meeting #108	R2-1916092

Reno,  USA, 18 – 22 November 2019

Agenda item:	6.2.3.5

Source:	Charter Communications

Title:	On NR-U Operation in DFS Channels

WID/SID:	NR-U WID - Release 16

Document for:	Discussion and Decision

# 1	Introduction

Operation in unlicensed spectrums requires conformance to local regulatory rules. In NR-Unlicensed WID [1], the operational unlicensed spectrum is defined to be the 5GHz spectrum, which also includes the spectrum of a few incumbent technologies. Particularly, in portions of 5GHz spectrum, various radars operate where the unlicensed RAT devices are required to protect them if the device happen to detect the presence of the radar signals.

In this contribution, we highlight issues related to operation in portions of 5GHz spectrum that is subject to such requirements.

# 2	Background on DFS regulatory requirements

5GHz spectrum is available worldwide for unlicensed use. Various unlicensed technologies such as IEEE’s 802.11, 3GPP’s LTE LAA/eLAA/FeLAA, and 3GPP’s NR-U employ use of this spectrum. In particular, in USA, portions of this spectrum is shared with Federal radar systems (e.g. weather Doppler radars or TWDR). Also, portions of 5GHz spectrum is allocated to radar operation worldwide [2].

Dynamic frequency selection (DFS) is a mechanism to allow (outdoor) unlicensed devices to operate in 5GHz frequency bands (which have been allocated to radar systems) without causing interference to radars operating in the channel [3]. A DFS-enabled device monitors the channel it operates at and if radar signals detected, the device will vacate that channel and switch to an alternate channel automatically. In addition, the channel which radar signals are detected will not be used for a period of time.

The channels subject to DFS for operation of LAA and NR-U devices are channels within 46B and 46C [4,5], where 46B ranges from 5250MHz to 5350MHz and 46C ranges from 5470MHz to 5725MHz. Corresponding channel(s) for NR are expected to be defined as part of the ongoing NR-U WID [1]. A full list of channels subject to DFS requirement worldwide is reported in NR-U SI report [2]. Note that the portion of the spectrum that requires DFS capability is more than half of the total 5GHz unlicensed spectrum in North America, and it becomes even a bigger portion in some other countries.  Appendix A includes a table of the sub-bands of 5GHz unlicensed spectrum [3] and an illustration of the sub-bands affected by DFS requirement in North America [2].

According to regulatory requirement, a master device (e.g. a gNB, an eNB or a WiFi access point) does the radar detection on behalf of all the devices that have associated with it (e.g. UEs or Wi-Fi stations). If the master device detects presence of radar signals, it’d administer that all the associated client devices vacate the channel subject to the radar signal presence. If a client device loses connection to the master device, it shall not transmit anything to gain access to its master node or another master node in the same channel (or another channel that requires DFS capability). The communications between master node and clients has to be sufficiently fast to meet the requirement that the device may only transmit for a total of Channel Closing Transmission Time during a period of Channel Move Time after detection of a radar signal. In summary, channel move time is less than 10 seconds, but the normal communication between the master and slave devices should be less than 200ms [2].

# 3	NR-U operation in DFS channels

NR operation in unlicensed spectrum is subject to LBT requirement [1]. However, LBT mechanism, while is required in the entire 5GHz spectrum, is not effective in detecting radar signals which are narrowband in frequency and appear in pulses [3]. The radar detection mechanism is a more effective mechanism to detect and protect radar signals. Regardless of effectiveness or ineffectiveness of LBT in detection of radar signals, the regulatory requirements are what NR-U devices have to follow which require implementation of DFS procedure.

It is noteworthy, that DFS requirement is applicable for each technology operating in the radar channels in 5GHz spectrum, e.g. IEEE 802.11, 3GPP LTE LAA, and 3GPP NR-U. In IEEE 802.11, there are mechanisms in place to perform above-described mechanism. In LTE LAA, since operation in unlicensed spectrum is limited to SCell, an eNB has the flexibility to deactivate an SCell if radar signals are detected in the channel of the operation of the SCell. For most of NR-U deployment scenarios, a gNB may do similarly to what an eNB does in case of radar detection. However, in NR-U standalone deployment, where all PSell and SCell(s) operate in unlicensed spectrum, the gNB may be required to do more in case of radar signal detection.

Observation 1: For NR-U deployment scenarios where PCell operates in licensed spectrum, a gNB has the flexibility to deactivate an SCell if radar signals are detected in the channel of the operation of the SCell. However, in NR-U standalone deployment, where all PSell and SCell(s) operate in unlicensed spectrum, the gNB may be required to do more in case of radar signal detection.

It is noteworthy that while the motivation of this topic is due to DFS regulatory requirement in 5GHz unlicensed spectrum, any mechanism that RAN2 proposes may also be used when a gNB wants to change its operational channel due to changes in channel usage by competing devices (e.g. NR-U/LAA/WiFi nodes). While such mechanism is unnecessary in licensed operation, in NR-U operation and particularly standalone deployment, competition to use the free unlicensed spectrum may cause high density of unlicensed devices, e.g. in urban environments or crowded venues such as malls and airports. Hence such capability helps a gNB to continuously evaluate and choose the best channel for NR-U operation and transfer its associated UEs smoothly to a less crowded channel.

Because NR-U is a synchronous scheduled system, an NR-U UE cannot initiate transmissions without the knowledge of gNB. Therefore, it is sufficient to assume that detection of radar signals is only performed at the gNB. It then becomes gNB’s responsibility to convey the expected behaviour (after radar detection) to UEs under its control.

### 3.1 UEs in RRC_Connected

For UEs in RRC_CONNECTED state, the normal handover mechanism may be used to provide the necessary information to the UE to move the transmission/reception to channels without radar signal presence. Depending on the number of UEs in RRC_Connected modes and whether the gNB gets to act promptly, there may be room for enhancements. Given the late stage of NR Release-16 we suggest that a gNB subject to detected radar signal to use handover mechanism to direct UEs, in RRC connected mode, to the new operating channel.

Proposal 1: In the event of radar signal detection in DFS channels, the gNB uses handover mechanism to direct RRC_Connected UEs to the new operating channel.

### 3.2 UEs in RRC_Idle and RRC_Inactive

No mechanism currently exists to inform UEs in RRC_IDLE or RRC_INACTIVE of events, such as radar detection, and direct the UEs to a new channel that the gNB is moving to. Without specifically informing UEs, in Idle and Inactive modes, know about the channel that the gNB is moving to, the UEs end up going through cell reselection procedure.

For instance, cell-barring mechanisms can be used to force UEs to go through cell reselection. Upon detection of radar in one of the DFS channels that the gNB operates, the gNB sets the following:

- in the MIB cellBarred from barred to notBarred

- in the MIB intraFreqReselection to notAllowed

- in the SIB1 trackingAreaCode to null i.e. remove the field

Upon above setting, UEs in RRC_IDLE or RRC_INACTIVE mode which read the changed MIB values will be forced to perform cell reselection. And upon the expiration of a gNB configurable time termed GracefulShutdownOfTransmissions the gNB stops all transmission in X.

Above solution is not ideal, since it forces UEs to do a full cell reselection while it is possible to avoid cell reselection. For instance, given that a gNB, subject to detected radar signals, knows what channel is moving to, and if right signalling mechanism is provided can let the UEs know what channel to move to. However, given the late stage of NR Release-16 we suggest to consider MIB cell-barring to be used in the event of radar signal detection in DFS channels.

Proposal 2: In the event of radar signal detection in DFS channels, the gNB uses MIB cell-barring mechanism to force RRC_Idle and RRC_Inactive UEs to perform cell reselection.

Proposal 3: Endorse Proposal 1 and Proposal 2 for NR-U operation in DFS channels in Relese-16.

# 4	Conclusions

In this contribution, we highlight issues related to operation in portions of 5GHz spectrum that is subject to requirements beyond LBT and particular to incumbent technologies such as weather radars. We have observed the following:

Observation 1: For NR-U deployment scenarios where PCell operates in licensed spectrum, a gNB has the flexibility to deactivate an SCell if radar signals are detected in the channel of the operation of the SCell. However, in NR-U standalone deployment, where all PSell and SCell(s) operate in unlicensed spectrum, the gNB may be required to do more in case of radar signal detection.

Proposal 1: In the event of radar signal detection in DFS channels, the gNB uses handover mechanism to direct RRC-Connected UEs to the new operating channel.

Proposal 2: In the event of radar signal detection in DFS channels, the gNB uses MIB cell-barring mechanism to force RRC_Idle and RRC_Inactive UEs to perform cell reselection.

Proposal 3: Endorse Proposal 1 and Proposal 2 for NR-U operation in DFS channels in Relese-16.

# References

RP-182878, “New WID on NR-based Access to Unlicensed Spectrum,” Qualcomm, RAN#82, December 2018

3GPP TR 38.889, Study on NR-based Access to Unlicensed Spectrum (Release 16)

Spectrum Sharing in the 5 GHz Band DFS Best Practices, The Wi-Fi Alliance Spectrum & Regulatory Committee, Spectrum Sharing Task Group, and Regulatory Task Group. URL: http://www.ieee802.org/18/Meeting_documents/2007_Nov/WFA-DFS-Best%20Practices.pdf

3GPP TS 36.104: “E-UTRA Base Station (BS) radio transmission and reception (Release 13)

3GPP TS 38.104: NR Base Station (BS) radio transmission and reception (Release 16)

# Appendix A: 5GHz Unlicensed Spectrum

Table 1: Sub-band for Band 46 (5GHz) [4]

Figure 1: Summary of existing and proposed new FCC part 15 rules for 5GHz unlicensed spectrum usage [2]