3GPP TSG-RAN WG2 Meeting #108	R2-1916274

Reno, NV, USA, Nov 18th – 22nd 2019

Agenda Item:	6.9.3.1 (NR_mob_enh)

Work Item(s):	LTE_feMob, NR_mob_enh

Source:	Charter Communications

Title:	On Maximum Number of CHO Candidate Cells and Nodes

Document for: Discussion and Agreement

# Introduction

A new WI [1] for even further Mobility enhancement in E-UTRAN was approved at RAN#80. The WI scope covers the following:

- Specify further enhancements to achieve following targets, [RAN2/3]

  - reduce user data interruption during handover, which targets as close as possible to 0ms, i.e. relaxed requirements could be considered.

  - improve the robustness during handover,

- Specify necessary core requirements for the identified solutions [RAN4]

This work has been split into the following phases:

- Study Phase, to evaluate the proposed solutions, e.g. simultaneous connectivity with both source and target eNB, conditional handover and enhancements to make-before-break, including support of carrier aggregation in source and carrier aggregation in target eNB during handover, and do down selection or merger, if necessary.

- Work Phase, to specify the chosen solution(s)

The following was agreed for LTE Conditional HO (CHO) at RAN2#104 [2]:

Agreements

1	Support configuration of one or more candidate cells for conditional handover.

=>	FFS how many candidate cells (UE and network impacts should be clarified).

In this contribution, we focus on the issue of number of candidate cells, and candidate nodes for CHO, and provide a way forward.

# Discussion

At RAN2#105, contributions in [3], [4], [5], [6], [7], [8] discussed the issue of number of candidate cells to be supported for CHO. Whilst the authors of this paper (we) acknowledge that technical complexities are of paramount importance when making design decisions on CHO, we’d like to highlight the usage of this CHO feature from a practical stand point.

Development of LTE specifications was centered around Macro eNBs. Although, HeNB support was added in later releases of LTE, given the challenges (e.g. cost, business models etc) associated with small cells, its deployment didn’t really achieve the expected penetration rate.

With developments such as inclusion of FR2 as a valid frequency range for deployment of 3GPP’s 5G NR technology, FCC’s R&O [9] for opening of 3.5GHz CBRS spectrum in USA which allows indoor (Cat A CBSDs) and outdoor (Cat B CBSDs) mode of service operation etc, small cell-based deployments are either already becoming or are soon to be a reality. An LTE network which uses a combination of macro nodes together w/ small cells or a small-cell only network (for regional deployments) is expected to benefit the most from CHO.

The readers may already be aware that neighbor cell relationship for macro nodes looks drastically different to that of small cell nodes. The density of target users adds further layer of complexity e.g. a deployment targeted for a downtown area is expected to be denser (more neighbor cells) than one which targets sub-urban or rural areas.

Observation 1: The number of target cells for CHO will vary depending on the type of targeted deployment (e.g. sparse v/s dense area).

Our initial radio planning for Multi Dwelling Units (MDUs ala low/mid/high-rise residential or enterprise buildings) in extremely dense areas such as downtown NewYork City (NY, USA) reveals that the number of candidate target cells (assuming omnidirectional cell for indoor small-cell eNBs, and omnidirectional cells for outdoor small-cell eNBs) could range from 8-20. Whereas, the number of candidate target cells for a sub-urban/rural macro-based deployment may be no more than 2-6.

Observation 2: The number of target (small-)cells for CHO for urban/sub-urban macro- deployment could be from 2-6, whereas for dense urban small cell-deployment could be up to 20.

We do acknowledge that various technical aspects, such as the ones mentioned below, associated with supporting candidate target cells for CHO need to be taken into considerations:

- How are the candidate target cells communicated to the UE? E.g. in a single message, or one message per target cell etc

- How are updates to the candidate targets cells done? E.g. multiple single messages, delta messages etc

- How often are such updates expected once CHO starts?

- How to balance resource reservation against cell’s performance?

- etc…

We understand that resolution of above questions is required. However, we assert that more important than the “hows” is the question of “why”? Which ties in with usability of this feature. An LTE small-cell deployment is likely to derive more benefit from CHO feature than a macro-cell based deployment.

To that extent, the following options have been proposed thus far:

- Support multiple candidate cells but only within a single candidate target node e.g. in [11]

- Support multiple candidate cells with a number ranging from 1 through 6 e.g. in [3], [10], [16], [17]

- Support multiple candidate cells but leave the exact number to network implementation e.g. in [13]

- Support multiple candidate cells but determine the exact number during Stage 3 e.g. in [7], [12]

/a/ whilst the simplest of the options would discourage deployment of CHO itself because it offers a limiting solution (not helpful for deployments w/ LTE small-cell type); Ergo, negating the value of a feature which has good potential to make a real difference to deployments.

/d/ whilst a sound approach from an engineering perspective lacks a design goal. This carries the risk of making it a subject of technical limitations imposed by selected solutions/mechanisms.

/c/ does offers flexibility to different operators based on their needs. However, it is no different to /d/ as it keeps the CHO feature a slave to protocol limitations e.g. if RAN2 agrees that only a single CHO command is allowed towards a UE, and such a command contains “full” configuration (not “delta” configuration), then it will implicitly limit the number of candidates cells any network implementation can support.

Various proposals in /b/ seem to offer a good balance between performance and feasibility. In [17] it was shown that even for a larger (>8) candidate target cells the PDCP limit of 9k isn’t exceeded. The authors understand that the higher the number of candidate cells the greater is the X2/RRC signaling overhead, target resource reservation burden, and also the UE memory and processing overhead. However, a network with small cell centric deployment (especially in dense urban deployments) will need 6-8 CHO candidate cells at the minimum. In the past, we had proposed number of candidate cells to be a number between 8-20 with a max set to 16. Then, at a subsequent meeting we proposed it to be set to 8. We understand that to support such a high number of candidate cells, although is a requirement from us, may result in rather complex UE operations. Given that, as a compromise solution we propose to set, as a design target, the maximum number of candidate cells for CHO for this release (Rel-16) be 6. Supporting a CHO candidate list of 2 or 3 would make CHO feature undeployable for us.

Conclusion 1: 	The design target for maximum number of candidate target cells to be supported for Rel-16 LTE CHO should be set to 6.

In previous meetings, there has also been a question on whether multiple candidate eNBs/gNBs for CHO are allowed? Figure 1 below depicts a sample of our dense urban strand-mounted deployment. In such a deployment, omni-directional small cell base stations (eNB or gNB) hang off a strand located on a single side of a street. Taking B1 (either an eNB or gNB) as the reference point, the ISD can be in the range of 150m-300m. This provides a neighbor node relationship to a node in the front (B2), and a node in the back (B3). Along with residential units (single equipment/CPE homes), Multi-dwelling units (MDUs) are expected along either side of the streets. MDU1 will have small cell nodes hanging off from at least each corner of the building denoted by B4, B5 etc. Here it is assumed that these are on the ground floor. Same is the case with MDU2 with B6, B7 etc. Whilst small cells are also expected along the corners of MDUs along each floor or every other floor (depending on factors such as the building material, O2I penetration, desired coverage etc), it is not assumed that B1 has neighbor node relationship with base stations on 4th, 5th and higher floors. As can be seen, in the most basic assumption of 2 MDUs (one along each side of the street), a given node will have at minimum 6 target neighbor nodes to which the UE may be CHO’ed to. Coupled with conclusion 1, and assuming omni-directional small cells, our requirement is to support atmost 6 candidate target nodes for LTE CHO.

Figure 1 Omni-directional small cell strand-mounted deployment for dense urban case

Conclusion 2: 	The design target for maximum number of candidate target nodes to be supported for Rel-16 LTE CHO should be set to 6.

Seeing that NR Mobility Enhancements work [15] is in close alignment with this WID, and given that NR FR1 operating range is the same as that for LTE, the above conclusion should also be applied to NR.

Given the nature of higher frequencies involved in NR FR2, the expected coverage range of NR FR2 gNBs is expected to be smaller compared to NR FR1 gNBs. Therefore, it can be safely assumed that the number of candidate targets (cells and nodes) required would be greater than LTE or NR FR1 range. [17] showed that configuration of up to 32 targets can be supported. Assuming we leave room for growth of target configuration container, supporting up to 8 for now should be sufficient.

To do so, the following two options exist:

- In 38.331, keep a single maximum allowed value in maxNrofCHO-Cells. Set the maximum value to 8.

- In 38.331, introduce two maximum allowed values. One for maxNrofCHOForFR1-Cells NR FR1, another one  maxNrofCHOForFR2-Cells for NR FR2. Set maxNrofCHOForFR1-Cells to 6. Set to maxNrofCHOForFR2-Cells to 8.

Disadvantage of Option /2/ is introduction of NTN (“FR3”), and frequencies >52.6GHz (“FR4”) in Rel-17 would require addition of more frequency-range specific maximum values for CHO targets. Hence, option /1/ is recommended.

Conclusion 3: 	The design target for maximum number of candidate target cells and nodes to be supported for Rel-16 NR CHO should be set to 8.

Conclusion 4:  If deemed necessary, send LS to RAN3 informing of this decision.

# Conclusions

The following proposals are provided in this contribution:

Observation 1: The number of target cells for CHO will vary depending on the type of targeted deployment (e.g. sparse v/s dense area).

Observation 2: The number of target (small-)cells for CHO for urban/sub-urban macro- deployment could be from 2-6, whereas for dense urban small cell-deployment could be up to 20.

Conclusion 1: 	The design target for maximum number of candidate target cells to be supported for Rel-16 LTE CHO should be set to 6.

Conclusion 2: 	The design target for maximum number of candidate target nodes to be supported for Rel-16 LTE CHO should be set to 6.

Conclusion 3: 	The design target for maximum number of candidate target cells and nodes to be supported for Rel-16 NR CHO should be set to 8.

Conclusion 4:  If deemed necessary, send LS to RAN3 informing of this decision.

# References

- RP-181475 New Work Item on even further Mobility enhancement in E-UTRAN

- R2-1818528, “Report from break-out session on Rel-16 NR Industrial IoT SID and Rel-16 LTE even further mobility enhancements WID”, Nokia

- R2-1900276, “Support of One or More Candidate Cells for Conditional HO”, CATT

- R2-1900511, “Discussion on the number of prepared cells for CHO”, vivo

- R2-1900924, “Number of Prepared Cells in Conditional Handover”, ETRI

- R2-1900988, “On Supporting Conditional HO”, Samsung

- R2-1901082, “Further details of conditional handover in LTE”, Ericsson

- R2-1901378, “Configuration of CHO”, China Telecom

- FCC 15-47, REPORT AND ORDER AND SECOND FURTHER NOTICE OF PROPOSED RULEMAKING

- R2-1903435, “Discussion on the number of prepared cells for CHO”, vivo

- R2-1905136, Consideration of multiple candidate target cell for LTE CHO”, Samsung

- R2-1903884, “Configuration of Conditional handover in LTE“, Ericsson

- R2-1904343, “Remaining Issues of LTE Conditional Handover”, CMCC

- void

- RP‑190489, New WID: NR mobility enhancements

- R2-1912736, On Maximum number of CHO candidate cells and nodes, Charter Communications

- R2-1912739, Discussion on configuration aspect for CHO, Huawei, HiSilicon, China Telecom

- R2-1914834, NR mobility running CR 38 331 for CHO -v02

Small cell refers to either micro, pico, or femto cells