3GPP TSG-RAN WG2 Meeting #108	R2-1916337

Reno, USA, 18 – 22 November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | Num | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for Non-Public Networks |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia (Rapporteur) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RAN_PRN |  |  |  |  |  | Date: |  |  | 2019-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR introduces the support for Non-Public Networks in NG-RAN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To be completed once the CR is stable. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No Stage 2 description of NPN for NG-RAN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | TBD |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Modified Subclause

## 3.1	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], in TS 36.300 [2] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] and TS 36.300 [2].

5GC	5G Core Network

5QI	5G QoS Identifier

A-CSI	Aperiodic CSI

AKA	Authentication and Key Agreement

AMBR	Aggregate Maximum Bit Rate

AMC	Adaptive Modulation and Coding

AMF	Access and Mobility Management Function

ARP	Allocation and Retention Priority

BA	Bandwidth Adaptation

BCH	Broadcast Channel

BPSK	Binary Phase Shift Keying

C-RNTI	Cell RNTI

CAG	Closed Access Group

CBRA	Contention Based Random Access

CCE	Control Channel Element

CD-SSB	Cell Defining SSB

CFRA	Contention Free Random Access

CMAS	Commercial Mobile Alert Service

CORESET	Control Resource Set

DFT	Discrete Fourier Transform

DCI	Downlink Control Information

DL-SCH	Downlink Shared Channel

DMRS	Demodulation Reference Signal

DRX	Discontinuous Reception

ETWS	Earthquake and Tsunami Warning System

GFBR	Guaranteed Flow Bit Rate

HRNN	Human-Readable Network Name

I-RNTI	Inactive RNTI

INT-RNTI	Interruption RNTI

LDPC	Low Density Parity Check

MDBV	Maximum Data Burst Volume

MIB	Master Information Block

MICO	Mobile Initiated Connection Only

MFBR	Maximum Flow Bit Rate

MMTEL	Multimedia telephony

MNO	Mobile Network Operator

MU-MIMO	Multi User MIMO

NCGI	NR Cell Global Identifier

NCR	Neighbour Cell Relation

NCRT	Neighbour Cell Relation Table

NGAP	NG Application Protocol

NID	Network Identifier

NPN	Non-Public Network

NR	NR Radio Access

P-RNTI	Paging RNTI

PCH	Paging Channel

PCI	Physical Cell Identifier

PDCCH	Physical Downlink Control Channel

PDSCH	Physical Downlink Shared Channel

PNI-NPN	Public Network Integrated NPN

PO	Paging Occasion

PRACH	Physical Random Access Channel

PRB	Physical Resource Block

PRG	Precoding Resource block Group

PSS	Primary Synchronisation Signal

PUCCH	Physical Uplink Control Channel

PUSCH	Physical Uplink Shared Channel

PWS	Public Warning System

QAM	Quadrature Amplitude Modulation

QFI	QoS Flow ID

QPSK	Quadrature Phase Shift Keying

RA-RNTI	Random Access RNTI

RACH	Random Access Channel

RANAC	RAN-based Notification Area Code

REG	Resource Element Group

RMSI	Remaining Minimum SI

RNA	RAN-based Notification Area

RNAU	RAN-based Notification Area Update

RNTI	Radio Network Temporary Identifier

RQA	Reflective QoS Attribute

RQoS	Reflective Quality of Service

RS	Reference Signal

RSRP	Reference Signal Received Power

RSRQ	Reference Signal Received Quality

SD	Slice Differentiator

SDAP	Service Data Adaptation Protocol

SFI-RNTI	Slot Format Indication RNTI

SIB	System Information Block

SI-RNTI	System Information RNTI

SLA	Service Level Agreement

SMC	Security Mode Command

SMF	Session Management Function

SNPN	Stand-alone Non-Public Network

S-NSSAI	Single Network Slice Selection Assistance Information

SPS	Semi-Persistent Scheduling

SR	Scheduling Request

SRS	Sounding Reference Signal

SS	Synchronization Signal

SSB	SS/PBCH block

SSS	Secondary Synchronisation Signal

SST	Slice/Service Type

SU-MIMO	Single User MIMO

SUL	Supplementary Uplink

TA	Timing Advance

TPC	Transmit Power Control

UCI	Uplink Control Information

UL-SCH	Uplink Shared Channel

UPF	User Plane Function

URLLC	Ultra-Reliable and Low Latency Communications

Xn-C	Xn-Control plane

Xn-U	Xn-User plane

XnAP	Xn Application Protocol

## 3.2	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1], in TS 36.300 [2] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1] and TS 36.300 [2].

CAG Cell: a cell broadcasting at least one Closed Access Group identity.

CAG Member Cell: for a UE, a cell broadcasting the identity of the selected PLMN, registered PLMN or equivalent PLMN for which the Allowed CAG list of the UE includes an entry comprising the CAG identifier of the cell.

Editor’s Note: definition of CAG-only cell should be added here once agreed in 38.304.

Cell-Defining SSB: an SSB with an RMSI associated.

CORESET#0: the control resource set for at least SIB1 scheduling, can be configured either via MIB or via dedicated RRC signalling.

gNB: node providing NR user plane and control plane protocol terminations towards the UE, and connected via the NG interface to the 5GC.

Intra-system Handover: Handover that does not involve a CN change (EPC or 5GC).

Inter-system Handover: Handover that involves a CN change (EPC or 5GC).

MSG1: preamble transmission of the random access procedure.

MSG3: first scheduled transmission of the random access procedure.

ng-eNB: node providing E-UTRA user plane and control plane protocol terminations towards the UE, and connected via the NG interface to the 5GC.

NG-C: control plane interface between NG-RAN and 5GC.

NG-U: user plane interface between NG-RAN and 5GC.

NG-RAN node: either a gNB or an ng-eNB.

Numerology: corresponds to one subcarrier spacing in the frequency domain. By scaling a reference subcarrier spacing by an integer N, different numerologies can be defined.

SNPN Access Mode: mode of operation whereby a UE only accesses SNPNs.

Editor’s Note: definition of SNPN-only cell should be added here once agreed in 38.304.

Xn: network interface between NG-RAN nodes.

Next Modified Subclause

## 4.6	Radio Access Network Sharing

NG-RAN supports radio access network sharing as defined in TS 23.501 [3].

If NR access is shared, system information broadcast in a shared cell indicates a TAC and a Cell Identity for each subset of PLMNs (up to 12). NR access provides only one TAC and one Cell Identity per cell per PLMN.

Each Cell Identity associated with a subset of PLMNs identifies its serving NG-RAN node.

Editor’s note: describe the general principles of RAN sharing between NPNs and between PLMNs and NPNs based on 3GPP TS 23.501 [RAN2, RAN3].

Next Modified Subclause (new)

## 4.x	Non-Public Networks

A Non-Public Network (NPN) is a network for non-public use (see 3GPP TS 22.261 [19]), which can be deployed as (see 3GPP TS 23.501 [3]):

-	a Stand-alone Non-Public Network (SNPN) when not relying on network functions provided by a PLMN; or

-	a Public Network Integrated (PNI) NPN when relying on the support of an existing PLMN.

Next Modified Subclause

## 8.2	Network Identities

The following identities are used in NG-RAN for identifying a specific network entity:

-	AMF Name: used to identify an AMF.

-	NR Cell Global Identifier (NCGI): used to identify NR cells globally. The NCGI is constructed from the PLMN identity the cell belongs to and the NR Cell Identity (NCI) of the cell.

-	gNB Identifier (gNB ID): used to identify gNBs within a PLMN. The gNB ID is contained within the NCI of its cells.

-	Global gNB ID: used to identify gNBs globally. The Global gNB ID is constructed from the PLMN identity the gNB belongs to and the gNB ID. The MCC and MNC are the same as included in the NCGI.

-	Tracking Area identity (TAI): used to identify tracking areas. The TAI is constructed from the PLMN identity the tracking area belongs to and the TAC (Tracking Area Code) of the Tracking Area.

-	Single Network Slice Selection Assistance information (S-NSSAI): identifies a network slice.

-	Network Identifier (NID): identifies an SNPN in combination with a PLMN ID.

-	Closed Access Group Identifier: identifies a CAG within a PLMN.

Next Modified Subclause

## 9.4	Roaming and Access Restrictions

The roaming and access restriction information for a UE includes information on restrictions to be applied for subsequent mobility action during CM-CONNECTED state. It may be provided by the AMF and also may be updated by the AMF later.

It includes the forbidden RAT, the forbidden area and the service area restrictions as specified in TS 23.501 [3]. It also includes serving PLMN and may include a list of equivalent PLMNs.

Upon receiving the roaming and access restriction information for a UE, if applicable, the gNB should use it to determine whether to apply restriction handling for subsequent mobility action, e.g., handover, redirection.

If the roaming and access restriction information is not available for a UE at the gNB, the gNB shall consider that there is no restriction for subsequent mobility actions.

Only if received over NG or Xn signalling, the roaming and access restriction information shall be propagated over Xn by the source gNB during Xn handover. If the Xn handover results in a change of serving PLMN (to an equivalent PLMN), the source gNB shall replace the serving PLMN with the identity of the target PLMN and move the serving PLMN to the equivalent PLMN list, before propagating the roaming and access restriction information.

Editor’s note: describe the impacts of NPNs on roaming and access restrictions [RAN2].

Next Modified Subclause

## 16.x	Stand-Alone NPN

### 16.x.1	General

A SNPN is a network deployed for non-public use which does not rely on network functions provided by a PLMN (see subclause 4.x). An SNPN is identified by a PLMN ID and NID (see subclause 8.2) broadcast in SIB1.

An SNPN-capable UE supports the SNPN access mode. When the UE is set to operate in SNPN access mode the UE only selects and registers with SNPNs.

Emergency services are not supported in SNPN.

### 16.x.2	Mobility

#### 16.x.2.1	Principles

#### The same principles as described in 9.2 apply to SNPN except for what is described in the subclauses below.

#### 16.x.2.2	Idle Mode

UEs operating in SNPN access mode only (re)select cells within the selected SNPN i.e. a cell is only considered as suitable if the PLMN and NID broadcast by the cell matches the selected SNPN.

Editor’s note: broadcast of a range of PCI values reserved by the network for use by SNPN cells is FFS.

In addition, manual selection of SNPN(s) is supported, for which HRNN(s) can be optionally provided.

Access attempts to SNPN-only cells are restricted to SNPN subscribers.

#### 16.x.2.3	Inactive Mode

Editor’s note: describe mobility in INACTIVE [RAN2].

#### 16.x.2.4	Connected Mode

Editor’s note: describe mobility in CONNECTED [RAN2]. Impacts to Xn/NG procedures including data forwarding [RAN3].

## 16.y	Public Network Integrated NPN

### 16.y.1	General

A PNI-NPN is a network deployed for non-public use which relies on network functions provided by a PLMN (see subclause 4.x). In PNI-NPN, a Closed Access Groups (CAG) identifies a group of subscribers who are permitted to access one or more CAG cells associated to the CAG. A CAG is identified by a CAG identifier broadcast in SIB1.

A CAG-capable UE can be configured with the following per PLMN (see clause 5.30.3.3 of TS 23.501 [3]):

-	an Allowed CAG list containing the CAG identifers the UE is allowed to access; and

-	a CAG-only indication if the UE is only allowed to access 5GS via CAG cells.

### 16.y.2	Mobility

#### 16.y.2.1	Principles

The same principles as described in 9.2 apply to CAG cells except for what is described in the subclauses below.

#### 16.y.2.2	Idle Mode

Cell selection/reselection to CAG cells may be based on a UE autonomous search function, which determines itself when/where to search.

Editor’s note: relationship between the autonomous search function and dedicated priorities is FFS.

A range of PCI values reserved by the network for use by CAG cells maybe broadcast.

A UE checks the suitability of CAG cells based on the Allowed CAG list provided by upper layers.

#### In addition, manual selection of CAG cell(s) is supported, for which an HRNN(s) can be optionally provided.

Access attempts to CAG-only cells are restricted as described in Table 16.y.2.2-2 below:

Table 16.y.2.2: Access Attempts to CAG-only cells

| UE type | CAG Member Cell | Service | Access Attempts |
| --- | --- | --- | --- |
| Not CAG capable No CAG functionality (Rel-15 UE) | - | Emergency | FFS |
|  |  | Normal | Not Allowed |
| Not CAG capable but can identify CAG cells from broadcast information | - | Emergency | Allowed |
|  |  | Normal | Not Allowed |
| CAG capable | No | Emergency | Allowed |
|  |  | Normal | Not Allowed |
|  | Yes | Emergency | Allowed |
|  |  | Normal | Allowed |

Editor’s note: this table is only a starting point and will be updated once the UE types are settled.

#### 16.y.2.3	Inactive Mode

Editor’s note: describe mobility in INACTIVE [RAN2].

#### 16.y.2.4	Connected Mode

Editor’s note: FFS is proximity indicator is needed.

Editor’s note: describe mobility in CONNECTED [RAN2]. Impacts to Xn/NG procedures including data forwarding [RAN3].

Annex A (informative):
WG Agreements

This temporary annex lists all agreements reached in RAN2 and RAN3 and will be removed in the final version.

## RAN2#107

Agreements

1	The SNPNs (identified by PLMN ID + NID) are broadcasted in SIB1,

FFS whether this is achieved by extending the legacy network list or by introducing a new SNPN specific network list or both.

2	The size and format of the NID will not be discussed in RAN2 (we will be informed by other groups)

3	Up to 12 different SNPNs can be broadcasted in a cell.

4	If “mixed” network sharing is allowed (i.e. a cell can contain both PLMNs and NPNs), the total number of networks indicated in SIB1 (i.e. #PLMN + #SNPN + #PNI-NPN) shall not exceed 12.

5	If HRNN are broadcast then the HRNN should a be broadcasted in a separate SIB (i.e. different from SIB1).

6	SNPN selection functions similar to normal PLMN selection: AS reports the found SNPNs (identified by PLMN ID + NID) to NAS which selects the network. In case of manual selection, the human readable network name (if broadcasted) may also be provided from AS to NAS.

7	Once the UE has selected an SNPN, cell selection/re-selection is only performed within the SNPN, i.e. a cell is only considered suitable if the broadcasted SNPN identifier matches the selected SNPN.

Agreements

1	The PNI-NPNs (identified by PLMN ID + CAG ID) are broadcasted in SIB1

FFS whether this is achieved by extending the legacy network list or by introducing a new PNI-NPN specific network list or both

2	The size and format of the CAG ID will not be discussed in RAN2 (we will be informed by other groups)

3	Up to 12 different PNI-NPNs can be broadcasted in a cell.

4	If HRNN are broadcast then the HRNN should a be broadcasted in a separate SIB (i.e. different from SIB1).

5	Network selection is triggered by NAS whereby AS reports the available PNI-NPNs (identified by PLMN ID + CAG ID) to NAS which selects the network to use. In case of manual network selection, the human readable network name (if broadcasted) may also be provided from AS to NAS.

6	The Allowed CAG list and “CAG only” indication received from upper layers are taken into account in the cell suitability check during cell selection/re-reselection.

Agreements

1	There is no issue identified to support E1 for Rel-16 UEs.

2	(Regarding question E2) Rel-16 UEs not supporting the CAG feature can camp on a CAG cell as an acceptable cell to obtain limited service

3	There is no issue identified to support RS1 for Rel-16 UEs

4	RS2 and RS3 can be supported from RAN2 point of view

## RAN2#107bis

Agreements:

- no new mechanism is introduced to handle the priority of a frequency layer of a CAG cell on which the UE is camped (beyond what cellReselectionPriority provides in SIB4 and in RRCRelease).

- the UE can optionally implement an autonomous search function of CAG cells. FFS on the relationship with dedicated priorities.

- reserving a PCI range for CAG cells is purely a deployment issue (does not need to be reflected in the spec)

- the PCI list of CAG cells can optionally be signalled to UEs. FFS on details of the list

- FFS whether proximity indication in CONNECTED mode is needed

- no preliminary access check for CAG cells in CONNECTED mode. The Allowed CAG list is provided to the gNB by the AMF.

- no new mechanism is introduced to handle the priority of a frequency layer of an SNPN cell on which the UE is camped (beyond what cellReselectionPriority provides in SIB4 and in RRCRelease).

- There is no autonomous search function of SNPN cells.

- reserving a PCI range for SNPN cells is purely a deployment issue (does not need to be reflected in the spec)

- FFS whether PCI range of SNPN cells can optionally be signalled to UEs.

- No proximity indication in CONNECTED mode is needed for SNPN.

- no preliminary access check for SNPN cells in CONNECTED mode.

Agreements:

- SIB1 of NPN-only cell prevents access attempts by Rel-15 UEs for normal services.

- SIB1/MIB supports prevention of access attempts by Rel-15 UEs on a SNPN-only cell for emergency services.

- SIB1/MIB supports prevention of access attempts by Rel-15 UEs on a CAG-only cell for emergency services (this does not mean that access attempts by Rel-15 UEs for emergency services on CAG-only cell are always not allowed. This is still FFS.The feasibility of allowing emergency services on CAG-only for Rel-15 UEs will be discussed in the email discussion on RRC aspects/SIB1 design)

- Access attempts by Rel-15 UEs for emergency services on SNPN-only cell are not allowed.

- In a NPN-only cell, access attempts for normal services by Rel-16 UEs without support for NPN is not allowed.

- In a SNPN-only cell, access attempts for emergency services by Rel-16 UEs without support for SNPNs is not allowed.

- For a PLMN+NPN cell, Rel-15 UEs should be able to access PLMNs associated with the cell for normal and/or limited service.

- A new Rel-16 IE is needed with a role similar to role of cellReservedForOtherUse for Rel-15 UEs (FFS whether this will be PLMN specific)

- SIB1 allows indication of TAC, RANAC, cellIdentity per SNPN (per PLMN ID + NID). FFS on other IEs. FFS whether Rel-15 IEs or Rel-16 IEs are used for the indication.

- SIB1 allows indication of TAC, RANAC, cellIdentity for each CAG. FFS on other IEs. The fields are indicated per PLMN-ID. FFS whether Rel-15 IEs or Rel-16 IEs are used for the indication.