3GPP TSG-RAN WG2 #108	Tdoc R2-1916390

Reno,  USA, 18 – 22 November 2019

Agenda Item:	11.6.4.1.1

Source:	Ericsson

Title:	CHO for NTN LEO

Document for:	Discussion, Decision

# Introduction

In RAN#80, a new SI “Solutions for NR to support Non-Terrestrial Network” was agreed [1]. It is a continuation of the preceding SI “NR to support Non-Terrestrial Networks” (RP-171450), where the objective was to study the channel model for the non-terrestrial networks, to define deployment scenarios, parameters and identify the key potential impacts on NR. The results are summarized in [2]. The new study item has the objective at evaluating potential solutions addressing the minimum necessary identified key impact areas from the previous activity and to study impact on RAN protocols/architecture. The objectives for layer 2 and above are:

- Study the following aspects and identify related solutions if needed: Propagation delay: Identify timing requirements and solutions on layer 2 aspects, MAC, RLC, RRC, to support non-terrestrial network propagation delays considering FDD and TDD duplexing mode. This includes radio link management. [RAN2] - Handover: Study and identify mobility requirements and necessary measurements that may be needed for handovers between some non-terrestrial space-borne vehicles (such as Non Geo stationary satellites) that move at much higher speed but over predictable paths [RAN2, RAN1] - Architecture: Identify needs for the 5G’s Radio Access Network architecture to support non-terrestrial networks (e.g. handling of network identities) [RAN3] - Paging: procedure adaptations in case of moving satellite foot prints or cells Note: - This new study item does not address regulatory issues.

In RAN2#104, the following is agreed for mobility:

Agreements:

1.	Satellite beams, satellites or satellite cells are not considered to be visible from UE perspective in NTN SI.  This does not preclude differentiating at the PLMN level the type of network (e.g. NTN vs. terrestrial).  This is up to SA2.

2.   Revise the current definition of satellite cell in TR 38.821 and refer to a satellite beam.  Definition of satellite beam can be discussed during email discussion.

3.	Add text in TR 38.821 stating that association between NR PCI and NR SSBs is left for implementation (i.e. it will not be specified)

4.	Consider Rel-15 definitions as a baseline for NTN

5.	Both option a and b can be considered in NTN SI with one or multiple SSBs per PCI.  The TR will capture a figure for both option.

In this paper, we discuss connected mode mobility for LEO NTN while feeder link stays the same. The feeder link switching is treated in [4][5].

# Cell level mobility for LEO satellites with earth moving beams (scenarios C2 and D2)

In the work item for NR Mobility Enhancements [1], one objective is to improve the robustness at handover. In RAN2#106 the following was agreed concerning Conditional Handover in NR:

Agreements

0:	CHO is introduced in NR to solve robustness/reliability issue.

a/ The source cell decides on the condition for the execution of CHO

b/ The source cell adds the condition for the execution of CHO to the RRC message sent to UE.

c/ Multiple CHO candidate cells can be sent in either one or multiple RRC messages.

=> FFS on signalling details

=> FFS how CHO execution is handled

. . .

Agreements

…

4	Multiple CHO candidate cells can be sent in either one or multiple RRC messages. FFS on signalling details. FFS how CHO execution is handled.

. . .

. . .

4	Deconfiguration of CHO candidates is performed by RRC signalling (we will not introduce timer based mechanism for the UE to deconfiguration of the CHO candidates).

. . .

From the above, it can be deduced that the work on CHO in mobility WI will continue in parallel to NTN SI and will handle also other aspects, like inter node signalling, in addition to possible triggering conditions. For the NTN discussions related to CHO, it seems that it would be beneficial to list possible triggering conditions from NTN operation perspective and give related pros and cons in the TR.

One triggering condition not yet mention in the draft TR is the timing advance based triggering. Timing advance based triggering fits well for the issue where UE needs to precompensate time when sending the RACH preamble in order the target cell to receive the preamble properly. Further, due to small difference to RSRP/RSRQ values between overlapping cells, timing advance based triggering may provide better accuracy for the triggering point. As a drawback, it requires GNSS capable UEs. We are proposing toe capture this triggering option in TR with mentioned pros and cons.

TP is revised to include suggestions from R2-1914739 and R2-1916242.

- Agree the TP in appendix.

# Conclusion

We propose the following:

Proposal 1	Agree the TP in appendix.

# References

- RP-181370, New SID on Solutions for NR to support Non Terrestrial Network, Thales, RAN#80, La Jolla, USA, June 2018

- TR 38.811, Study on New Radio (NR) to support non terrestrial networks, Release 15

- TR 38.821, Solutions for NR to support Non Terrestrial Network

- R2-181xxx, Mobility for GEO, Ericsson, RAN2#105, Athens Greece

# Appendix

------------------------start----------------------------------

##### 7.3.2.2.2	Conditional Handover

- Measurement-based triggering: Agreements in the mobility enhancements WI are to be taken as baseline. Configuration of triggering thresholds and/or which measurement events to use as triggers should consider the NTN environment e.g. the small variation of the cell quality measured in cell center and at cell edge in NTN.

- Location (UE and Satellite) triggering: additional triggering conditions based on UE and satellite location can be considered in NTN and may be considered independently or jointly with another trigger (e.g. measurement based). Location-based conditional HO in LEO scenarios should consider deterministic satellite movement.

- Time(r)-based triggering: Several triggering conditions considering the time a region is served can be considered. This may be based on UTC time, or a timer-based solution, and may be considered independently or jointly with another trigger (e.g. measurement based). Time-based conditional HO in LEO scenarios should consider deterministic satellite movement.

- Timing advance value based triggering: additional triggering conditions based on timing advance value to the target cell can be considered in NTN and may be considered independently or indepenedly or jointly with another trigger.

- Distance based triggering: Additional triggering conditions based on the distance between UE and the satellite and may be considered independently or jointly with another trigger.

- Elevation angles of source and target cells based triggering: additional triggering conditions based on elevation angles of source and target cells can be considered in NTN and may be considered independently or jointly with another trigger.

- Pre-triggering based triggering: additional triggering conditions configurable (by the RAN) mechanisms where the UE needs only to select among a few possible ‘trends’ in the signal levels (e.g. RSRP, RSRQ, SINR), and these signal trends are pre-calculated, updated and signalled to the UE(s), by the RAN. This would allow for RAN adaptation, so that not all NTN cells use the same set of signal trend configurations (possibly also changing vs. time) and, also accommodate the UE specific radio channel conditions. Note that precise geographical area of the UE would not be needed, because in NTN the signal trends are very similar for larger geographical areas. Furthermore, the network could also include the estimated movement of the UE in the expected signal trends, when this information is available.

- .

Table 7.3.2.2.2-1: Pros and Cons of the different triggering conditions

| Triggering Method | Advantages | Disadvantages |
| --- | --- | --- |
| Measurement-based | - Less specification impact (i.e. similar to terrestrial network); - Is supported in Rel-16 mobility enhancements WI; - Relies on UE estimates and established channel estimation techniques; - Is based on receiving power and cell quality. | - Would require neighbouring cell lists, which may be difficult given the fast-moving nature of LEO satellites or under inconsistent/deviating cell coverage; - Small RSRP/RSRQ variation in regions of cell overlap and propagation delay may make measurement-based triggering (e.g. A3 events) unreliable; - May be difficult to ensure UE performs handover to a specific country. |
| Location-based | - Useful when cell boundaries are dispersed/undefined; - Can enable mandatory HO based on UE location; - A precise trigger as UE location can be known with a degree of accuracy; - Able to predict/pre-emptively configure triggering condition using satellite ephemeris and deterministic satellite movement; - Would be useful to overcome issue of small RSRP/RSRQ variation in regions of cell overlap; - UE may need to perform fewer measurements for HO purposes. | - The UE may trigger HO to an unavailable cell (e.g. the NTN cell has deviated or is inconsistent, under varying channel conditions, or if the network sets the wrong triggering condition); - Some UEs may not have positioning capability. - UE must continuously track the satellites trajectory, and the network will need up-to-date UE location information which may introduce high overhead. |
| Time/Timer-based | - Can be useful to maintain service continuity if UE loses terrestrial coverage; - Can enable mandatory HO based on timing; - Network can configure different timing lengths to mitigate possible RACH congestion; - Can work with satellite ephemeris and exploit the deterministic movement of satellites; - UE may need to perform fewer measurements for HO purposes. | - The UE may trigger HO to an unavailable cell (e.g. the NTN cell has deviated or is inconsistent, varying channel conditions, or if the network sets the wrong triggering condition); - Depending on the accuracy of the ephemeris data and mobility of the UE, this may not be an accurate trigger which could, e.g. result in early/late HO; - Maintaining multiple timers for every UE could introduce high overhead. |
| Timing advance value based triggering | - Timing advance based triggering fits well for the issue where UE needs to precompensate time when sending the RACH preamble in order the target cell to receive the preamble properly. - Further, due to small difference to RSRP/RSRQ values between overlapping cells, timing advance based triggering may provide better accuracy for the triggering point. | - Requires GNSS capable UEs. Support for this needs to be explicitly added as not part of Rel-16. |
| Distance based triggering | - Since timing advance is derived based on the distance between UE and the satellite, configuring the distance as a triggering condition directly can help reduce complexity and power consumption in the UE side in calculating the timing advance. | - Requires GNSS capable UEs. |
| Elevation angles of source and target cells based triggering: | - Beneficial for handover regions that are irregular shaped | - UE needs to evaluate elevation angle based on UE location and satellite ephemeris data |
| Pre-triggering based triggering: | - Using the pre-triggering mobility measurements as described above, the UE can determine in advance the status of the condition(s) configured for HO, thus is able to safely initiate the HO to the target cell while still begin in good radio conditions in the source cell. | - Exact difference and trade off between implementation complexity/specification effort complexity versus performance gain needs further study when comparing to existing Rel-16 CHO which is based on RSRP/RSRQ threshold. |

-------------------------------end------------------------------------