3GPP TSG-RAN WG2 Meeting #108	R2-1916496

Reno, USA, 18 – 22 November 2019

Title:	[DRAFT] LS on default codebook parameters

Response to:	-

Release:	Release 15

Work Item:	NR_newRAT-Core

Source:	Nokia [TSG RAN WG2]

To:	TSG RAN2

Cc:

Contact Person:

#### Name:	Amaanat Ali

E-mail Address:	amaanat.ali@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN2 would like to update TS 38.306 (based on the ongoing discussion on R2-1915508) with default capabilities for the codebookParameters UE capability (see below).

| codebookParameters Indicates the codebooks and the corresponding parameters supported by the UE. Parameters for type I single panel codebook (type1 singlePanel) supported by the UE, which is mandatory to report: - supportedCSI-RS-ResourceList; - modes indicates supported codebook modes (mode 1, both mode 1 and mode 2); - maxNumberCSI-RS-PerResourceSet indicates the maximum number of CSI-RS resource in a resource set. Parameters for type I multi-panel codebook (type1 multiPanel) supported by the UE, which is optional: - supportedCSI-RS-ResourceList; - modes indicates supported codebook modes (mode 1, mode 2, or both mode 1 and mode 2); - maxNumberCSI-RS-PerResourceSet indicates the maximum number of CSI-RS resource in a resource set; - nrofPanels indicates supported number of panels. Parameters for type II codebook (type2) supported by the UE, which is optional: - supportedCSI-RS-ResourceList; - parameterLx indicates the parameter "Lx" in codebook generation where x is an index of Tx ports indicated by maxNumberTxPortsPerResource; - amplitudeScalingType indicates the amplitude scaling type supported by the UE (wideband or both wideband and sub-band); - amplitudeSubsetRestriction indicates whether amplitude subset restriction is supported for the UE. Parameters for type II codebook with port selection (type2-PortSelection) supported by the UE, which is optional: - supportedCSI-RS-ResourceList; - parameterLx indicates the parameter "Lx" in codebook generation where x is an index of Tx ports indicated by maxNumberTxPortsPerResource; - amplitudeScalingType indicates the amplitude scaling type supported by the UE (wideband or both wideband and sub-band). supportedCSI-RS-ResourceList includes list of the following parameters: - maxNumberTxPortsPerResource indicates the maximum number of Tx ports in a resource; - maxNumberResourcesPerBand indicates the maximum number of resources across all CCs within a band simultaneously; - totalNumberTxPortsPerBand indicates the total number of Tx ports across all CCs within a band simultaneously. | Band | FD | No | No |
| --- | --- | --- | --- | --- |

RAN2 requests RAN1 to provide the default values for the following parameters for type I single panel codebook (type1 singlePanel) which the UE is mandated to report:

-	supportedCSI-RS-ResourceList;

-	modes indicates supported codebook modes (mode 1, both mode 1 and mode 2);

-	maxNumberCSI-RS-PerResourceSet indicates the maximum number of CSI-RS resource in a resource set.

2. Actions:

To RAN1.

ACTION: 	RAN2 respectfully asks RAN1 to provide the default values for the above listed parameters for type I single panel codebook (type1 singlePanel) which the UE is mandated to report.

3. Date of Next TSG-RAN WG2 Meetings:

3GPP RAN2#109		24 – 28 February 2020		Athens

3GPP RAN2#109bis		04 – 20 April 2020			Japan