3GPP TSG RAN WG2 Meeting #109          		          	    	 	 R2-2000469

Electronic meeting, Feb 24-Mar 6, 2020 	Revision of R2-1914733

Agenda Item:	6.1.2

Source:	Intel Corporation

Title:	Parent selection at IAB nodes during initial setup

Document for:	Discussion and Decision

# Introduction

As part of the IAB node integration procedure RAN3 has agreed (LS in R3-194787) that:

- IAB node sends an “IAB indication” in the RRC setup complete message (indicating that it is an IAB node). This is used by the IAB donor CU to identify an AMF/MME that supports IAB.

- The CU includes an explicit IAB indication in the INITIAL UE MESSAGE to the AMF/MME

- The IAB node and IAB donor broadcast an indication over the air indicating support of IAB (i.e., another IAB node’s MT can attach to it).

The contribution further explores the sequence of actions needed to establish the network. In particular, we focus on the need to minimize topology changes and handovers/reselections of IAB nodes immediately after the IAB network is setup. A companion contribution[2] considers the issue of parent selection at UEs.

# Discussion

IAB networks are setup to improve capacity and coverage in a fairly planned manner. This implies that IAB nodes near a given IAB donor are all activated at roughly the same time. This is especially true for scenarios where IAB networks are used to provide additional capacity at events – e.g., at sporting events and concerts.

IAB nodes follow the same procedures as UEs for attaching to the network. The overall procedure for IAB node integration is shown in Figure 1 below (from TS 38.401 [1]). In the first stage the IAB MT setup is performed. The MT of an IAB node, in its role as a regular UE, identifies a parent node (another IAB node or an IAB donor). The MT then performs random access and transmits an RRC connection setup request to the CU via the parent node. Following that, the backhaul RLC channel for carrying CP traffic to and from the IAB node is established. This is followed by a routing update phase which includes configuration of BAP routing identifiers and updating of routing tables of the IAB donor DU and all IAB nodes on the path to the IAB node. Following that, in the IAB DU setup phase, the DU functionality of the IAB node is configured (which consists of setting up of the F1-C connection between the IAB node and the IAB donor CU). Once this is completed, the IAB node can provide service to UEs.

Figure 1: Integration procedure for IAB node (TS 38.401)

IAB nodes can be integrated into the network (referred to as IAB node “activation” below) in different sequences within the same area. Even if all IAB nodes are to be activated at about the same time, the completion of the node integration phases will take different durations for different IAB nodes. There will be variations in the amount of time taken due to the number of hops and signal conditions. Given that UEs/MTs can attach to the IAB node upon completion of the IAB DU setup, the differences in the durations to complete the integration procedure at different IAB nodes can result in IAB nodes selecting sub-optimal parents.

We analyze the extent of the sub-optimal parent selection in simulations described below.

## IAB node parent selection

IAB node integration and resulting topology are studied under certain assumptions. One IAB donor with three sectors is considered. N IAB nodes are randomly dropped per sector (N = 3 and N = 5 are considered). To understand how the order in which IAB nodes are activated affects the topology, we compare the following two activation schemes:

- Randomly chosen sequencing: the IAB nodes are activated in a randomly chosen order, while ensuring that there are no cycles in the network.

- The IAB nodes are activated in an “Ideal” sequence

The ideal sequencing tries to ensure that IAB nodes are activated in the order of links with best RSRP. This results in each node attaching to a parent with the best RSRP thereby avoiding further topology changes immediately after the nodes are activated. Note that the ideal sequencing cannot be implemented in practice as the RSRP measurements made by the IAB nodes are not available to the network to make activation decisions. The “Ideal” sequencing procedure is implemented in simulations as described below:

- The set of potential parents P initially consists of the IAB donor only.

- While there are IAB nodes to be activated:

  - From the set of un-activated IAB nodes, select the IAB node N with the strongest signal to a potential parent Pi in P, make Pi the parent node of N, and activate N

  - Add N to P

Additional simulation assumptions are listed in the appendix (for example, donor and IAB node transmit powers, path-loss models, etc). Figure 2 shows the topology resulting from activating the nodes in one randomly chosen sequence in a random placement of IAB nodes with 3 IAB nodes per sector. The sequence of activation of IAB nodes is [2, 7, 3, 6, 9, 8, 4, 5, 1]. Figure 3 shows the topology for the same IAB node placement with the “Ideal” activation sequence.

| Figure 2: Topology for one drop (LoS) with 3 IAB nodes per sector using node activation sequence: [2, 7, 3, 6, 9, 8, 4, 5, 1] | Figure 3: Topology for same drop as Figure 2 using “Ideal” sequence of node activation. |
| --- | --- |

Similarly, Figure 4 and Figure 5 show the resulting topology for 5 IAB nodes per sector with random placement with activation according to a chosen sequence and for the ideal activation sequence respectively. The sequence of activation of IAB nodes for the scenario in Figure 4 is [14, 5, 3, 10, 13, 9, 15, 2, 8, 11, 6, 1, 4, 12, 7].

| Figure 4: Topology for one drop (LoS) with 5 IAB nodes per sector using node activation sequence: [14, 5, 3, 10, 13, 9, 15, 2, 8, 11, 6, 1, 4, 12, 7] | Figure 5: Topology for same drop as Figure 4 using “Ideal” sequence of node activation. |
| --- | --- |

The topologies of Figure 2 and Figure 4 are not efficient. The nodes are not attached to the best candidate parents resulting in sub-optimal performance (due to interference, low throughput, unnecessary latency etc). The network would need to change the topology shortly after the node integration procedure.

In order to change the topology, the network needs to perform handovers of the IAB nodes. The changes to the topology can be quite large:

- To go from the topology in Figure 2 to the topology in Figure 3, 4 of the 9 IAB nodes (2, 4, 6, 9) have to change parents.

- To go from the topology in Figure 4 to the topology in Figure 5, 10 of the 15 IAB nodes (2, 4, 5, 7, 8, 9, 10, 13, 14, 15) have to change parents.

The examples of Figures 2-5 assume that all links are strictly LoS. If the links can be NLoS, the resulting topology can be still more problematic. Figure 6 and Figure 7 show information about the number of links (referred to as “edges” of the graph representing the topology) in the network that are different for the topology resulting from random sequencing and the topology resulting from ideal sequencing.

The links are assumed to be LoS or NLoS (according to the path loss model in 38.300 – see appendix for details). This information is shown as a distribution. That is, for example, the probability of 6 links being different for the 3 IAB node per sector case (Figure 6) is more than ~0.23; the probability of 5 links being different is ~0.18.

| Figure 6: Distribution of the number of links that are different between random sequencing of IAB node activation and Ideal sequencing of IAB node activation for 3 IAB nodes per sector | Figure 7: Distribution of the number of links that are different between random sequencing of IAB node activation and Ideal sequencing of IAB node activation for 5 IAB nodes per sector |
| --- | --- |

Overall Figure 6 and Figure 7 emphasize that the topology resulting from the random selection can be very different from the ideal sequencing.

Observation 1: The resulting topology after node integration is highly dependent on the chosen sequence for activation of IAB nodes. The resulting topology can be inefficient and the network may need to change the topology shortly after the node integration procedure.

Furthermore, in order to go from a topology such as the one in Figure 2 or Figure 4 to the corresponding one in Figure 3 or Figure 5, the network has to perform very specific sequences of handovers. Determining the sequence of handovers is non-trivial. For example, considering nodes 2, 4 and 5 in Figure 4 and Figure 5, the network has to first handover node 4 (along with any UEs attached) to the IAB donor; following that the network has to handover node 2 from node 5 to node 4; finally the network has to handover node 5 from the donor to node 2. These handovers are atypical. The handover of node 4 is to a node with worse radio conditions than its current parent (node 2). The handover of node 2 from 5 to 4 reverses the parent child relationship between node 4 and node 2. Managing the complexity of such sequences of handovers can be very challenging as the number of nodes and UEs increases.

Observation 2: To reorganize an inefficient topology towards a more efficient topology, the network needs to perform handovers in very specific sequences. Determining the sequences of such handovers is non-trivial and managing the complexity of such sequences of handovers can be challenging.

Thus, to avoid having to perform large topology reorganization of the network upon the node integration procedure, it is beneficial to ensure that the parent selection process yields a reasonable topology.

Observation 3: An initial topology that is efficient can significantly reduce the burden of topology reorganization and optimization for network operators.

While the ideal sequencing described above cannot be implemented in practice, something close to it can be achieved by using RSRP thresholds for IAB node parent selection. The IAB donor and the activated IAB nodes broadcast an RSRP threshold and un-activated IAB nodes attach to the best candidate parent as long as the measured RSRP is above the threshold, and are integrated into the network. The threshold is successively reduced until all IAB nodes are activated. Such a mechanism can achieve results similar to the Ideal sequencing shown in Figure 3 and Figure 5 (with the result approaching the ideal sequencing as step size of the threshold reduction gets smaller).

Proposal 1: IAB node selects another IAB node or an IAB donor as a parent only if the RSRP of the IAB node or IAB donor exceeds a threshold (which is provided in system information). The threshold is successively decreased in steps to allow all IAB nodes to integrate into the network.

It should be noted that UEs may already be connected to the IAB nodes. The use of the threshold should not cause UEs to take cell selection/reselection actions as this would be disadvantageous. Therefore, the threshold used for such topology formation should be independent of thresholds used by UEs for cell selection/reselection (e.g., QRxlevMin).

Proposal 2: The threshold used for IAB topology reorganization should be independent of cell selection/reselection thresholds.

# Conclusion

In this contribution we have studied the network topology and UE association that results from the initial network setup and IAB node integration procedure. Our simulation study shows that the resulting topology can be inefficient even for small networks. Performing topology adaptation after the initial network setup to correct the inefficiencies requires complex handover sequences and potentially large numbers of handovers. The following are our observations and proposals:

Observation 1: The resulting topology after node integration is highly dependent on the chosen sequence for activation of IAB nodes. The resulting topology can be inefficient and the network may need to change the topology shortly after the node integration procedure.

Observation 2: To reorganize an inefficient topology towards a more efficient topology, the network needs to perform handovers in very specific sequences. Determining the sequences of such handovers is non-trivial and managing the complexity of such sequences of handovers can be challenging.

Observation 3: An initial topology that is efficient can significantly reduce the burden of topology reorganization and optimization for network operators.

Proposal 1: IAB node selects another IAB node or an IAB donor as a parent only if the RSRP of the IAB node or IAB donor exceeds a threshold (which is provided in system information). The threshold is successively decreased in steps to allow all IAB nodes to integrate into the network.

Proposal 2: The threshold used for IAB topology reorganization should be independent of cell selection/reselection thresholds.

A text proposal to TS 38.331 corresonding to the above proposals is provided in [3].

# References

- R3-193351, “Support for IAB”, CR to 38.401, Aug 2019.

- R2-1914747, “Parent Selection at UEs during Initial Setup”, RAN2#108, Intel Corp.

- R2-2000479, “Text proposal for Network Setup – IAB”, RAN2#109e, Intel Corp.

# Appendix

Table 3: Simulation Assumptions

| Parameter | Value |
| --- | --- |
| Layout | Heterogeneous scenario, two layer: Macro layer: Hex. Grid, 1 site, 3 sectors Micro layer: random drop, 3 micro BSs per macro sector |
| Inter-BS distance | 500 m |
| Min distance | Minimum distance between Micro TRPs; 10m. Minimum distance between Macro TRP and UE; 35m. Minimum distance between Micro TRP and UE; 10m. Minimum distance between Micro TRPs and Macro TRP; 40 m. |
| Large scale channel parameters | Above 6GHz: - Macro-to-UE: 5GCM UMa - Micro-to-UE: UMi-Street canyon - Macro-to-Macro: 5GCM UMa (hUE =25m) - Macro-to-Micro: 5GCM UMa (hUE =10m) - Micro-to-Micro: UMi-Street canyon (hUE =10m) According to 3GPP TR 38.900 Table 7.4.1-1 |
| BS Tx power | Macro layer: 40 dBm Micro layer: 33 dBm |
| # random drops | 500 |