3GPP TSG-RAN WG2 Meeting #109-e	R2-2000617

# Electronic meeting, 24th February – 06th March 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.322 | CR | 0143 | rev | - | Current version: | 14.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on polling bit for RRCConnectionRelease |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh-Core |  |  |  |  |  | Date: |  |  | 2020-02-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In Rel-14, Fast RRC connection release was agreed (TS 36.331 CR 3157): In case the UE is not polled, the UE can consider the receipt of the RRCConnectionRelease message to be succesfully acknowledged when the UE has send HARQ ACK. However, this is is not allowed by the RLC specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Impact Analysis Add a note to clarify that, in case the RLC PDU carries RRCConnectionRelease, the eNB may omit the poll bit. Impacted functionality: Fast RRC connection release. Inter-operability: The CR only impacts the NW, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Fast RRC connection release cannot be used. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 5.2.2.1	Transmission of a AMD PDU or AMD PDU segment

Upon assembly of a new AMD PDU, the transmitting side of an AM RLC entity except for NB-IoT shall:

-	increment PDU_WITHOUT_POLL by one;

-	increment BYTE_WITHOUT_POLL by every new byte of Data field element that it maps to the Data field of the RLC data PDU;

-	if PDU_WITHOUT_POLL >= pollPDU; or

-	if BYTE_WITHOUT_POLL >= pollByte;

-	include a poll in the RLC data PDU as described below.

Upon assembly of an AMD PDU or AMD PDU segment, the transmitting side of an AM RLC entity shall:

-	if both the transmission buffer and the retransmission buffer becomes empty (excluding transmitted RLC data PDU awaiting for acknowledgements) after the transmission of the RLC data PDU; or

-	if no new RLC data PDU can be transmitted after the transmission of the RLC data PDU (e.g. due to window stalling);

-	include a poll in the RLC data PDU as described below.

NOTE 1:	Empty RLC buffer (excluding transmitted RLC data PDU awaiting for acknowledgements) should not lead to unnecessary polling when data awaits in the upper layer. Details are left up to UE implementation.

NOTE 2:	In case the RLC data PDU carries RRCConnectionRelease, E-UTRAN may omit the poll in the RLC data PDU.

To include a poll in a RLC data PDU, the transmitting side of an AM RLC entity shall:

-	set the P field of the RLC data PDU to "1";

-	set PDU_WITHOUT_POLL to 0, except for NB-IoT;

-	set BYTE_WITHOUT_POLL to 0, except for NB-IoT;

After delivering a RLC data PDU including a poll to lower layer and after incrementing of VT(S) if necessary, the transmitting side of an AM RLC entity shall:

-	set POLL_SN to VT(S) – 1;

-	if t-PollRetransmit is not running:

-	start t-PollRetransmit;

-	else:

-	restart t-PollRetransmit;