3GPP TSG-RAN WG2 Meeting #109-e	R2-2000743

Online, February 24th– March 6th 2020

Agenda Item:	6.1.2

Source:	Ericsson

Title:	On Multi-connectivity for IAB

Document for:	Discussion, Decision

# 1	Introduction

At RAN2#107, it was agreed that multi-connectivity can be supported by dual connectivity (DC), by assigning the roles of MN and SN to the IAB nodes serving the outer leaf access IAB node. Furthermore, RAN3 #105bis made the following agreements:

BAP – Intra-donor configuration

- Configuration of downlink bearer mapping and routing should be performed by F1-AP

- Path id is derived from IP header and mapping provided by CU

- In the DL, for BAP path id derivation on the donor DU: IP address, IPv6 flow level and/or DS/DSCP can be used; all of these fields are optional in F1AP message to configure routing

IP address management

- IAB node can obtain an IP address via OAM

- The donor CU or donor DU can use OAM or DHCP to allocate IAB node IP address

- IAB node can request one or more IP addresses from donor CU via RRC

- CU can obtain IAB node IP address from donor DU via F1AP (other methods are not precluded)

- CU can send IP address to IAB node via RRC

This paper goes one step further by proposing multi-connectivity via multiple MTs per IAB node.

# 2	Discussion

Currently, NR supports DC by setting up multiple UE bearer contexts in the DUs that serve the UE. These different UE contexts are identified as part of F1-U (GTP tunnels) towards the DU serving the UE. This is shown in Figure 1.

The DC aspects are transparent to UE application layers, i.e. the UEs just send/receive data from a DRB which could be configured as an MCG, SCG or split DRB. In case of split DRB, the splitting point is below PDCP and relying on various NR PDCP functions to handle re-ordering, re-transmission and duplication removal.

RAN2#105bis agreed to reuse the NR-DC framework for setting up multi-connectivity to IAB nodes. It was also agreed that only MCG or SCG BH RLC channels should be used, meaning that the traffic will not be split, i.e., the split bearer will not be supported as there is no NR PDCP for F1-U traffic to the IAB node.

Figure 1: Support for NR-DC to UEs

RAN2 agreed that NR-DC framework should be used to set up multi-connectivity for IAB nodes, but there will be no support for split bearers to the IAB node.

If an IAB node has been set up multiple connections, it is possible to associate the different connections with different BAP Path IDs as part of the agreed BAP routing ID.

The BAP Path ID field of the BAP routing ID can be used to identify the different paths in the BAP layer.

The DC framework currently has some limitations:

- DC is only supported between different frequency bands;

- Only 2 cell groups are currently supported;

- For IAB nodes using EN-DC, only a single NR SCG is supported.

One alternative to get around these limitations is to extend the DC framework in the 3GPP specifications by allowing intra-frequency DC, multi-connectivity with more than two cell groups, etc. However, a simpler approach for solving the same problem is to allow the IAB to support multiple logical MTs, which set up independent connections to the network. This approach can support any number of connections on any frequency and, from a user plane point of view, is identical to the current DC-based solution.

It is expected that some IAB nodes will support multiple radio units, capable of operating independently or only loosely coupled with each other, on the same frequency. These radio units could be associated with different antennas pointed in different directions, e.g. covering different sectors. Although these radio units would still be limited by half-duplex constraints (i.e. not being capable of simultaneous transmission and reception on the same frequency), they would support simultaneous transmission to, and simultaneous reception from multiple parent nodes on the same frequency.

In this respect, some IAB nodes will likely be very different from current UE chipset modems, which support tightly integrated DC solutions.

Some IAB nodes are expected to support multiple radio units capable of simultaneous TX (no RX) and simultaneous RX (no TX) in different directions, on the same frequency band.

Current DC-based solution does not support simultaneous TX or RX scenarios on the same band.

One simple way to allow these nodes to set up multiple connections to the network is to allow them to operate the MT parts as independent MTs with separate control RRC/NAS connections to the network, and separate BAP addresses, etc. Similar to DC-based solution, each connection can be treated as a separate NIC from the higher-layer point of view. With such a solution, the normal connectivity management procedure can be used for setting up the MT connections. The only additional functionality required is to ensure that the different MT connections are set up via different radio paths. Most likely, this can be supported based on IAB node implementation, i.e. the IAB node will only connect multiple paths if multiple parent cells are available. An added benefit of using multiple MTs is that more than 2 NR paths (or more than 1 NR path for EN-DC) can be supported without standard impacts.

Using multiple MTs is a way to support multiple connections to the IAB node(s) without the limitations associated with the current DC-based solution.

If an IAB node has multiple MTs, no interface or communication between the different MTs is required.

Multiple MTs can be supported by reusing the existing connection setup mechanism. It could be left to IAB node implementation/configuration to set up multiple MT connections when it can connect via different parent cells.

The multiple MTs of a given IAB node need to be associated/linked to the DU of the same IAB node. For the baseline case of a single MT IAB node, RAN3 has agreed (from baseline CR 38.401):

The IAB-donor CU discovers collocation of IAB MT and IAB DU from the IP address used by the IAB DU for F1-C, or from the IAB-node’s BAP address included in F1AP (e.g., F1 SETUP REQUEST).

Thus, the same approach can be used to associate more than one MT to a given DU.

The current agreed mechanism to associate the MT and DU of an IAB node can easily be adopted to associate multiple MTs to one DU of an IAB node.

Based on the above we propose the following:

Allow IAB node(s) to support multiple MTs based on implementation/configuration to enable simultaneous multi-connectivity.

In order to associate/link multiple MT connections to the same F1 connection it should be possible to include multiple BAP addresses in the F1 SETUP REQUEST.

# 4	Conclusion

Based on the discussion in the previous sections we observe the following:

Observation 1	RAN2 agreed that NR-DC framework should be used to set up multi-connectivity for IAB nodes, but there will be no support for split bearers to the IAB node.

Observation 2	The BAP Path ID field of the BAP routing ID can be used to identify the different paths in the BAP layer.

Observation 3	Some IAB nodes are expected to support multiple radio units capable of simultaneous TX (no RX) and simultaneous RX (no TX) in different directions, on the same frequency band.

Observation 4	Current DC-based solution does not support simultaneous TX or RX scenarios on the same band.

Observation 5	Using multiple MTs is a way to support multiple connections to the IAB node(s) without the limitations associated with the current DC-based solution.

Observation 6	If an IAB node has multiple MTs, no interface or communication between the different MTs is required.

Observation 7	Multiple MTs can be supported by reusing the existing connection setup mechanism. It could be left to IAB node implementation/configuration to set up multiple MT connections when it can connect via different parent cells.

Observation 8	The current agreed mechanism to associate the MT and DU of an IAB node can easily be adopted to associate multiple MTs to one DU of an IAB node.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Allow IAB node(s) to support multiple MTs based on implementation/configuration to enable simultaneous multi-connectivity.

Proposal 2	In order to associate/link multiple MT connections to the same F1 connection it should be possible to include multiple BAP addresses in the F1 SETUP REQUEST.