3GPP TSG-RAN WG2 #109-e	Tdoc R2-2000978

Electronic meeting, 24 February – 6 March 2020	Revision of R2-1915396

Agenda Item:	7.1.7

Source:	Ericsson

Title:	MPDCCH performance improvement for LTE-M

Document for:	Discussion, Decision

# 1	Introduction

The Rel-16 work item (WI) on “Additional MTC enhancements for LTE” [1] has the following objective:

Improved DL transmission efficiency and/or UE power consumption: - […] - Specify MPDCCH performance improvement by using CRS [RAN1, RAN2, RAN4] - […]

RAN1 started the discussion on this objective in RAN1#94 and in subsequent meetings until RAN1#99  reached the following agreements relevant to RAN2 [2]:

Agreement For distributed MPDCCH, mapping between CRS and DMRS is defined in specifications. - FFS the details (e.g. number of PRBs over which mapping applies, fixed or configurable) Agreement Power offset between CRS and DMRS antenna ports of MPDCCH is signaled by eNB. FFS the signaling details (e.g. provisioning for default power offset value). Agreement For UE in Connected/Idle mode, the power offset between CRS and DMRS antenna ports of MPDCCH is indicated by SIB. Agreement - For UE in Idle mode, the configuration of CRS for MPDCCH performance improvement is indicated via SIB/SI. FFS the details of configuration. - For UE in Connected mode, the configuration of CRS for MPDCCH performance improvement is indicated via UE-specific RRC signalling. FFS the details of configuration. Agreement [36.213, 36.331] The higher layer parameter mpdcch-crs-power-ratio can be configured to values in the set {-4.77, -3, -1.77, 0, 1, 2, 3, 4.77} dB. Agreement (RRC impact) [36.211, 36.213, 36.331] For TDD operation, enable closed loop operation with reciprocity-based beamforming: - The close loop candidates do not have an association with CRS ports. - A separate RRC signalling is introduced to enable this feature

RAN2#103bis started the discussion on the feature from RAN2 point of view which continued also in RAN2#104 and RAN2#106 and reached the following agreements [3]:

RAN2#103bis agreements: - eNB enables CRS for improving channel estimation on MPDCCH via broadcasted RRC signalling. RAN2#104 agreements: - CRS for improving channel estimation on MPDCCH is configured via SIB2. This agreement can be revisited if RAN2 identifies that it would be more reasonable to configure it via SIB1-BR. It is FFS whether configuration would implicitly mean to enable the feature. - UEs report capability to indicate support of using CRS for improving MPDCCH channel estimation. - It is FFS if enabling/disabling is needed for the UE. RAN2#106 agreements: - No need to introduce a separate indication to enable the feature. Configuration of CRS for MPDCCH performance improvement enables the feature implicitly for UEs in Idle mode. - Working assumption: If dedicated configuration is not provided in connected mode, UE can use the broadcasted configuration if it exists. RAN2#107bis agreements: - UE shall consider the feature disabled in idle mode when configuration in idle mode is not broadcasted. - The dedicated configuration is released when UE is released to idle mode. FFS when UE is suspended. RAN2#108 agreements: If a dedicated configuration is not provided, the UE uses the broadcasted configuration. When UE is suspended, dedicated configuration is released.

In this contribution we discuss signalling details based on RAN1 progresses and agreements focusing on the interpretation of the various parameters in IDLE and CONNECTED mode. This paper is a revision of [4].

# 2	Discussion

So far, the demodulation of MPDCCH is only based on DMRS. For Rel-16, agreements have been made in RAN1 and RAN2 to make it possible to improve the MPDCCH channel estimation by enabling the possibility to use CRS for MPDCCH demodulation. Depending on the type of the UE, i.e. BL or non-BL, and details of the mapping between DMRS and CRS, gains can be expected for both high and low SNR levels resulting in fewer number of repetitions needed for successful MPDCCH decoding. This results in less time needed for the UE to monitor MPDCCH and, consequently, improvements in power consumption, provided that the potentially increased load of processing the two types of reference signals is not too costly.

This feature aims both idle and connected mode so that UEs can use CRS for improved MPDCCH channel estimation both when they are in connected mode and for example while monitoring paging and during the RA procedure prior to entering the connected mode.

## 2.1		Details of signaling

In the running CRs for TS 36.331, a new IE (CRS-ChEstMPDCCH-Config) in RadioResourceConfigDedicated and SystemInformationBlockType2-BR has been introduced. This IE should contain all the configuration related to MPDCCH performance improvement which will be present or not depending on whether it is a broadcasted or dedicated configuration.

The L1 parameter list [6] contains the following parameters for this feature:

| mpdcch-crs-idle-config | New | MPDCCH DMRS/CRS based demodulation is enabled in RRC_IDLE | If enabled, MPDCCH performance improvement can be achieved by using CRS (beside DMRS) for CE mode A/B in RRC_IDLE. |
| --- | --- | --- | --- |
| mpdcch-crs-power-ratio | New | MPDCCH DMRS/CRS power ratio | DMRS-to-CRS power ratio in dB for MPDCCH performance improvement for CE mode A/B in RRC_IDLE and RRC_CONNECTED. |
| mpdcch-crs-connected-config | New | MPDCCH DMRS/CRS based demodulation is enabled in RRC_CONNECTED | If enabled, MPDCCH performance improvement can be achieved by using CRS (beside DMRS) for CE mode A/B in RRC_CONNECTED. |
| mpdcch-crs-connectedlocalized-mapping-type | New | MPDCCH DMRS/CRSlocalized mapping type in RRC_CONNECTED | DMRS mapping type for MPDCCH performance improvement with localized MPDCCH allocation for CE mode A/B in RRC_CONNECTED. If CSI-based mapping type is selected then the DMRS/CRS combining is based on CSI feedback, otherwiseand if Predefined mapping type is selected then a predefined cycling is used. |

Based on earlier RAN2 agreements, it seems mpdcch-crs-idle-config and mpdcch-crs-connected-config are not explicitly needed, i.e. based on:

- No need to introduce a separate indication to enable the feature. Configuration of CRS for MPDCCH performance improvement enables the feature implicitly for UEs in Idle mode. - UE shall consider the feature disabled in idle mode when configuration in idle mode is not broadcasted. - If a dedicated configuration is not provided, the UE uses the broadcasted configuration.

Therefore, it remains to capture the remaining parameters in idle and connected mode configurations:

When transmitted in SIB2, CRS-ChEstMPDCCH-Config contains the power-ratio  applied in RRC_IDLE and in RRC_CONNECTED.

When transmitted in RadioResourceConfigDedicated, CRS-ChEstMPDCCH-Config  contains the CRS localizted mapping type applied in RRC_CONNECTED.

In other words, the mpddch-crs-power-ratio is configured only through SIB2 both for idle and connected mode, and is a cell-specific parameter, whereas mpddch-crs-localized-mapping-type is only for connected mode and is UE-specific.

Capture the text proposal in Section 2.2.

## 2.2	Text proposal

#### CRS-ChEstMPDCCH-Config

The IE CRS-ChEstMPDCCH-Config is used to configure and enable use of CRS for MPDCCH performance improvement, see TS 36.213 [23], clause xx.

CRS-ChEstMPDCCH-Config information elements

-- ASN1START

CRS-ChEstMPDCCH-Config-r16 ::=		SEQUENCE {

mpddch-CRS-PowerRatio-r16				ENUMERATED {dB-4dot77, dB-3, dB-1dot77, dB0, dB1, dB2, dB3, dB4dot77}		OPTIONAL,	-- Cond SIB2

mpdcch-CRS-LocalizedMappingType-r16	ENUMERATED {predefined, csiBased, reciprocityBased}							OPTIONAL	-- NEED ON

}

-- ASN1STOP

CRS-ChEstMPDCCH-Config field descriptions

mpddch-CRS-PowerRatio Power ratio in dB between CRS and DMRS antenna ports for MPDCCH performance improvement for CE Mode A and B and for RRC_IDLE and RRC_CONNECTED, see TS 36.213 [23], clause xx.

mpdcch-CRS-LocalizedMappingType DMRS mapping type for MPDCCH performance improvement with localized MPDCCH allocation for CE mode A and B in RRC_CONNECTED. When CSI-based precoding is configured, the UE monitors a set of MPDCCH candidates assuming the mapping between DMRS and CRS ports is based on a previously reported PMI. (For distributed MPDCCH, only predefined mapping between CRS ports and MPDCCH DMRS ports is supported.) When Reciprocity-based mapping is configured (only with TDD), closed loop operation is based on reciprocity-based beamforming, see TS 36.213 [23], clause xx.

| Conditional presence | Explanation |
| --- | --- |
| SIB2 | The field is mandatory present if powerOffset is included in crs-ChEstMPDCCH-ConfigCommon-r16 in SIB2. |

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	When transmitted in SIB2, CRS-ChEstMPDCCH-Config contains the power-ratio  applied in RRC_IDLE and in RRC_CONNECTED.

Proposal 2	When transmitted in RadioResourceConfigDedicated, CRS-ChEstMPDCCH-Config  contains the CRS localizted mapping type applied in RRC_CONNECTED.

Proposal 3	Capture the text proposal in Section 2.2.

# 4	References

RP-192875, “Revised WID: Additional MTC enhancements for LTE”, Ericsson, RAN#86, Sitges, Spain, December 2019

R1-1913594, RAN1 agreements for Rel-16 Additional MTC Enhancements for LTE, Reno, USA, November 2019

R2-1916424, RAN2 agreements for Rel-16 additional enhancements for NB-IoT and MTC, Reno, USA, November 2019

R2-1915396, MPDCCH performance improvement for LTE-M, Reno, Nevada, US, 18 – 22 November 2019

TS 36.331, v15.8.0

R1-1913673, Updated consolidated parameter list for Rel-16 LTE, Reno, Nevada, US, 18 – 22 November 2019