3GPP TSG-RAN WG2#109 eMeeting	Tdoc R2-20xxxx

# Online, Greece, 24th February- 4th March 2020

Agenda Item:		6.0.1

Souce:	Samsung

Title:	Notes from 36.331 Rel-16 CR merge [108#28][R16 RRC]

Document for:		Discussion and decision

# Introduction

This document includes some notes regarding general issues identified during the merge R16 CRs to 36.331 conducted as part of the following RAN2 e-mail:

[108#28][R16 RRC] RRC Merge (Ericsson Samsung)

Detection and Correction of RRC Merge Issues. This discussion is expected to start at 2020-01-23 when RRC running CRs are ready.

Intended outcome: Issue List, suggested corrections etc (details up to rapporteurs)

Deadline:  2020-02-13

RAN WG2#109 has become an online meeting, meaning that time for discussion will be quite limited. It may be best to discuss these more general issues in a further e-mail as part of the upcoming review in preparation for ASN.1 freeze of R16.

# Discussion

A list of the CRs that have been implemented is provided in the annex of the TDoc. The following table provides a number of more general issues regarding the merging of R16 CRs to 36.331.

| No | Description |
| --- | --- |
| 1 | For the cause in ResumeRequest, last spare is proposed to be taken for indicating MT EDT request. Seems good to confirm outside specific WI session (as it has general consequences) |
| 2 | Quite a few new messages/ procedures are introduced. Some concern NR specific versions, some concern critical extensions. Although specification is probably inconsistent already, some general review seems useful. |
| 2.1 | Related to this, it may be good to have some general discussion regarding extension mechanism for introducing further messages in future. I.e. use of ourter/ inner message branches (number of spares to introduce) and/ or whether to use extension marker |
| 2.2 | UE assistance: An NR specific message is introduced merely including an octet string carrying an NR IE, for which handling is specified in NR. Approach can be considered as part of general review regarding introduction of messages |
| 2.3 | FailureInformation2: Rather than creating an entirely new message, it seems possible/ appropriate to introduce a regular critical extension i.e. FailureInformation-r16. In procedure we would then just add setting of the new failure type (no real need to mention –r16 message version) and we would re-use existing ASN.1 section |
| 2.4 | SidelinkUEInformationNR is merely a container carrying an NR UL DCCH message within octet string. Similar functionality is provided by ULInformationTransferMRDC, that may be possible to re-use |
| 2.5 | ULInformationTransfer is extended for IAB by means of a critical extension even though only an optional IE is added for F1AP It seems this approach was selected because in the orginal version field dedicatedInfoType is mandatory If UE cannot ignore dedicatedInfoType whenever F1AP is included (i.e. when simultaneous transfer needs to be supported, such critical extension seems inevitable |
| 3 | One area where CRs typically result in changes to the same (ASN.1) section concerns the UE capabilities. Although merging may not always be a real problem, it may be good to discuss/ conclude which handling is preferred i.e. whether to have a CR collecting the changes to UE capabilities across the WIs |

# Conclusion & recommendation

This document includes some notes regarding general issues identified during the merge R16 CRs to 36.331. These more general issues may be handled in a further e-mail as part of the upcoming review in preparation for ASN.1 freeze of R16:

# References

[1] TS 36.331 Radio Resource Control

# Overview of implemented CRs

| Tdoc | Title | Source | CR | WI | E-mail# |
| --- | --- | --- | --- | --- | --- |
| R2-1911503 | Introduction of RLOS support indicator and RLOS request indicator | Qualcomm Incorporated | 4049 |  |  |
| R2-1913059 | Introduction of RRC parameters and UE capabilities for enhanced high speed scenario |  | 4095 |  |  |
| R2-1914018 | Introduction of UECapabilityInformation segmentation in 36.331 | MediaTek Inc., CATT, Ericsson, Spreadtrum Communications, ZTE Corporation, Sanechips, OPPO, Qualcomm Incorporated |  |  |  |
| R2-1914640 | Running CR for Introduction of Even futher Mobility enhancement in E-UTRAN | Ericsson |  | feMob | 35 |
| R2-1914661 | Introduction of a second SMTC for inter-RAT cell reselection | Orange, AT&T, Vodafone, Telecom Italia S.p.A., CMCC, NTT Docomo Inc., Samsung, Ericsson | 4114 | NR TEI16 |  |
| R2-1915034 | Introduction of voice fallback indication | Qualcomm Incorporated, T-Mobile USA, Verizon, China Telecom, Softbank, Ericsson | 4136 | NR TEI16 |  |
| R2-1915281 | Running CR for 36.331 for CA/DC Enhancements | Ericsson |  | eDCCA | 33 |
| R2-1915979 | Running CR to 36.331 for NR V2X | Huawei, HiSilicon |  | NR-V2X | 44 |
| R2-1916316 | Correction on non-3GPP paging | Huawei, HiSilicon | 4172 | LTE TEI16 |  |
| R2-1916318 | Introduction of DL MIMO efficiency enhancement | Huawei, HiSilicon |  | LTE TEI16 |  |
| R2-1916364 | Introduction of Rel-16 eMTC enhancements | Qualcomm Incorporated |  | eMTC | 29 |
| R2-1916407 | CR of TS 36.331 for introducing NavIC in LTE | Reliance Jio, MediaTek Inc., Huawei, CEWiT, Saankhya Labs Private Limited, Tejas Networks Ltd., Qualcomm Incorporated | 4137 | NavIC |  |
| R2-1916490 | Correction of UE assistance information | Samsung Telecommunications | 4164 |  |  |
| R2-1916554 | Text Proposal for TS 36.331 for support of F1AP signalling over LTE leg | Nokia, Nokia Shanghai Bell |  |  |  |
| R2-1916566 | Introduction of additional enhancements for NB-IoT in TS 36.331 | Huawei |  |  | 109 |
|  | Running CR for IAB | Ericsson |  | IAB | 31 |
|  | Running CR for MDT | Huawei |  | MDT | 43 |
|  | DL segmentation | Ericsson |  | TEI16 | 59 |
| R2-1915561 | Running LTE RRC CR for PPP-RTK support (SSR) | Qualcomm Incorporated |  | NR_pos-Core |  |
| R2-1912737 | Addition of broadcast of barometric pressure assistance data | Polaris Wireless, FirstNet, Intel, AT&T, NextNav | 4026 | TEI16, LCS_LTE_acc_enh-Core |  |
| R2-1914075 | Broadcast of TBS assistance data | NextNav, AT&T, FirstNet, Polaris Wireless | 4134 | TEI16, LCS_LTE_acc_enh-Core |  |