3GPP TSG-RAN WG2 Meeting #109-e	R2-2001181

Electronic meeting, 24 Feb - 6 Mar 2020

Title:	[DRAFT] LS on potential issue of Counter Check

Response to:

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	RAN2

To:	SA3

CC:

Contact Person:

#### Name:	David Lecompte

Email Address:	david.lecompte@huawei.com

Attachments:	None

1. Overall Description:

RAN2 discussed the existing Counter Check mechanism and confirmed that with a certain high packet rate and/or the configuration of SN terminated bearer, the Counter Check procedure may always consider that there is a traffic-insertion attach, even when no traffic was inserted.

To resolve this issue, RAN2 are thinking whether it is feasible to check less than the 25 MSBs.

2. Actions:

To SA3:

ACTION: 	RAN2 respectfully ask SA3 to check whether it is acceptable for the counter check procedure to check less than the 25 MSBs and indicate the minimum number of bits to be checked

3. Date of Next TSG-RAN WG2 Meetings:

RAN2#109bis		20-24 April 2020					Sapporo, Japan

RAN2#110		25-29, May, 2020					Athens, Greece