3GPP TSG-RAN WG2 Meeting #109-e                             R2-2001571

Electronic meeting, 24 February – 6 March 2020

Agenda item:	6.4.2.1

Source:	Samsung

Title:	Further Discussion on SL-RSRP reporting

Document for:	Discussion & Decision

# Introduction

This contribution discusses whether any further triggering condition on SL-RSRP reporting needs to be introduced in Rel-16.

# Discussion

At RAN2#108 meeting, the following agreements on SL-RSRP measurement and reporting were made:

3:	TX UE provides SL measurement configuration via PC5-RRC signalling to Rx UE.

4:	Rx UE should be able to report the SL-RSRP to Tx UE via PC5-RRC signalling. It is not necessary to notify the gNB of the SL-RSRP measurement result.

5:	Both event triggered and periodical sidelink RSRP measurement report could be supported.

6:	For the event triggered report, at least the Event A1 (SL-RSRP exceeds a threshold) or the Event A2 (SL-RSRP is lower than a threshold) shall be supported.

For the event triggered report on SL-RSRP, RAN2 introduced two events i.e. events S1 and S2 as specified in the draft running 38.331 CR. On top of these events, it would be beneficial to know the delta in finer granularity from the previously reported SL-RSRP measurement result in order to set appropriate transmit power from a Tx point of view. Our understanding is that introducing such new events is just a minor optimization with marginal benefits and brings more specification efforts given the limited time in Rel-16. Therefore, we propose that no further events are introduced for SL-RSRP reporting in Rel-16.

Proposal: Do not introduce additional event for SL-RSRP measurement reporting in Rel-16.

# Conclusion

Based on the above, RAN2 is requested to discuss and if possible agree on the following proposal:

Proposal: Do not introduce additional event for SL-RSRP measurement reporting in Rel-16.

# References