3GPP TSG-RAN WG2 Meeting #109e	R2-2002070

Elbonia, Online, 24 February – 6 March 2020

Agenda item:	6.9.3.6

Source:	Samsung (summary rapporteur)

Title:	Summary of AI 6.9.3.4 (Fast handover failure recovery)

WID/SID:	LTE_feMob-Core, NR_Mob_enh-Core - Release 16

Document for:	Discussion and Decision

# 1	Brief scope of the remaining issues of T312 handling

This document contains the summary of below listed Tdoc from agenda item 6.9.3.4 (Fast handover Failure Recovery). One issue concerning T312 handling in [1] has been discussed in the email discussion 108#66, therefore there is no proposal on this issue. This summary report takes into account other issues concerning T312 handling for which analysis is provided in Section 2.1 and 2.2.

| Sr. No | TDoc | Title | Source | Section Assignment |
| --- | --- | --- | --- | --- |
| [1] | R2-2000652 | Discussion on CHO impact on T312 | OPPO | 2.3 |
| [2] | R2-2000928 | T312 handling in NR | Sharp | 2.1 |
| [3] | R2-2001609 | Discussion on T312 support in CHO events | Samsung | 2.2 |
| [4] | R2-2001623 | Introduction of T312 for NR PSCell in (NG)EN-DC | Samsung | 2.1 |

# 2	T312 handling Summary

## 2.1	T312 handling w.r.t endorsed CR from [108#16]

Issue#1: Remaining issue for T312 handling in TS 38.331

In RAN2#108, the following has been agreed regarding T312 [5]:

1 Confirm the working assumption to Introduce T312 based mechanism on PSCell for fast declaration of SCG failure 2 T312 on PSCell can be configured for SCG measurement configurations provided over SRB3 or SRB1.

Stage 3 details have been progressed in email discussion [108#16] which resulted in endorsed CR for TS 38.331, which was then merged with running CR discussed as part of email discussion [108#34]. The below Tdoc raises some remaining updates to T312 handling which is aligned with T310 handling when the UE receives reconfiguration of reference signal type and reconfiguration of RLF timers and constants.

| Company/Tdoc | Proposals from Tdoc | Rapporteur Remark |
| --- | --- | --- |
| Sharp/R2-2000928 | Proposal 1: UE stops T312 for the corresponding SpCell if running if any of the reference signals that used for RLM are reconfigured for the SpCell. | Proposals are aligned with T310 handling in TS 38.331 when UE receives: a) reconfiguration of reference signal type used for RLM b) reconfiguration of RLF timers and constants |
|  | Proposal 2: UE stops T312 for the corresponding SpCell if running if it receives rlf-TimersAndConstants reconfiguration for the SpCell |  |

Recommended Proposal#1: RAN2 is requested to agree the proposals in [R2-2000928] and adopt in running CR the proposed TP.

Issue#2: Remaining issue for T312 handling in TS 36.331

Since the scope of email discussion [108#16] was endorsed CR for TS 38.331, the below CR proposes code-point to be introduced in the SCGFailureInformationNR message for (NG)EN-DC in TS 36.331. This issue was briefly discussed during email discussion [108#16] and it was concluded this issue is addressed in RAN2#109-e. The below CR proposes two alternatives to realize the reason for change.

| Company/Tdoc | Proposal from Tdoc | Rapporteur Remark |
| --- | --- | --- |
| Samsung/R2-2001623 | CR to TS 36.331: Reason for change: T312 expiry code point is introduced in the SCGFailureInformationNR message for (NG)EN-DC. | The reason for changes seems reasonable but the CR proposes two alternatives |

Recommended Proposal#2: RAN2 is requested to quickly check the two alternatives proposed in [R2-2001623] and agree one of them.

## 2.2	T312 configuration for CHO events

Issue#3: Whether configuration of T312 in the CHO event configuration is allowed? If allowed, then the trigger condition to start T312.

Configuration of T312 for SpCell is agreed and also captured in running CR based on the outcome of email discussion [108#16]. The T312 configuration is a flag within ReportConfig within the event triggered configuration and the T312 value is in MeasObject. In the running CRs as outcome of email discussion [108#34] and [108#35] the CHO events are introduced in ReportConfig as a CHOICE between triggered events and CHO events. The below Tdoc proposes to introduce T312 configuration as part of CHO events and specify trigger condition to start T312 for CHO events.

| Company/Tdoc | Proposals from Tdoc | Rapporteur Remark |
| --- | --- | --- |
| Samsung/R2-2001609 | Proposal 1: In LTE and NR, Timer T312 can be configured as part of events configured as CHO condition. | This is new issue not discussed as part of [108#66] |
|  | Proposal 2: T312 configured for CHO condition is started when the entry condition is not satisfied for a candidate cell on expiry of its corresponding timeToTrigger, while T310 is running. |  |

Recommended Proposal#3: RAN2 is requested to discuss the two proposals in [R2-2001609].

## 2.3	T312 handling upon reception of CHO configuration

As mentioned earlier, the issue raised in below Tdoc is discussed as part of email discussion [108#66]. It is expected the rapporteur report of email discussion [108#66] will be discussed in RAN2#109-e. Therefore, no proposal is recommended on the below Tdoc in this summary report.

| Company/Tdoc | Proposal from Tdoc | Rapporteur Remark |
| --- | --- | --- |
| OPPO/R2-2000652 | Proposal 1: Reception of CHO configuration does not stop T312. | Aligned with Proposal from [108#66] report |

# 3	Conclusions

Agreements proposed to be agreed in this meeting

Recommended Proposal#1: RAN2 is requested to agree the proposals in [R2-2000928] and adopt in running CR the proposed TP.

After the meeting starts on 24th Feb, companies can provide view if they object the proposal

| Company | If you object Proposal#1, then please provide the technical reason for objection |
| --- | --- |

Recommended Proposal#2: RAN2 is requested to quickly check the two alternatives proposed in [R2-2001623] and agree one of them.

After the meeting starts on 24th Feb, companies can provide view which alternative they prefer.

| Company | If you agree with Proposal#2, please indicate the alternative you prefer and why? |
| --- | --- |

| Company | If you object Proposal#2, then please provide the technical reason for objection |
| --- | --- |

Open items proposed to be further discussed in this meeting

DISC S1_1: Whether configuration of T312 in the CHO event configuration is allowed? If allowed, then the trigger condition to start T312.

Recommended Proposal#3: RAN2 is requested to discuss the two proposals in [R2-2001609].

After the meeting starts on 24th Feb, company views can be solicited on Proposal#3.

# 4	List of referenced documents

[5] R2-1916281, Report from session on LTE legacy, RAN2#108 Reno, USA, 18 – 22 Oct 2019