3GPP TSG-RAN WG2 Meeting #109	R2-200xxxx

# E-meeting, 24th Feb – 6th Mar 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 1445 | rev | 1 | Current version: | 15.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on introduction of extended capabilities for NR-DC only BCs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2020-02-13 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the current RF-ParametersMRDC structure, the field supportedBandCombinationList is used to indicate the EN-DC only BCs, or the BCs that support both EN-DC and NE-DC. And the fields with suffix "-v1540/ v1550/ v1560/ v1570/ v1580" are used to indicate the extended features/capabilities for each BC in the supportedBandCombinationList. For the NE-DC only BCs, a new field supportedBandCombinationListNEDC-Only is introduced, the corresponding Asn.1is copied as follow: RF-ParametersMRDC information element -- ASN1START -- TAG-RF-PARAMETERSMRDC-START RF-ParametersMRDC ::= SEQUENCE { supportedBandCombinationList BandCombinationList OPTIONAL, appliedFreqBandListFilter FreqBandList OPTIONAL, ..., [[ srs-SwitchingTimeRequested ENUMERATED {true} OPTIONAL, supportedBandCombinationList-v1540 BandCombinationList-v1540 OPTIONAL ]], [[ supportedBandCombinationList-v1550 BandCombinationList-v1550 OPTIONAL ]], [[ supportedBandCombinationList-v1560 BandCombinationList-v1560 OPTIONAL, supportedBandCombinationListNEDC-Only BandCombinationList OPTIONAL ]], [[ supportedBandCombinationList-v1570 BandCombinationList-v1570 OPTIONAL ]], [[ supportedBandCombinationList-v1580 BandCombinationList-v1580 OPTIONAL ]] } -- TAG-RF-PARAMETERSMRDC-STOP -- ASN1STOP However, for the NE-DC only BCs, only the BandCombinationList structure was included in supportedBandCombinationListNEDC-Only, therefore the UE is unable to report the extended features for the NE-DC only BCs. This CR aims to introduce the extensions for the NE-DC only BCs in supportedBandCombinationListNEDC-Only. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the supportedBandCombinationListNEDCOnly-v15xy field to the RF-ParametersMRDC, includes the extended features (i.e.BandCombinationList-v1540/v1560/v1580) for the NE-DC only BCs. Impact analysis Impacted 5G architecture options: NE-DC Impacted functionality: UE capability Inter-operability: - If UE implements according to the CR and the network does not, the network will ignore the extended features sent by UE for NE-DC only BCs, and it is unclear for network how to interpret whether those features are supported or not; - If the network implements according to the CR and the UE does not, the UE is unable to report extended capabilities for NE-DC only BCs. The network may assume those features are not supported by UE, and results in low performance. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE is unable to report the extended capabilities(e.g. BandCombinationList-v1540/ v1560/ v1580) for the NE-DC only BCs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of change

6.3.3	UE capability information elements

–	RF-ParametersMRDC

The IE RF-ParametersMRDC is used to convey RF related capabilities for MR-DC.

RF-ParametersMRDC information element

-- ASN1START

-- TAG-RF-PARAMETERSMRDC-START

RF-ParametersMRDC ::=                   SEQUENCE {

supportedBandCombinationList            BandCombinationList                 OPTIONAL,

appliedFreqBandListFilter               FreqBandList                        OPTIONAL,

...,

[[

srs-SwitchingTimeRequested              ENUMERATED {true}                   OPTIONAL,

supportedBandCombinationList-v1540      BandCombinationList-v1540           OPTIONAL

]],

[[

supportedBandCombinationList-v1550      BandCombinationList-v1550           OPTIONAL

]],

[[

supportedBandCombinationList-v1560      BandCombinationList-v1560           OPTIONAL,

supportedBandCombinationListNEDC-Only   BandCombinationList                 OPTIONAL

]],

[[

supportedBandCombinationList-v1570      BandCombinationList-v1570           OPTIONAL

]],

[[

supportedBandCombinationList-v1580      BandCombinationList-v1580           OPTIONAL

]],

[[

supportedBandCombinationListNEDC-Only-v15xy ::=   SEQUENCE {

supportedBandCombinationListNEDC-Only1      BandCombinationList-v1540   OPTIONAL,

supportedBandCombinationListNEDC-Only2      BandCombinationList-v1560   OPTIONAL,

supportedBandCombinationListNEDC-Only3      BandCombinationList-v1580   OPTIONAL,

...

}                                                                           OPTIONAL

]]

}

-- TAG-RF-PARAMETERSMRDC-STOP

-- ASN1STOP

RF-ParametersMRDC field descriptions

appliedFreqBandListFilter In this field the UE mirrors the FreqBandList that the NW provided in the capability enquiry, if any. The UE filtered the band combinations in the supportedBandCombinationList in accordance with this appliedFreqBandListFilter.

supportedBandCombinationList A list of band combinations that the UE supports for (NG)EN-DC and/or NE-DC. The FeatureSetCombinationId:s in this list refer to the FeatureSetCombination entries in the featureSetCombinations list in the UE-MRDC-Capability IE.

supportedBandCombinationListNEDC-Only A list of band combinations that the UE supports only for NE-DC. The FeatureSetCombinationId:s in this list refer to the FeatureSetCombination entries in the featureSetCombinations list in the UE-MRDC-Capability IE.

supportedBandCombinationListNEDC-Only1, supportedBandCombinationListNEDC-Only2, supportedBandCombinationListNEDC-Only3 The UE shall include the same number of entries, and listed in the same order, as in BandCombinationList of supportedBandCombinationListNEDC-Only field.

End of change