3GPP TSG-RAN WG2 Meeting #109e	R2-200xxxx

Online, 24 February – 6 March 2020

Agenda item:	7.2.10

Source:	Huawei  (offline email discussion rapporteur)

Title:	Revised Report of [AT109e][310][ NBIOT] 5GC open issues in AI 7.2.10  (Huawei)

Document for:	Report

# 1	Scope of the offline email discussion

This document contains the summary of the offline email discussion “[AT109e][310][NBIOT] 5GC open issues in AI 7.2.10 ”, as indicated below:

[AT109e][310][NBIOT] 5GC open issues in AI 7.2.10  (Huawei)

Scope: Progress the open issues and proposals listed in R2-2002015, not already agreed.

Intended outcome: Report.

Deadline: 06-03-2020, 12:00 CET

The proposals from R2-2002015 [1] below were agreed in the first session of RAN2-109e:

Agreements - Similar as UP CIoT EPS Optimization, rrc-SuspendIndication in RRCConnectionReject can be supported for UP CIoT 5GS Optimization. No change for specification is needed. - DL channel quality report can be supported for both NB-IoT and eMTC connected to 5GC. - Confirm the working assumption that cause delayTolerantAccess it not applicable to 5GC. - Confirm the working assumption that there is no need for an indication of extended Idle mode DRX support in system information for NB-IoT. - Confirm the working assumption that there is a new IE up-EDT-5GC-r16 in SIB2-BR/SIB2-NB to indicate ng-eNB connected to 5GC supports CP MO-EDT. - Revert the working assumption that the values ‘n’ and ‘m’ for the truncation of the 5G-S-TMSI are signalled per PLMN in SystemInformationBlockType2-NB. - Remove the IE cp-ReestablishmentPLMNList-5GC-r16 in SystemInformationBlockType2-NB. - The existing capability multipleDRB-r13 is also applicable to 5GC - PUR is supported in EPC and 5GC. - Introduce separate indications up-PUR-5GC-r16 and cp-PUR-5GC-r16 in SIB2-BR/SIB2-NB - Introduce separate UE capabilities pur-UP-5GC-r16 and pur-CP-5GC-r16. - Add ab-PerRSRP-r16 parameter (same definition as SIB14-BR) in SIB25-BR. - BL UEs or UEs in CE in RRC_CONNECTED mode performs access barring check based on the latest UAC parameters acquired prior to entering RRC_CONNECTED.

In [1], it was indicated that proposals 1, 2, 3 and 4 in [3] are comments on the NB-IoT and eMTC running CRs and should be discussed in the e-mail discussion in the running CRs. They are not discussed here.

In [1], it was indicated that proposal 8 in [3] and proposals 1, 2 and 3 in [7] should be discussed with ping-pong between CN types in 5GC. They are not discussed here.

In [1], it was indicated that proposals 1 and 2 in [8] should be postponed. They are not discussed here.

The document discusses the other remaining proposals and open issues in [1].

# 2	Discussion

## 2.1	RRC connection re-establishment for CP in NB-IoT

In [2], it is proposed that for 5GC, CP re-establishment is always enabled and there is no need for an indication in system information. This is based on absence of legacy eNB.

Offline Discussion Point 1:  Please provide comment on whether you agree  or disagree with the above proposal.

| Company | do you agree with the proposal | Comments |
| --- | --- | --- |
| QC | Yes |  |
| Ericsson | Yes | The comment raised online was related to that there might be eNBs not supporting the Rel-14 feature, and in practice the proposal makes re-establishment support mandatory from NW side. |
| Huawei, HiSilicon | Yes |  |
| LG | Yes |  |

Conclusion:

All companies agree with the proposal.

Proposal:

Proposal S1-1: For 5GC, CP re-establishment is always enabled and there is no need for an indication in system information

## 2.2	Access barring for eMTC

System information update mechanism for SIB25-BR in 5GC

In [5], it is proposed that systemInformationBlockType25-BR follows the same system information update mechanism as SIB14-BR and does not affect the value tag.

Offline Discussion Point 2:  Please provide comment on whether you agree or disagree with the above proposal.

| Company | Do you agree with the proposal | Comments |
| --- | --- | --- |
| QC | yes |  |
| Ericsson | Yes |  |
| Huawei, HiSilicon | Yes |  |
| LG | Yes |  |

Conclusion:

All companies agree with the proposal.

Proposal:

Proposal S2-1: systemInformationBlockType25-BR follows the same system information update mechanism as SIB14-BR and does not affect the value tag

System information update notification for SIB25-BR in 5GC

In [4], it is proposed that a new parameter uac-ParamModification (similar to eab-ParamModification) is introduced in the Paging message and a new parameter systemInfoModification-UAC (similar to eab-ParamModification ) in introduced in the Direct Indication Information to indicate SIB25-BR modification and scheduling.

In [5], it is proposed that a new parameter uac-ParamModification (similar to eab-ParamModification) is introduced in the Paging message and in the Direct Indication Information to indicate SIB25-BR modification and scheduling.

Offline Discussion Point 3:  Do you agree with introducing a notification in Paging message and Direct Indication Information. If yes, please provide comment on the meaning on the indication:

- option a): same indication (e.g. uac-ParamModification) in paging message and Direct Indication information with similar handling to eab-ParamModification in EPC

- option b): one indication (e.g. uac-ParamModification) in paging message with similar handling to eab-ParamModification in EPC and one indication in Direct Indication information (e.g. systemInfoModification-UAC) with similar handling to systemInfoModification-eDRX in EPC

- option c). Other, please describe

| Company | Do you agree with introducing notification ? option a, b or c ? | Comments |
| --- | --- | --- |
| QC | A (means separate indicators for EPC and 5GC UEs) | The access barring can be enabled in a cell due to access network issues or core network issues. The current indication is sufficient for access control due to access network issues regardless of core network (EPC or 5GC) serving the UE. But if access needs to be controlled due to 5GC then it does not make sense to impact UEs that are served by EPC. Depending on whether UE wants to access EPC or 5GC, UE uses appropriate access barring mechanisms. |
| Ericsson | Introducing notifications is OK Tend to think a) better | We think a), i.e. in a way re-using existing mechanisms should be fine. |
| Huawei, HiSilicon | a) | a) is reusing the EPS mechanisms for EAB but using separate flags for EPC and 5GC to avoid impacting each other. we are confused about b). the indications in Paging message and Direct Information are always the same. so we wonder if the proposals is to introduce two new indications: uac-ParamModification and systemInfoModification-eDRX-5GC in both Paging message and Direct Information. If this is the case, then we think the proposed systemInfoModification-eDRX-5GC is not related to access control. |
| LG | A | The separate indication for each core network would be needed. The suggested name in proposals are fine to us; uac-ParamModification in Paging and systemInfoModificaiton-uac. |

Conclusion:

All companies support option a).

Proposals:

Proposal S2-2: A new parameter uac-ParamModification (similar to eab-ParamModification) is introduced in the Paging message and in the Direct Indication Information to indicate SIB25-BR modification and scheduling

# 3	Summary

Conclusions:

Potential easy agreements

Proposal S1-1: For 5GC, CP re-establishment is always enabled and there is no need for an indication in system information

Proposal S2-1: systemInformationBlockType25-BR follows the same system information update mechanism as SIB14-BR and does not affect the value tag

Proposal S2-2: A new parameter uac-ParamModification (similar to eab-ParamModification) is introduced in the Paging message and in the Direct Indication Information to indicate SIB25-BR modification and scheduling.

# 4	List of referenced documents

[1] R2-2002015 	Summary of contributions for connection to 5GC  (AI 7.2.10)	Huawei	discussion	Rel-16	NB_IOTenh3-Core, LTE_eMTC5-Core

[2] R2-2000647 	Miscellaneous for NB-IoT and eMTC RRC CRs	Huawei, HiSilicon	discussion	Rel-16	NB_IOTenh3-Core, LTE_eMTC5-Core

[3] R2-2000517 	Remaining FFSs for connection to 5GC	ZTE Corporation, Sanechips	discussion	Rel-16	LTE_eMTC5-Core, NB_IOTenh3-Core

[4] R2-2000539 	UAC information change indication for eMTC UE connected to 5GC Qualcomm Incorporated

[5] R2-2000648 	Access barring for eMTC connected to 5GC Huawei, HiSilicon

[6] R2-2000540 	Email discussion report [108#97] for how to minimize ping-pong between CN types in RRC_IDLE/RRC_INACTIVE 	Qualcomm India Pvt Ltd	discussion	Rel-16	LTE_eMTC5-Core, NB_IOTenh3-Core

[7] R2-2001014 	UE redirection to a specific CN type and ping-pong behavior	Sony Europe B.V.	discussion	NB_IOTenh3-Core

[8] R2-2001478 	AS RAI and optimization of release in EDT	Ericsson	discussion	LTE_eMTC5-Core, NB_IOTenh3-Core	Late