3GPP TSG-RAN WG2 Meeting #109-e	draftR2-2001871

# Online, 24th February – 6th March 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.306 | CR | 1735 | rev | 1 | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of additional enhancements for eMTC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eMTC5-Core |  |  |  |  |  | Date: |  |  | 2019-03-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduction of UE capabilities related to additional enhancements for eMTC in Rel-16 |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following capabilities and agreements have been captured in this CR: MT-EDT: RAN2#108:  If the UE supports MT-EDT (optional) for the CP solution, it shall also support MO-EDT for the CP solution.  If the UE supports MT-EDT (optional) for the UP solution, it shall also support MO-EDT for the UP solution.  Support of MT-EDT is optional at the UE without AS capability.  It should be possible for the UE to indicate separate capability for CP MT-EDT and UP MT-EDT at NAS level. PUR RAN2#107bis:  PUR configuration can be provided without PUR Configuration Request from the UE, therefore optional radio access capabilities (separate for UP and CP) to indicate UE is capable of performing UL transmissions using PUR are introduced. Multiple TB Scheduling RAN2#106  UE capability for multiple TB is indicated separately for CE Mode A and CE mode B.  UE capability for multiple TB is indicated separately for uplink and downlink. Channel Quality Report RAN2#107  UE capability reporting is not needed for the DL quality report in Msg3 in idle mode.  UE capability reporting is introduced for the aperiodic DL quality report in connected mode. MPDCCH performance improvement using CRS RAN2#104  UEs report capability to indicate support of using CRS for improving MPDCCH channel estimation. ETWS/CMAS indication reception in RRC CONNECTED RAN2#107bis  Introduce capability for support of ETWS/CMAS indication reception in RRC CONNECTED by UE supporting CE Mode A.  Introduce capability for support of ETWS/CMAS indication reception in RRC CONNECTED by UE supporting CE Mode B. Feedback based on CSI-RS RAN2#108  For feedback based on CSI-RS, capability bit is introduced only for CE Mode A. Stand-alone deployment RAN2#105  Introduce UE capability to indicate that UE supports unicast PDSCH reception in LTE control channel region. Connection to 5GC RAN2#106  RRC_INACTIVE state with short eDRX cycles is optionally supported for eMTC connected to 5GC with capability signalling.  UP optimization solution is supported for both eMTC and NB-IoT connected to 5GC with capability signalling. RAN2#107  Control Plane optimization feature is optional for eMTC devices connecting to 5GC and mandatory for NB-IoT devices connected to 5GC.  Support of User Plane CIoT 5GS optimization is optional for both eMTC and NB-IoT devices connected to 5GC without capability signaling. Indication for support is provided in Msg5, i.e. RRCConnectionSetupComplete.  For eMTC, introduce new parameters, up-CIoT-5GS-Optimisation-r16 and cp-CIoT-5GS-Optimisation-r16 in RRCConnectionSetupComplete when accessing 5GC.  Introduce a new UE capability earlyData-UP-5GC-r16 in UE-EUTRA-Capability, UE-Capability-NB to indicate support of UP MO-EDT in 5GC.  No additional capability is needed to indicate support for short extended DRX operation in RRC_INACTIVE state.  UE signaling category M1 shall set the LTE-M indication in RRCConnectionSetupComplete message.  Introduce ‘LTE-M Indication enumerated {TRUE} optional’ in RRCConnectionSetupComplete message. RAN2#108  For EPS it is optional for a UE to support AS RAI, with capability reporting. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | WI cannot be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.3, 4.3.4.xa(new), 4.3.4.xb(new), 4.3.4.xc(new), 4.3.4.xd(new), 4.3.4.xe(new), 4.3.4.xf(new), 4.3.4.xg(new), 4.3.6.xa(new), 4.3.8.7, 4.3.8.xa(new), 4.3.8.xb(new), 4.3.19.xa(new), 4.3.29.xa(new), 4.3.29.xb(new) 4.3.36.xa(new), 4.3.36.xb(new), 6.8.4, 6.8.xa(new), 6.8.xb(new), 6.17.2, 6.xy (new), 6.xy.a (new), 6.xy.b (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS/TR 36.331 CR xxxx |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR 36.304 CR xxxx |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 36.323: "Evolved Universal Terrestrial Radio Access (E-UTRA) Packet Data Convergence Protocol (PDCP) specification".

[3]	3GPP TS 36.322: "Evolved Universal Terrestrial Radio Access (E-UTRA) Radio Link Control (RLC) specification".

[4]	3GPP TS 36.321: "Evolved Universal Terrestrial Radio Access (E-UTRA) Medium Access Control (MAC) specification".

[5]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA) Radio Resource Control (RRC) specification".

[6]	3GPP TS 36.101: "Evolved Universal Terrestrial Radio Access (E-UTRA) radio transmission and reception".

[7]	IETF RFC 5795: "The RObust Header Compression (ROHC) Framework".

[8]	IETF RFC 6846: "RObust Header Compression (ROHC): A Profile for TCP/IP (ROHC-TCP)".

[9]	IETF RFC 3095: "RObust Header Compression (RoHC): Framework and four profiles: RTP, UDP, ESP and uncompressed".

[10]	IETF RFC 3843: "RObust Header Compression (RoHC): A Compression Profile for IP".

[11]	IETF RFC 4815: "RObust Header Compression (ROHC): Corrections and Clarifications to RFC 3095".

[12]	IETF RFC 5225: "RObust Header Compression (ROHC) Version 2: Profiles for RTP, UDP, IP, ESP and UDP Lite".

[13]	3GPP TS 36.355: "Evolved Universal Terrestrial Radio Access (E-UTRA) LTE Positioning Protocol (LPP)".

[14]	3GPP TS 36.304: "Evolved Universal Terrestrial Radio Access (E-UTRA); UE Procedures in Idle Mode".

[15]	3GPP TS 37.320: "Universal Terrestrial Radio Access (UTRA) and Evolved Universal Terrestrial Radio Access (E-UTRA); Radio measurement collection for Minimization of Drive Tests (MDT); Overall description; Stage 2".

[16]	3GPP TS 36.133: "Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management".

[17]	3GPP TS 36.211: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical Channels and Modulation".

[18]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[19]	3GPP TS 23.216: "Single Radio Voice Call Continuity (SRVCC)".

[20]	3GPP TS 25.307: "Requirement on User Equipments (UEs) supporting a release-independent frequency band".

[21]	3GPP TS 24.312: "Access Network Discovery and Selection Function (ANDSF) Management Object (MO)".

[22]	3GPP TS 36.213: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures".

[23]	3GPP TS 36.214: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer - Measurements".

[24]	3GPP TS 23.303: "Proximity-based services (ProSe); Stage 2".

[25]	3GPP TS 36.314: "Evolved Universal Terrestrial Radio Access (E-UTRA); Layer 2- Measurements".

[26]	3GPP TS 36.212: "Evolved Universal Terrestrial Radio Access (E-UTRA); Multiplexing and channel coding".

[27]	3GPP TS 36.307: "Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements on User Equipments (UEs) supporting a release-independent frequency band".

[28]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3".

[29]	3GPP TS 23.285: "Technical Specification Group Services and System Aspects; Architecture enhancements for V2X services".

[30]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access (E-UTRAN); Overall description; Stage 2".

[31]	3GPP TS 23.246: "Multimedia Broadcast/Multicast Service (MBMS); Architecture and functional description".

[32]	3GPP TS 38.306 "NR; UE Radio Access Capabilities".

[33]	3GPP TS 38.101-1: "NR User Equipment (UE) radio transmission and reception Part 1: Range 1 Standalone".

[34]	3GPP TS 38.101-2: "NR User Equipment (UE) radio transmission and reception Part 2: Range 2 Standalone".

[35]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification".

[36]	3GPP TS 38.215: "NR; Physical layer measurements".

[37]	3GPP TS 38.133: "NR; Requirements for support of radio resource management".

[xx]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3"

NEXT CHANGE

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

1xRTT	CDMA2000 1x Radio Transmission Technology

ACK	Acknowledgement

ACDC	Application specific Congestion control for Data Communication

ANDSF	Access Network Discovery and Selection Function

BCCH	Broadcast Control Channel

CG	Cell Group

CRS	Cell-specific Rerefence Signal

CSG	Closed Subscriber Group

CSI	Channel State Information

DC	Dual Connectivity

DCI	Downlink Control Information

DL-SCH	Downlink Shared Channel

E-UTRA	Evolved Universal Terrestrial Radio Access

E-UTRAN	Evolved Universal Terrestrial Radio Access Network

FDD	Frequency Division Duplex

GERAN	GSM/EDGE Radio Access Network

HARQ	Hybrid Automatic Repeat Request

HRPD	High Rate Packet Data

IRC	Interference Rejection Combining

MAC	Medium Access Control

MMSE	Minimum Mean Squared Error

MO-EDT	Mobile Originated Early Data Transmission

MRO	Mobility Robustness Optimisation

MT-EDT	Mobile Terminated Early Data Transmission

MTSI	Multimedia Telephony Service for IMS

MUST	MultiUser Superposition Transmission

NAICS	Network Assisted Interference Cancellation/Suppression

NB-IoT	Narrow Band Internet of Things

OS	OFDM Symbol

PCell	Primary Cell

PDCCH	Physical Downlink Control Channel

PDCP	Packet Data Convergence Protocol

PDSCH	Physical Downlink Shared Channel

PHR	Power Headroom Reporting

ProSe	Proximity-based Services

PUCCH	Physical Uplink Control Channel

PUR	Preconfigured Uplink Resource

PUSCH	Physical Uplink Shared Channel

QoE	Quality of Experience

RACH	Random Access CHannel

RAI	Release Assistance Indication

RAT	Radio Access Technology

RLC	Radio Link Control

ROHC	RObust Header Compression

RRC	Radio Resource Control

SC-PTM	Single Cell Point to Multipoint

SCC	Secondary Component Carrier

SCell	Secondary Cell

SI	System Information

SL	Sidelink

SL-DCH	Sidelink Discovery CHannel

SL-SCH	Sidelink Shared CHannel

SON	Self Organizing Networks

SPT	Short Processing Time

SR	Scheduling Request

SSAC	Service Specific Access Control

SSTD	SFN and Subframe Timing Difference

STTI	Short TTI

TDD	Time Division Duplex

TTI	Transmission Time Interval

UCI	Uplink Control Information

UDC	Uplink Data Compression

UE	User Equipment

UL-SCH	Uplink Shared Channel

UMTS	Universal Mobile Telecommunications System

UTRA	UMTS Terrestrial Radio Access

V2X	Vehicle-to-Everything

WLAN	Wireless Local Area Network

NEXT CHANGE

#### 4.3.4.181	srs-DCI7-TriggeringFS2-r15

This field indicates whether the UE supports SRS triggerring via DCI format 7 for FS2.

#### 4.3.4.xa	ce-ModeA-PUSCH-MultiTB-r16

This field indicates whether the UE supports multiple TB scheduling in the uplink as specified in TS 36.213 [22]. This feature is only applicable if the UE supports ce-ModeA-r13.

#### 4.3.4.xb	ce-ModeA-PDSCH-MultiTB-r16

This field indicates whether the UE supports multiple TB scheduling in the downlink as specified in TS 36.213 [22]. This feature is only applicable if the UE supports ce-ModeA-r13.

#### 4.3.4.xc	ce-ModeB-PUSCH-MultiTB--r16

This field indicates whether the UE supports multiple TB scheduling in the uplink in CE Mode B as specified in TS TS 36.213 [22]. This feature is only applicable if the UE supports ce-ModeB-r13.

#### 4.3.4.xd	ce-ModeB-PDSCH-MultiTB-r16

This field indicates whether the UE supports multiple TB scheduling in the downlink in CE Mode B as specified in TS 36.213 [22]. This feature is only applicable if the UE supports ce-ModeB-r13.

#### 4.3.4.xe	ce-ModeA-CSI-RS-Feedback-r16

This field indicates whether the UE supports CSI-RS based feedback when the UE is operating in coverage enhancement mode A, as specified in TS 36.213 [22]. This feature is only applicable if the UE supports ce-ModeA-r13 and a UE Category other than Category M1 and M2.

#### 4.3.4.xf	ce-RxInLTEControlRegion-r16

This field indicates whether the UE supports PDSCH or MPDCCH reception in the LTE control channel region feedback when the UE is operating in coverage enhancement mode A or Bas specified in TS 36.211 [17]. This feature is only applicable if the UE supports ce-ModeA-r13.

#### 4.3.4.xg	ce-CRS-ChannelEstMPDCCH-r16

This field defines whether the UE supports CRS for improving MPDCCH channel estimation, as specified in TS 36.211 [17]. This feature is only applicable if the UE supports ce-ModeA-r13.

NEXT CHANGE

#### 4.3.6.36	measGapPatterns-r15

This field defines whether the UE that supports NR supports gap patterns 4 to 11 in LTE standalone as specified in TS 36.133 [16], and for independent measurement gap configuration on FR1 and per-UE gap in (NG)EN-DC as specified in TS38.133 [37].

#### 4.3.6.xa	dl-ChannelQualityReporting-r16

This field defines whether the UE supports DL channel quality reporting of the serving cell or configured carrier for FDD in RRC_CONNECTED, as specified in TS 36.331 [5]. This feature is only applicable if the UE supports ce-ModeA-r13 or if the UE supports any ue-Category-NB.

Editor’s note: Whether to have a common or separate capability with NB-IoT, and how to name it if common.

NEXT CHANGE

#### 4.3.8.7	earlyData-UP-r15

This field defines whether the UE supports MO-EDT for User Plane CIoT EPS optimizations for FDD, as defined in TS 24.301 [28]. This feature is only applicable if the UE supports ce-ModeA-r13 or if the UE supports any ue-Category-NB.

#### 4.3.8.8	void

#### 4.3.8.9	extendedNumberOfDRBs-r15

This field defines whether the UE supports up to 15 DRBs. The UE shall support any combination of RLC AM and RLC UM entities for the configured DRBs. A UE that supports extendedNumberOfDRBs-r15 shall also support the extended LCID as specified in TS 36.321 [4].

#### 4.3.8.10	reducedCP-Latency-r15

This field defines whether the UE supports reduced control plane latency as defined in TS 36.213 [22] and TS 36.331 [5].

#### 4.3.8.xa	pur-CP-EPC-r16

This field indicates whether the UE supports Transmission using PUR for Control Plane CIoT EPS optimisation as specified TS 36.300 [30]. This feature is only applicable if the UE supports ce-ModeA-r13, or for FDD if the UE supports any ue-Category-NB.

#### 4.3.8.xb	pur-UP-EPC-r16

This field indicates whether the UE supports Transmission using PUR for User Plane CIoT EPS optimisation as specified TS 36.300 [30]. This feature is only applicable if the UE supports ce-ModeA-r13, or for FDD if the UE supports any ue-Category-NB.

NEXT CHANGE

#### 4.3.19.21	eLCID-Support-r15

This field indicates whether the UE supports LCID "10000" and MAC PDU subheader containing the eLCID field as specified in TS 36.321 [4].

#### 4.3.19.xa	rai-SupportEnh-r16

This field defines whether the UE supports 2 bit Release Assistance Indication (RAI) when connected to EPC as specified in TS 36.321 [4]. This feature is only applicable if the UE supports ce-ModeA-r13 or if the UE supports any ue-Category-NB.

### 4.3.20	Dual Connectivity parameters

NEXT CHANGE

#### 4.3.29.12	tm6-CE-ModeA-r13

This field indicates whether the UE supports tm6 operation in CE mode A as specified in TS 36.213 [22] and TS 36.331 [5]. A UE indicating support of tm6-CE-ModeA-r13 shall also indicate support of ce-ModeA-r13.

#### 4.3.29.xa	ce-ModeA-ETWS-CMAS-RxInConn-r16

This field indicates whether the UE supports ETWS/CMAS indication reception in RRC_CONNECTED state when the UE is operating in coverage enhancement mode A as specified in TS 36.331 [5]. This feature is only applicable if the UE supports ce-ModeA-r13 except for Category M1 and Category M2 UEs.

#### 4.3.29.xb	ce-ModeB-ETWS-CMAS-RxInConn-r16

This field indicates whether the UE supporting CE Mode B supports ETWS/CMAS indication reception in RRC CONNECTED state when the UE is operating in coverage enhancement mode B as specified in TS 36.331 [5]. This feature is only applicable if the UE supports ce-ModeB-r13 and a UE Category other than Category M1 and M2.

NEXT CHANGE

#### 4.3.36.8	reflectiveQoS-r15

This field indicates whether the UE supports AS reflective QoS.

#### 4.3.36.xa	ce-RRC-INACTIVE-r16

This field indicates whether the UE supports RRC_INACTIVE state with extended DRX cycles up to 10.24s without PTW when the UE is operating in coverage enhancement mode A or B as specified in TS 36.331 [5] . This feature is only applicable if the UE supports ce-ModeA-r13.

#### 4.3.36.xb	earlyData-UP-5GC-r16

This field indicates whether the UE supports MO-EDT for User Plane CIoT 5GS optimisations, as defined in TS 24.501 [xx]. This feature is only applicable if the UE supports ce-ModeA-r13, or for FDD if the UE supports any ue-Category-NB.

NEXT CHANGE

### 6.8.4	MO-EDT for Control Plane CIoT EPS Optimization

It is optional for UE to support MO-EDT for Control Plane CIoT EPS optimizations for FDD, as defined in TS 24.301 [28]. This feature is only applicable if the UE supports ce-ModeA-r13 or if the UE supports any ue-Category-NB.

Editor’s note: FFS if we should have the equivalent for 5GS in section 6.xy.

### 6.8.5	Void

### 6.8.6	Enhanced PHR

It is optional for UE to support enhanced PHR in MSG3 for FDD, as defined in TS 36.321 [4]. This feature is only applicable if the UE supports any ue-Category-NB.

### 6.8.7	void

.

### 6.8.8	Resynchronization Signals

It is optional for UE to support resynchronization signals, as defined in TS 36.211 [17]. This feature is only applicable if the UE supports ce-ModeA-r13.

### 6.8.9	Measurement gaps for higher UE velocity

It is optional for UE to support measurement gaps for higher UE velocity, as defined in TS 36.331 [5] and TS 36.133[16]. This feature is only applicable if the UE supports ce-ModeA-r13.

### 6.8.xa	MT-EDT for Control Plane CIoT EPS Optimisation

It is optional for UE to support MT-EDT for Control Plane CIoT EPS Optimisation, as defined in TS 24.301 [28]. If the UE supports ‘MT-EDT for Control Plane CIoT EPS Optimisation’ it shall support ‘MO-EDT for Control Plane CIoT EPS Optimisation’ as described in clause 6.8.4. This feature is only applicable if the UE supports ce-ModeA-r13, or for FDD if the UE supports any ue-Category-NB.

### 6.8.xb	MT-EDT for User Plane CIoT EPS Optimisation

It is optional for UE to support MT-EDT for User Plane CIoT EPS Optimisation, as defined in TS 24.301 [28]. If the UE supports ‘MT-EDT for User Plane CIoT EPS Optimisation’ it shall support earlyData-UP-r15 as described in clause 4.3.8.7. This feature is only applicable if the UE supports ce-ModeA-r13, or for FDD if the UE supports any ue-Category-NB.

NEXT CHANGE

### 6.17.2	DL channel quality reporting in Msg3

It is optional for UE to support DL channel quality reporting of the serving cell for FDDin Msg3, as specified in TS 36.331 [5]. This feature is only applicable if the UE supports any ue-Category-NB or if the UE supports ce-ModeA-r13.

Editor’s note: Whether to have a common or separate capability with NB-IoT.

### 6.17.3	Serving cell idle mode measurements reporting

It is optional for UE to include measResultServCell-r14 in RRCConnectionRestablishmentComplete-NB, RRCConnectionResumeComplete-NB and RRCConnectionSetupComplete-NB messages as specified in TS 36.331 [5]. This feature is only applicable if the UE supports any ue-Category-NB.

### 6.17.4	NSSS-Based RRM measurements

It is optional for UE to support NSSS-Based RRM measurements for FDD, as specified in TS 36.211 [17] and TS 36.214 [23]. This feature is only applicable if the UE supports any ue-Category-NB.

### 6.17.5	NPBCH-Based RRM measurements

It is optional for UE to support NPBCH-Based RRM measurements for the serving cell for FDD, as specified in TS 36.214 [23]. This feature is only applicable if the UE supports any ue-Category-NB.

## 6.xy	E-UTRA/5GC Parameters

### 6.xy.a	User Plane CIoT 5GS optimisations

It is optional for UE to support User Plane CIoT 5GS optimisations for FDD, as defined in TS 24.501 [xx]. This feature is only applicable if the UE supports any ue-Category-NB or if the UE supports ce-ModeA-r13.

### 6.xy.b	Control Plane CIoT 5GS optimisations

It is optional for UE to support Control Plane CIoT 5GS optimisations for FDD, as defined in TS 24.501 [xx]. This feature is only applicable if the UE supports ce-ModeA-r13.

END OF CHANGES