3GPP TSG-RAN WG2 Meeting #109-e	R2-200xxxx

Online, 24th Feb - 6th Mar, 2020

Title:	[Draft] LS on DCP

Response to:

Release:	Release 16

Work Item:	NR_UE_pow_sav-Core

Source:	Huawei [to be RAN2]

To:	RAN1

Cc:

Contact Person:

Name:	Baokun Shan

E-mail Address:	baokun.shan@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

In RAN2#109-e, RAN2 discussed the MAC-PHY interactions for DCP (DCI with CRC scrambled by PS-RNTI) monitoring and the start of drx-onDurationTimer. The following understanding regarding how to capture DCP between MAC and PHY was agreed from RAN2 point of view:

MAC specification:

- MAC specifies the start of drx-onDurationTimer and Active Time, including:

- MAC should start drx-onDurationTimer according to indication provided by PHY

- MAC should start drx-onDurationTimer in case DCP is overlapped with Active time, measurement gap and BWP switching period

- MAC should start drx-onDurationTimer in case ps-Wakeup is set to true and no DCP indication is received by PHY

PHY specification:

- PHY specifies DCP monitoring, including:

- When to start the monitoring (ps_offset) and stop the monitoring (minimum gap based on UE capability)

- In case DCP is considered invalid from PHY perspective (scenarios FFS in RAN1), PHY should not monitor DCP and indicates to MAC to start the drx-onDurationTimer for the next DRX cycle

- PHY indicates to MAC whether a received DCP indicates to start the drx-onDurationTimer for the next DRX cycle or not.

- PHY should not specify the start of drx-onDurationTimer and Active Time.

The RAN1 LS (R1 -1913480) on CSI/SRS reporting has also been discussed in RAN2#109-e. RAN2 understands the intention to control L1-RSRP reporting separately when drx-onDurationTimer is not running due to DCP. There are two options to interpret the two flags for CSI/SRS reporting in RAN1 LS:

Option 1:

ps-TransmitPeriodicCSI = TRUE: Report all types of periodic CSI, including L1-RSRP (i.e. cri-RSRP and ssb-Index-RSRP)

ps-TransmitPeriodicL1-RSRP = TRUE: Only report L1-RSRP (i.e. cri-RSRP and ssb-Index-RSRP)

In this option, the two flags cannot both be set to TRUE and it is not possible to control the UE only to report periodic CSI apart from L1-RSRP.

Option 2:

ps-TransmitPeriodicCSI = TRUE: Report all types of periodic CSI apart from L1-RSRP (i.e. cri-RSRP and ssb-Index-RSRP)

ps-TransmitPeriodicL1-RSRP = TRUE: Only report L1-RSRP (i.e. cri-RSRP and ssb-Index-RSRP)

In this option, the two flags are independent and it is not possible to control the UE only to report periodic CSI apart from L1-RSRP.

2. Actions:

To RAN1:

RAN2 respectfully asks RAN1 to:

- Take above MAC-PHY interactions for DCP into account and update 38.213 running CR accordingly.

- Provide feedback on the preference of the two options for CSI reporting.

3. Date of Next TSG-RAN WG2 Meetings:

3GPP RAN2#109bis 		TBD    				TBD

3GPP RAN2#110		25 - 29 May, 2020   	 		Athens, Greece