3GPP TSG-RAN WG2 Meeting #109 electronic	 R2-200xxxx

Elbonia, 24 Feb – 6 Mar 2020

Agenda item:	6.8.1

Source:	Intel Corporation

Title:	Report of [AT109e][601][POS] Harmonise posSIB numbering across CRs (Intel)

Document for:	Discussion and Decision

# Introduction

This is the email discussion report on below email discussion:

- [AT109e][601][POS] Harmonise posSIB numbering across CRs (Intel)

Status: Started

Scope: Align posSIB numbering between the following CRs:

- R2-2001333 (38.331 running CR)

- R2-2001216 (36.331 CR to introduce PPP-RTK)

- R2-2001255 (38.331 CR to introduce on-demand SI request in connected mode)

- R2-2001230 (37.355 CR to introduce PPP-RTK)

- R2-2001234 (TPs to 37.355 to introduce UE-based DL positioning)

- R2-2000006 (37.355 CR to introduce barometric pressure broadcast)

- R2-2000188 (36.331 CR to introduce barometric pressure broadcast)

- R2-2000396 (36.331 CR to introduce TBS AD broadcast)

- R2-2000426 (37.355 CR to introduce TBS AD broadcast)

- R2-2000153 (37.355 CR to introduce NavIC)

- R2-2000157 (36.331 CR to introduce NavIC)

Intended Outcome: Agreed document listing the posSIB numbers, for reference by individual CR authors

Deadline: Thursday 2020-02-27 1200 CET

# Discussion

The posSIB number is defined in section 7.2 in LPP specification TS36.355 or TS37.355 as below, and to be used in RRC specification.

The supported posSibType's are specified in Table 7.2-1. The GNSS Common and Generic Assistance Data IEs are defined in sub-clause 6.5.2.2. The OTDOA Assistance Data IEs are defined in sub-clause 7.4.2.

Table 7.2-1: Mapping of posSibType to assistanceDataElement

|  | posSibType [12] | assistanceDataElement |
| --- | --- | --- |
| GNSS Common Assistance Data (clause 6.5.2.2) | posSibType1-1 | GNSS-ReferenceTime |
|  | posSibType1-2 | GNSS-ReferenceLocation |
|  | posSibType1-3 | GNSS-IonosphericModel |
|  | posSibType1-4 | GNSS-EarthOrientationParameters |
|  | posSibType1-5 | GNSS-RTK-ReferenceStationInfo |
|  | posSibType1-6 | GNSS-RTK-CommonObservationInfo |
|  | posSibType1-7 | GNSS-RTK-AuxiliaryStationData |
| GNSS Generic Assistance Data (clause 6.5.2.2) | posSibType2-1 | GNSS-TimeModelList |
|  | posSibType2-2 | GNSS-DifferentialCorrections |
|  | posSibType2-3 | GNSS-NavigationModel |
|  | posSibType2-4 | GNSS-RealTimeIntegrity |
|  | posSibType2-5 | GNSS-DataBitAssistance |
|  | posSibType2-6 | GNSS-AcquisitionAssistance |
|  | posSibType2-7 | GNSS-Almanac |
|  | posSibType2-8 | GNSS-UTC-Model |
|  | posSibType2-9 | GNSS-AuxiliaryInformation |
|  | posSibType2-10 | BDS-DifferentialCorrections |
|  | posSibType2-11 | BDS-GridModelParameter |
|  | posSibType2-12 | GNSS-RTK-Observations |
|  | posSibType2-13 | GLO-RTK-BiasInformation |
|  | posSibType2-14 | GNSS-RTK-MAC-CorrectionDifferences |
|  | posSibType2-15 | GNSS-RTK-Residuals |
|  | posSibType2-16 | GNSS-RTK-FKP-Gradients |
|  | posSibType2-17 | GNSS-SSR-OrbitCorrections |
|  | posSibType2-18 | GNSS-SSR-ClockCorrections |
|  | posSibType2-19 | GNSS-SSR-CodeBias |
| OTDOA Assistance Data (clause 7.4.2) | posSibType3-1 | OTDOA-UE-Assisted |

The new posSIBs introduced in Rel-16 are:

LTE:

- Barometric pressure R2-2000006 [6]:

| Barometric Assistance Data (clause 6.5.5.8) | posSibTypeX-Y | Sensor-AssistanceDataList |
| --- | --- | --- |

- TBS AD in R2-2000426 [9];

| TBS Assistance Data (clause 7.4.2) | posSibTypeX-Y | TBS-AssistanceDataList |
| --- | --- | --- |

- NavIC R2-2000153 [10];

| GNSS Generic Assistance Data (clause 6.5.2.2) | posSibType2-xx | NavIC-DifferentialCorrections |
| --- | --- | --- |
|  | posSibType2-yy | NavIC-GridModelParameter |

NR/LTE:

- PPP-RTK R2-2001230 [4];

|  | posSibType [12] | assistanceDataElement |
| --- | --- | --- |
| GNSS Common Assistance Data (clause 6.5.2.2) | posSibType1-8 | GNSS-SSR-CorrectionPoints |
| GNSS Generic Assistance Data (clause 6.5.2.2) | posSibType2-20 | GNSS-SSR-URA |
|  | posSibType2-21 | GNSS-SSR-PhaseBias |
|  | posSibType2-22 | GNSS-SSR-STEC-Correction |
|  | posSibType2-23 | GNSS-SSR-GriddedCorrection |

NR:

posSIB related agreements are

- Agreements:

- The mapping table 7.2-1 defined in TS36.355 is reused for A-GNSS, RTK, and LTE OTDOA

- Introduce new posSibType(s) for PPP-RTK

- Introduce new posSibType(s) for NR DL TDOA

- UE based DL positioning R2-2001234 [5], aligned with agreement “Split the position calculation assistance data into two separate posSIBs, one containing the TRP coordinates and one containing the RTDs.”;

'posSibTypeX-y' NR-UEB-TRP-LocationData

'posSibTypeX-z'. NR-UEB-TRP-RTD-Info

So far, the necessary posSIBs for PPP-RTK, NavIC, TBS aD and barometric pressure are quite stable. We should do the numbering for them first.

The example could be:

Table 7.2-1: Mapping of posSibType to assistanceDataElement

|  | posSibType [12] | assistanceDataElement |
| --- | --- | --- |
| GNSS Common Assistance Data (clause 6.5.2.2) | posSibType1-1 | GNSS-ReferenceTime |
|  | posSibType1-2 | GNSS-ReferenceLocation |
|  | posSibType1-3 | GNSS-IonosphericModel |
|  | posSibType1-4 | GNSS-EarthOrientationParameters |
|  | posSibType1-5 | GNSS-RTK-ReferenceStationInfo |
|  | posSibType1-6 | GNSS-RTK-CommonObservationInfo |
|  | posSibType1-7 | GNSS-RTK-AuxiliaryStationData |
|  | posSibType1-8 | GNSS-SSR-CorrectionPoints |
| GNSS Generic Assistance Data (clause 6.5.2.2) | posSibType2-1 | GNSS-TimeModelList |
|  | posSibType2-2 | GNSS-DifferentialCorrections |
|  | posSibType2-3 | GNSS-NavigationModel |
|  | posSibType2-4 | GNSS-RealTimeIntegrity |
|  | posSibType2-5 | GNSS-DataBitAssistance |
|  | posSibType2-6 | GNSS-AcquisitionAssistance |
|  | posSibType2-7 | GNSS-Almanac |
|  | posSibType2-8 | GNSS-UTC-Model |
|  | posSibType2-9 | GNSS-AuxiliaryInformation |
|  | posSibType2-10 | BDS-DifferentialCorrections |
|  | posSibType2-11 | BDS-GridModelParameter |
|  | posSibType2-12 | GNSS-RTK-Observations |
|  | posSibType2-13 | GLO-RTK-BiasInformation |
|  | posSibType2-14 | GNSS-RTK-MAC-CorrectionDifferences |
|  | posSibType2-15 | GNSS-RTK-Residuals |
|  | posSibType2-16 | GNSS-RTK-FKP-Gradients |
|  | posSibType2-17 | GNSS-SSR-OrbitCorrections |
|  | posSibType2-18 | GNSS-SSR-ClockCorrections |
|  | posSibType2-19 | GNSS-SSR-CodeBias |
|  | posSibType2-20 | NavIC-DifferentialCorrections |
|  | posSibType2-21 | NavIC-GridModelParameter |
|  | posSibType2-22 | GNSS-SSR-URA |
|  | posSibType2-23 | GNSS-SSR-PhaseBias |
|  | posSibType2-24 | GNSS-SSR-STEC-Correction |
|  | posSibType2-25 | GNSS-SSR-GriddedCorrection |
| OTDOA Assistance Data (clause 7.4.2) | posSibType3-1 | OTDOA-UE-Assisted |
| Barometric Assistance Data (clause 6.5.5.8) | posSibType4-1 | Sensor-AssistanceDataList |
| TBS Assistance Data (clause 7.4.2) | posSibType5-1 | TBS-AssistanceDataList |

Question 1: Do companies agree the above number allocation for TBS AD, barometric pressure, NavIC and PPP-RTK?

| Company | Yes/No | Remark |
| --- | --- | --- |

For NR posSIB, the agreement should be updated a bit to “Introduce new posSibType(s) for NR DL TDOA and DL AoD” since we have agreed that UE based positioning can be applied for DL-TDOA and DL AoD.

Question 2: Do companies agree the new posSIB should be applied for both NR DL TDOA and DL AoD?

| Company | Yes/No | Remark |
| --- | --- | --- |

We assume same as OTDOA, a new posSIB is sufficient for NR DL TDOA and NR DL AoD since the assistance data for them are same, except referencing cell. Then it will be:

| NR DL Assistance Data (clause 7.4.2) | posSibType6-1 | NR-DL-UE-Assisted |
| --- | --- | --- |

Question 3: Do companies agree the a new posSIB posSibType6-1 is introduced for both DL TDOA and DL AoD?

| Company | Yes/No | Remark |
| --- | --- | --- |

The posSIB for UE based positioning can be introduced under NR DL Asssitance data, i.e.

| NR DL Assistance Data (clause 7.4.2) | posSibType6-1 | NR-DL-UE-Assisted |
| --- | --- | --- |
|  | posSibType6-2 | NR-UEB-TRP-LocationData |
|  | posSibType6-3 | NR-UEB-TRP-RTD-Info |

Question 4: Do companies agree posSibType6-2 and posSibType6-3 for NR-UEB-TRP-LocationData and NR-UEB-TRP-RTD-Info?

| Company | Yes/No | Remark |
| --- | --- | --- |

Since so far, the posSIB for LTE and NR are specified in the same table, it would be good to add columns to show whether a posSIB is applied for LTE and/or NR.

The example is shown as below:

Table 7.2-1: Mapping of posSibType to assistanceDataElement

|  | posSibType [12] | assistanceDataElement | LTE | NR |
| --- | --- | --- | --- | --- |
| GNSS Common Assistance Data (clause 6.5.2.2) | posSibType1-1 | GNSS-ReferenceTime | Yes | Yes |
|  | posSibType1-2 | GNSS-ReferenceLocation | Yes | Yes |
|  | posSibType1-3 | GNSS-IonosphericModel | Yes | Yes |
|  | posSibType1-4 | GNSS-EarthOrientationParameters | Yes | Yes |
|  | posSibType1-5 | GNSS-RTK-ReferenceStationInfo | Yes | Yes |
|  | posSibType1-6 | GNSS-RTK-CommonObservationInfo | Yes | Yes |
|  | posSibType1-7 | GNSS-RTK-AuxiliaryStationData | Yes | Yes |
|  | posSibType1-8 | GNSS-SSR-CorrectionPoints | Yes | Yes |
| GNSS Generic Assistance Data (clause 6.5.2.2) | posSibType2-1 | GNSS-TimeModelList | Yes | Yes |
|  | posSibType2-2 | GNSS-DifferentialCorrections | Yes | Yes |
|  | posSibType2-3 | GNSS-NavigationModel | Yes | Yes |
|  | posSibType2-4 | GNSS-RealTimeIntegrity | Yes | Yes |
|  | posSibType2-5 | GNSS-DataBitAssistance | Yes | Yes |
|  | posSibType2-6 | GNSS-AcquisitionAssistance | Yes | Yes |
|  | posSibType2-7 | GNSS-Almanac | Yes | Yes |
|  | posSibType2-8 | GNSS-UTC-Model | Yes | Yes |
|  | posSibType2-9 | GNSS-AuxiliaryInformation | Yes | Yes |
|  | posSibType2-10 | BDS-DifferentialCorrections | Yes | Yes |
|  | posSibType2-11 | BDS-GridModelParameter | Yes | Yes |
|  | posSibType2-12 | GNSS-RTK-Observations | Yes | Yes |
|  | posSibType2-13 | GLO-RTK-BiasInformation | Yes | Yes |
|  | posSibType2-14 | GNSS-RTK-MAC-CorrectionDifferences | Yes | Yes |
|  | posSibType2-15 | GNSS-RTK-Residuals | Yes | Yes |
|  | posSibType2-16 | GNSS-RTK-FKP-Gradients | Yes | Yes |
|  | posSibType2-17 | GNSS-SSR-OrbitCorrections | Yes | Yes |
|  | posSibType2-18 | GNSS-SSR-ClockCorrections | Yes | Yes |
|  | posSibType2-19 | GNSS-SSR-CodeBias | Yes | Yes |
|  | posSibType2-20 | NavIC-DifferentialCorrections | Yes | No |
|  | posSibType2-21 | NavIC-GridModelParameter | Yes | No |
|  | posSibType2-22 | GNSS-SSR-URA | Yes | Yes |
|  | posSibType2-23 | GNSS-SSR-PhaseBias | Yes | Yes |
|  | posSibType2-24 | GNSS-SSR-STEC-Correction | Yes | Yes |
|  | posSibType2-25 | GNSS-SSR-GriddedCorrection | Yes | Yes |
| OTDOA Assistance Data (clause 7.4.2) | posSibType3-1 | OTDOA-UE-Assisted | Yes | No |
| Barometric Assistance Data (clause 6.5.5.8) | posSibType4-1 | Sensor-AssistanceDataList | Yes | No |
| TBS Assistance Data (clause 7.4.2) | posSibType5-1 | TBS-AssistanceDataList | Yes | No |
| NR DL Assistance Data (clause 7.4.2) | posSibType6-1 | NR-DL-UE-Assisted | No | Yes |
|  | posSibType6-2 | NR-UEB-TRP-LocationData | No | Yes |
|  | posSibType6-3 | NR-UEB-TRP-RTD-Info | No | Yes |

Question 5: Do companies agree to introduce columns on whether the posSIBs are applied for LTE and/or NR?

| Company | Yes/No | Remark |
| --- | --- | --- |

Question 6: If answer to 5 is yes, Do companies agree to the table shown as above on LTE/NR column?

| Company | Yes/No | Remark |
| --- | --- | --- |

This is placeholder. Companies are invited to add if anything is missing in above discussion.

Question 7: Is anything missing?

| Company | Remark |
| --- | --- |

# Conclusion

The followings are proposed:

# References

[1] R2-2001333 (38.331 running CR)

[2]	R2-2001216 (36.331 CR to introduce PPP-RTK)

[3] R2-2001255 (38.331 CR to introduce on-demand SI request in connected mode)

[4] R2-2001230 (37.355 CR to introduce PPP-RTK)

[5] R2-2001234 (TPs to 37.355 to introduce UE-based DL positioning)

[6] R2-2000006 (37.355 CR to introduce barometric pressure broadcast)

[7] R2-2000188 (36.331 CR to introduce barometric pressure broadcast)

[8] R2-2000396 (36.331 CR to introduce TBS AD broadcast)

[9] R2-2000426 (37.355 CR to introduce TBS AD broadcast)

[10] R2-2000153 (37.355 CR to introduce NavIC)

[11] R2-2000157 (36.331 CR to introduce NavIC)