3GPP TSG-RAN WG2 #109-e	R2-200xxxx

24 February – 6 March 2020

Agenda Item:	6.8.2.2

Source:	Ericsson

Title:	[AT109e][614][POS] Running CR to 38.331 on positioning

Document for:	Discussion, Decision

# Introduction

During online session, companies raised the concern on the terminology “Type2” that has been used in the current running RRC CR. It was agreed to proceed with the offline discussion to find consensus.

[AT109e][614][POS] Running CR to 38.331 on positioning (Ericsson)

Intended outcome: Agreeable CR in R2-2001938

Deadline:  Wednesday 2020-03-04 1300 CET

# Discussion

## 2.1	Terminology

The running RRC CR uses the terminology “Type2” which is unclear since “Type1” which does not exist in the specification. From Rapporteur perspective, the intention is to provide a generic name rather than specific name. In general, it is desired to have generic naming convention to be future proof. Besides, the existing Rel-15 configuration can be also used for positioning purpose. Hence, rather than a specific a generic terminology for Rel-16 may be preferred.

-- TAG-SRS-CONFIG-START

SRS-Config ::=                          SEQUENCE {

srs-ResourceSetToReleaseList            SEQUENCE (SIZE(1..maxNrofSRS-ResourceSets)) OF SRS-ResourceSetId    OPTIONAL,   -- Need N

srs-ResourceSetToAddModList             SEQUENCE (SIZE(1..maxNrofSRS-ResourceSets)) OF SRS-ResourceSet      OPTIONAL,   -- Need N

srs-ResourceToReleaseList               SEQUENCE (SIZE(1..maxNrofSRS-Resources)) OF SRS-ResourceId          OPTIONAL,   -- Need N

srs-ResourceToAddModList                SEQUENCE (SIZE(1..maxNrofSRS-Resources)) OF SRS-Resource            OPTIONAL,   -- Need N

tpc-Accumulation                        ENUMERATED {disabled}                                               OPTIONAL,   -- Need S

...,

[[

srs-Type2-ResourceSetToReleaseList-r16      SEQUENCE (SIZE(1..maxNrofSRS-ResourceSets)) OF SRS-Type2-ResourceSetId-r16		OPTIONAL,   -- Need N

srs-Type2-ResourceSetToAddModList-r16       SEQUENCE (SIZE(1..maxNrofSRS-ResourceSets)) OF SRS-type2-ResourceSet-r16   		OPTIONAL,   -- Need N

srs-Type2-ResourceToReleaseList-r16         SEQUENCE (SIZE(1..maxNrofSRS-Resources)) OF SRS-Type2-ResourceId-r16		      OPTIONAL,   -- Need N

srs-Type2-ResourceToAddModList-r16          SEQUENCE (SIZE(1..maxNrofSRS-Resources)) OF SRS-Type2-Resource-r16	         	OPTIONAL   -- Need N

]]

Editor’s Note: The name of the IE is FFS.

Question 1: Companies are requested to express their view on the terminology to use.

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | SRS-PosResource, and SRS-PosResourceSet |
| Qualcomm | Proposal from Huawei is fine. Alternatives would be SRSforPositioningResource, SRSforPositioningResourceSet. |

# Conclusion

Based on the discussion in section 2 we propose the following:

No table of contents entries found.

# References

- [1] R2-2001333, Running CR for the introduction of NR positioning, Ericsson