3GPP TSG-RAN WG2 #109-e	Draft R2-2001943

Electronic meeting, 24 February – 6 March 2020

Agenda Item:	6.8.1

Source:	Ericsson

Title:	Summary of [AT109e][617][POS] Single positioning method

Document for:	Discussion, Decision

# 1	Introduction

This document is for the following email discussion:

- [AT109e][617][POS] Single positioning method (Ericsson)

Status: Started

Scope: Discuss the possible restructuring of 37.355 to use a single NR positioning method.

Intended outcome: Summary of discussion and potential agreements.  Summary in R2-2001943.

Deadline:  Wednesday 2020-03-04 1300 CET

# 2	Discussion

The discussion on how to represent the RAN1 agreements in RAN2 signalling, based on a single or multiple positioning methods, was summarized prior to the RAN2 #109e-meeting [1]. It concerned comments to the single positioning method LPP CR [2]. Only two companies provided input, each with different preference. However, during the online session, there was a larger number of companies with a preference for a multiple positioning method representation. Therefore, it seems more efficient to use this email discussion to capture what can be adopted from the single positioning method discussion [1] and CR [2] into the running CR [3], already with multiple positioning methods.

We would like to ask companies to express their view about a few aspects from the single positioning methods CR:

- Consider adding a separate positioning method for hybrid NR positioning to ensure matching timings of measurements, based on the single positioning method CR

- Consider adding a reference TRP time of arrival quality metric to the DL-TDOA reference TRP signal measurement IE

- Consider adding a reference TRP UE RxTx measurement and associated quality metric to the DL-TDOA reference TRP signal measurement IE

- Consider adding a DL-PRS RSRP quality metric for positioning methods featuring RSRP measurements

- Anything else to consider from the single positioning method CR

Comments are collected in the following sub-sections:

###### 2.1 Addition of a separate positioning method for hybrid NR positioning

One concern raise related to the configuration of multiple positioning methods in parallel is that measurements might be performed at different times for different positioning methods, when it could have been possible to derive the per positioning method measurements from the same fundamental measurement (such as the DL-PRS cross-correlation function). A possible way to address this is to introduce a separate hybrid positioning method based on the single positioning method set of messages and IEs.

Question 1: Companies are requested to express their view on introducing a separate method/set of messages/set of subsections for hybrid NR positioning.

| Company | Comments |
| --- | --- |

In case a hybrid positioning method is not preferred, there are some important attributes to consider from the single positioning method CR.

###### 2.2 Addition of an attribute for reference TRP TOA quality metric as part of the DL-TDOA method

DL-TDOA is based on RSTD measurements of neighbour TRPs/resource sets/resources in relation to a reference TRP, producing a set of measurement equations. Each measurement equation corresponds to the reference signal time difference (here translated to meters by multiplication of the speed of light) between the signal from a neighbour TRP/resource sets/resources at location p2 .. pM to the UE position pm and the signal from the reference TRP at location p1 to the UE position. Assuming perfectly synchronized base stations and denoting the time of arrival estimation error of the reference signal k by eTOA,k, where E{ e2TOA,k } = 2k., Measurement equations in meters:

y21=p2-pm-p1-pm+eTOA,2-eTOA,1

…

yM1=pM-pm-p1-pm+eTOA,M-eTOA,1

These measurement equations, given the error vector e=[(eTOA,k - eTOA,1)]k=2:M and an assumption of independent TOA errors, have the following error covariance matrix,

R=EeTe=σ22+σ12σ12…σ12σ12σ32+σ12…σ12⋮⋮⋱⋮σ12σ12…σM2+σ12

The error covariance matrix R is crucial in estimating the UE position based on the RSTD measurements. The elements of the diagonal of the R matrix 2k + 21 is the variance of the kth RSTD measurement, and the off-diagonal elements of the R matrix are all the variance 21 of the reference TRP TOA measurement.

Hence, it is crucial to include the reference TRP TOA variance as part of the DL-TDOA signal measurement IE, and the generic timingQualityMetric IE can be used for that purpose.

Question 2: Companies are requested to express their view on providing reference the TRP TOA quality metric as part of the DL-TDOA method measurement report.

| Company | Comments |
| --- | --- |

###### 2.3 Addition of a reference TRP UE RxTx measurement and associated quality metric to DL-TDOA

Reporting DL-PRS TOA only in relative terms to the reference TRP TOA means that the only (M-1) time difference measurements are reported from the original M TOA measurements. This implies a loss of dimensionality. A natural way to regain the full dimensionality is to also include a reference TRP UE RxTx measurement in the DL-TDOA measurement report.

Question 3: Companies are requested to express their view on providing reference the TRP UE RxTx and quality as part of the DL-TDOA method measurement report.

| Company | Comments |
| --- | --- |

###### 2.4 Addition of DL-PRS RSRP measurement quality metric to positioning methods based on RSRP

The timing measurements are associated to a generic timing quality metric and is listed among the RAN1 parameters for Rel. 16 NR positioning. However, the RSRP measurements of the positioning methods DL-AoD and also, if RSRP is added to DL-TDOA and Multi-RTT, are not associated to any quality metric, which should be the case for consistency and completeness.

Question 4: Companies are requested to express their view on defining and providing RSRP quality with RSRP in NR positioning methods DL-AoD, DL-TDOA and Multi-RTT.

| Company | Comments |
| --- | --- |

###### 2.5 Any other components from the single positioning method CR that can be considered

Question 5: Companies are requested to express their view on any other possible components from the single positioning method CR to bring to the baseline CR.

| Company | Comments |
| --- | --- |

# 3		Conclusion

From the provided input, we have the following summary of the comments:

Based on the discussion in section 2, the following is proposed:

Proposal 1

# 4		References

R2-2002115	Summary of [108#86][NR/Pos] Single positioning method approach in LPP, Ericsson

R2-2001278	Single positioning method approach in LPP, Draft CR 37.355, Ericsson

R2-2000474	LPP CR Capturing RAN1 parameters for positioning ([108#85][NR Pos]), Intel Corporation

# Appendix A	Text Proposal for 37.355 based on [108#85]

The following snippets are only for placing the Additional Path reporting for NR into a context. With the separate positioning method representation, there are several parallel places where the additional path reporting will be included in this representation alternative.

#### 6.x.1.4	NR-DL-TDOA Location Information Elements

#### –	NR-DL-TDOA-SignalMeasurementInformation

The IE NR-DL-TDOA-SignalMeasurementInformation is used by the target device to provide NR-DL TDOA measurements to the location server. The measurements are provided as a list of TRPs, where the first TRP in the list is used as reference TRP in case RSTD measurements are reported. The first TRP in the list may or may not be the reference TRP indicated in the NR-DL-PRS-AssistanceData. Furthermore, the target device selects a reference resource per TRP, and compiles the measurements per TRP based on the selected reference resource.

-- ASN1START

# NR-DL-TDOA-SignalMeasurementInformation-r16 ::= SEQUENCE {

# dl-PRS-ReferenceInfo-r16		DL-PRS-IdInfo-r16,

nr-DL-TDOA-MeasList-r16	NR-DL-TDOA-MeasList-r16,

...

}

# NR-DL-TDOA-MeasList-r16 ::= SEQUENCE (SIZE(1.. nrMaxTRPs)) OF NR-DL-TDOA-MeasElement-r16

# NR-DL-TDOA-MeasElement-r16 ::= SEQUENCE {

# trp-ID-r16						TRP-ID-r16			OPTIONAL,

nr-DL-PRS-ResourceId-r16		NR-DL-PRS-ResourceId-r16	 OPTIONAL,

nr-DL-PRS-ResourceSetId-r16		NR-DL-PRS-ResourceSetId-r16 OPTIONAL,

nr-TimeStamp-r16				NR-TimeStamp-r16,

nr-RSTD-r16						INTEGER (0..ffs),	-- FFS on the value range

# nr-RSTD-MeasQuality-r16				NR-TimingMeasQuality-r16,

nr-PRS-RSRP-Result-r16			INTEGER (FFS)			OPTIONAL, -- FFS, value range to be decided in RAN4.

# nr-RSRP-MeasQuality-r16			NR-RSRP-MeasQuality-r16,

nr-UE-RxTxTimeDiff-r16			INTEGER (0..ffs)	OPTIONAL,	-- Cond RefTRP

nr-UE-RxTx-MeasQuality-r16		NR-TimingMeasQuality-r16,

nr-DL-TDOA-AdditionalMeasurements-r16				NR-DL-TDOA-AdditionalMeasurements-r16,

...

}

NR-DL-TDOA-AdditionalMeasurements-r16 ::= SEQUENCE (SIZE (1..3)) OF NR-DL-TDOA-AdditionalMeasurementElement-r16

NR-DL-TDOA-AdditionalMeasurementElement-r16 ::= SEQUENCE {

nr-DL-PRS-ResourceId-r16        NR-DL-PRS-ResourceId-r16	 OPTIONAL,

nr-DL-PRS-ResourceSetId-r16		NR-DL-PRS-ResourceSetId-r16 OPTIONAL,

nr-TimeStamp-r16				NR-TimeStamp-r16,

nr-RSTD-ResultDiff-r16			INTEGER (0..ffs),	-- FFS on the value range

# nr-RSTDdiff-MeasQuality-r16		NR-MeasQuality-r16,

dl-PRS-RSPR-ResultDiff-r16		INTEGER (FFS)		OPTIONAL, -- FFS on the value range

# nr-RSRP-MeasQuality-r16			NR-RSRP-MeasQuality-r16,

...

}

nrMaxTRPs		INTEGER ::= 256		-- Max TRPs per UE

-- ASN1STOP

[…]

#### 6.x.1.5	NR-DL-TDOA Location Information Request

#### –	NR-DL-TDOA-RequestLocationInformation

The IE NR-DL-TDOA-RequestLocationInformation is used by the location server to request NR DL-TDOA location measurements from a target device.

-- ASN1START

# NR-DL-TDOA-RequestLocationInformation-r16 ::= SEQUENCE {

nr-DL-PRS-RstdMeasurementInfoRequest-r16	ENUMERATED { true }				OPTIONAL,		-- Need ON

# nr-RequestedMeasurements-r16		BIT STRING {	prsrsrpReq		(0)

# } (SIZE(1..8)),

nr-AssistanceAvailability-r16		BOOLEAN,

nr-DL-PRS-ReportConfig-r16		NR-DL-PRS-ReportConfig-r16		OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

[…]

#### 6.x.1.6	NR-DL-TDOA Capability Information

#### –	NR-DL-TDOA-ProvideCapabilities

The IE NR-DL-TDOA-ProvideCapabilities is used by the target device to indicate its capability to support NR DL-TDOA and to provide its NR DL-TDOA positioning capabilities to the location server.

-- ASN1START

# NR-DL-TDOA-ProvideCapabilities-r16 ::= SEQUENCE {

nr-DL-TDOA-Mode-r16					PositioningModes,

nr-DL-TDOA-MeasCapability-r16 		NR-DL-PRS-MeasCapability-r16	OPTIONAL,

nr-DL-TDOA-MeasSupported-r16			BIT STRING {	prsrsrpSup		(0)} (SIZE(1..8)),

periodicalReporting-r16				ENUMERATED { supported }				OPTIONAL,

...

}

-- ASN1STOP

[…]

#### 6.y.1.4	NR-DL-AoD Location Information Elements

#### –	NR-DL-AoD-SignalMeasurementInformation

The IE NR-DL-AoD-SignalMeasurementInformation is used by the target device to provide NR DL AoD measurements to the location server. The measurements are provided as a list of TRPs, where the first TRP in the list is used as reference TRP.

-- ASN1START

# NR-DL-AoD-SignalMeasurementInformation-r16 ::= SEQUENCE {

nr-DL-AoD-MeasList-r16			NR-DL-AoD-MeasList-r16,

# ...

# }

# NR-DL-AoD-MeasList-r16 ::= SEQUENCE (SIZE(1..nrMaxTRPs)) OF NR-DL-AoD-MeasElement-r16

# NR-DL-AoD-MeasElement-r16 ::= SEQUENCE {

trp-ID-r16							TRP-ID-r16			OPTIONAL, 	nr-DL-PRS-ResourceId-r16			NR-DL-PRS-ResourceId-r16	 OPTIONAL,

nr-DL-PRS-ResourceSetId-r16			NR-DL-PRS-ResourceSetId-r16 OPTIONAL,

nr-TimeStamp-r16					NR-TimeStamp-r16,

nr-PRS-RSRP-Result-r16				INTEGER (FFS)			OPTIONAL, -- Need RAN4 inputs on value range

# nr-RSRP-MeasQuality-r16			NR-RSRP-MeasQuality-r16,

nr-DL-PRS-RxBeamIndex-r16			INTEGER (1..8),

nr-MeasQuality-r16				NR-MeasQuality-r16,

nr-DL-Aod-AdditionalMeasurements-r16		NR-DL-AoD-AdditionalMeasurements-r16,

...

}

NR-DL-AoD-AdditionalMeasurements-r16 ::= SEQUENCE (SIZE (1..7)) OF NR-DL-AoD-AdditionalMeasurementElement-r16

NR-DL-AoD-MeasurementElement-r16 ::= SEQUENCE {

nr-DL-PRS-ResourceId-r16			NR-DL-PRS-ResourceId-r16	 OPTIONAL,

nr-DL-PRS-ResourceSetId-r16			NR-DL-PRS-ResourceSetId-r16 OPTIONAL,

nr-TimeStamp-r16					NR-TimeStamp-r16,

nr-PRS-RSRP-ResultDiff-r16			INTEGER (FFS)			OPTIONAL, -- Need RAN4 inputs on value range

# nr-RSRP-MeasQuality-r16			NR-RSRP-MeasQuality-r16,

nr-DL-PRS-RxBeamIndex-r16			INTEGER (1..8),

...

}

nrMaxTRPs		INTEGER ::= 256		-- Max TRPs

-- ASN1STOP

[…]

#### 6.y.1.5	NR-DL-AoD Location Information Request

#### –	NR-DL-AoD-RequestLocationInformation

The IE NR-DL-AoD-RequestLocationInformation is used by the location server to request NR DL-AoD location measurements from a target device.

-- ASN1START

# NR-Dl-AoD-RequestLocationInformation-r16 ::= SEQUENCE {

nr-AssistanceAvailability-r16		BOOLEAN,

nr-DL-PRS-ReportConfig-r16		NR-DL-PRS-ReportConfig-r16,

...

}

-- ASN1STOP

[…]

#### 6.y.1.6	NR-DL-AoD Capability Information

#### –	NR-DL-AoD-ProvideCapabilities

The IE NR-DL-AoD-ProvideCapabilities is used by the target device to indicate its capability to support NR DL-AoD and to provide its NR DL-AoD positioning capabilities to the location server.

-- ASN1START

# NR-DL-AoD-ProvideCapabilities-r16 ::= SEQUENCE {

nr-DL-TDOA-Mode-r16		PositioningModes,

# periodicalReporting-r16				ENUMERATED { supported }		OPTIONAL,

nr-DL-PRS-MeasCapability-r16 		NR-DL-PRS-MeasCapability-r16	OPTIONAL,

...

}

-- ASN1STOP

[…]

#### 6.z.1.4	NR-Multi-RTT Location Information Elements

#### –	NR-Multi-RTT-SignalMeasurementInformation

The IE NR-Multi-RTT-SignalMeasurementInformation is used by the target device to provide NR Multi-RTT measurements to the location server. The measurements are provided as a list of TRPs, where the first TRP in the list is used as reference TRP.

-- ASN1START

# NR-Multi-RTT-SignalMeasurementInformation-r16 ::= SEQUENCE {

nr-Multi-RTT-MeasList-r16	NR-Multi-RTT-MeasList-r16,

...

}

# NR-Multi-RTT-MeasList-r16 ::= SEQUENCE (SIZE(1.. nrMaxTRPs)) OF NR-Multi-RTT-MeasElement-r16

# NR-Multi-RTT-MeasElement-r16 ::= SEQUENCE {

# trp-ID-r16							TRP-ID-r16			OPTIONAL,

nr-DL-PRS-ResourceId-r16			NR-DL-PRS-ResourceId-r16	OPTIONAL,

nr-DL-PRS-ResourceSetId-r16			NR-DL-PRS-ResourceSetId-r16 OPTIONAL,

nr-UE-RxTxTimeDiff-r16				INTEGER (0..ffs)	OPTIONAL,	-- FFS on the value range

nr-TimeStamp-r16					NR-TimeStamp-r16,

nr-UE-RxTx-MeasQuality-r16					NR-TimingMeasQuality-r16,

nr-PRS-RSRP-Result-r16				INTEGER (FFS)			OPTIONAL, -- FFS, value range to be decided in RAN4.

# nr-RSRP-MeasQuality-r16			NR-RSRP-MeasQuality-r16,

nr-Multi-RTT-AdditionalMeasurements-r16		NR-Multi-RTT-AdditionalMeasurements-r16,

...

}

NR-Multi-RTT-AdditionalMeasurements-r16 ::= SEQUENCE (SIZE (1..3)) OF NR-Multi-RTT-AdditionalMeasurementElement-r16

NR-Multi-RTT-AdditionalMeasurementElement-r16 ::= SEQUENCE {

nr-DL-PRS-ResourceId-r16			NR-DL-PRS-ResourceId-r16	OPTIONAL,

nr-DL-PRS-ResourceSetId-r16			NR-DL-PRS-ResourceSetId-r16 OPTIONAL,

nr-PRS-RSRP-ResultDiff-r16			INTEGER (FFS)			OPTIONAL, -- FFS, value range to be decided in RAN4.

# nr-RSRP-MeasQuality-r16			NR-RSRP-MeasQuality-r16,

nr-UE-RxTxTimeDiffAdditional-r16				INTEGER (0..ffs)	OPTIONAL,	-- FFS on the value range

nr-TimeStamp-r16					NR-TimeStamp-r16,

...

}

nrMaxTRPs		INTEGER ::= 256		-- Max TRPs

-- ASN1STOP

[…]

#### 6.z.1.5	NR-Multi-RTT Location Information Request

#### –	NR-Multi-RTT-RequestLocationInformation

The IE NR-Multi-RTT-RequestLocationInformation is used by the location server to request NR Multi-RTT location measurements from a target device.

-- ASN1START

# NR-Multi-RTT-RequestLocationInformation-r16 ::= SEQUENCE {

# nr-RequestedMeasurements-r16		BIT STRING {	prsrsrpReq		(0)} (SIZE(1..8)),

nr-AssistanceAvailability-r16		BOOLEAN,

nr-DL-PRS-ReportConfig-r16		NR-DL-PRS-ReportConfig-r16,

...

}

-- ASN1STOP

[…]

#### 6.z.1.6	NR-Multi-RTT Capability Information

#### –	NR-Multi-RTT-ProvideCapabilities

The IE NR-Multi-RTT-ProvideCapabilities is used by the target device to indicate its capability to support NR Multi-RTT and to provide its Multi-RTT positioning capabilities to the location server.

-- ASN1START

# NR-Multi-RTT-ProvideCapabilities-r16 ::= SEQUENCE {

nr-DL-PRS-MeasCapability-r16 		NR-DL-PRS-MeasCapability-r16,

nr-UL-SRS-MeasCapability-r16			NR-UL-SRS-MeasCapability-r16,

nr-Multi-RTT-MeasSupported-r16			BIT STRING {	prsrsrpSup		(0)} (SIZE(1..8)),

periodicalReporting-r16				ENUMERATED { supported }						OPTIONAL,

...

}

-- ASN1STOP

[…]

The TP below would naturally be placed in the common part in section 6.4.3

[…]