3GPP TSG-RAN WG2 Meeting #109 electronic	R2-200xxxx

Home, 24 Feb – 6 Mar 2020

Agenda Item:

Source: 			Session chair (CMCC)

Title: 			SON/MDT agreements from email discussions

Document for:	Agreement

The following proposals are proposed to be agreed through email. Please indicate your opinion in the table.

Proposals trying to be greed:

Proposal 1	Introduce a capability in 38.306 for cross-RAT RLF report delivery.

Proposal 2	The UE shall include the TAC of the source cell (previousPCellId-r16) in the RLF report.

Proposal 3	Re-connection attempt cell is not included in the RLF report.

Proposal 4	UE shall include absoluteFrequencyPointA-r16, locationAndBandwidth, subcarrierSpacing, msg1-FrequencyStart, msg1-FDMInfo and msg1-SubcarrierSpacing in the RLF report when the rlf-Cause is set to beamFailureRecoveryFailure or randomAccessProblem.

Proposal 5	Include the following frequency location related information of the RA resources used by the UE in the RAReport:

a.	msg1-FDM (e.g., in RACH-ConfigGeneric)

b.	msg1-FrequencyStart (e.g., in RACH-ConfigGeneric)

c.	msg1-SubcarrierSpacing  (e.g., in RACH-ConfigCommon)

Proposal 6	Agree on one of the following option is to be adopted for RAReport retaining at the UE:

The oldest RA entry will be replaced by new RA entry if the corresponding RA report is full (i.e., all entries are filled) and has not been fetched. A RA entry within a RA report will be deleted if it is already been stored for 48 hours.

Proposal 7	No need to add a new UE capability indication or not regarding the ability to include the locationInformation in SCGFailureInformationNR and SCGFailureInformation messages.

Proposal 8	For per DRB per UE measurement (e.g. UL delay meas), add the following description inside the table, drbid: The identity of the measured DRB.

Proposal 9	For number of active UE, add the following description inside the table, drbid : the DRBs mapped with the same 5QI for NR SA or mapped with the same QCI for EN-DC.

Proposal 10	For the UL PDCP packet average queuing delay measurement for split bearer in EN-DC, UE reports a single D1 value to the node where it receives the measurement configuration.

Proposal 11	For split bearer with PDCP duplication, reuse the same mechanism as non-duplication case for UL D1 delay measurement.

Proposal 12	0.1ms is applied for UL delay measurement.

Proposal 13	UL F1-U delay is measured using the same matrix as DL F1-U delay defined in TS 28.552.

### Table for opinion:

| Company name | Which proposals are not acceptable |
| --- | --- |
| CMCC | non |
| QUALCOMM | P4,P5,P6,P7 |
| Huawei, HiSilicon | P10, P11 If there is no consensus on P10 and P11, we think that the consequence is that: - UL delay measurement is not supported for split bearer(s) for EN-DC case |
| Ericsson | - P10 is not agreeable - If we cannot agree on P10 or other alternaties of P10, then how to interpret P11 is not clear to us. - For P12- we can clarify that the granularity mentioned in the proposal is for UL PDCP queueing delay measurement i.e., the granularity of UL PDCP queueing delay measurement is 0.1ms. |