3GPP TSG-RAN2 Meeting #109bis-e	R2-2002815

Online, 20 April - 30 April 2020

Agenda Item:	[6.7.6] UE Capabilities - PDCP Duplication

Source: 	Apple Inc

Title:  	Discussion on DRBs Supported with Rel16 PDCP Duplication Enhancement

Document for:	Discussion and Decision

# Introduction

As per 38.306, Section 8, UE constraint is specified to support a minimum of 16 DRBs (or 8 per MAC entity with PDCP Duplication).

In this contribution, we would like to consider a clarification to the UE Capability constraint in light of 4 leg support for PDCP Duplication Enhancement specified in Release 16.

# Discussion

As per TS 38.306 v16.0.0, Section 8 on UE Capability Constraints, NR UE is required to support 16 DRBs or 8 DRBs per MAC entity if PDCP Duplication is configured.

| Parameter | Description | Value |
| --- | --- | --- |
| #DRBs | The number of DRBs that a UE shall support. | 16 per UE. NOTE: 8 per MAC entity with duplication. |

This capability constraint was introduced in Release 15 in RAN2#101 when PDCP Duplication was restricted to 2 legs.

Agreements

1:	The number of DRBs a UE must support in NR is 16 (split and duplicated DRBs count as 1 DRB)

However, in Release 16, support for up to 4 leg PDCP Duplication was introduced.

In light of that, note in the spec mentioned above needs clarification to avoid mismatch of value statement of “16 DRB per UE” and “8 per MAC entity with duplication” as it could be interpreted as minimum DRB capability constraint of 32 per UE.

Observation 1: Minimum #DRB capability constraint could be interpreted as “16 per UE without PDCP Duplication” or “32 per UE with PDCP Duplication”.

Since, each PDCP entity for a DRB can be associated up to 4 lower layer entities when PDCP Duplication is configured, existing NOTE: “8 per MAC entity with duplication” should be updated to “8 per MAC entity with Release15 PDCP Duplication”.  Additional NOTE should be added to indicate that “4 per MAC entity with Release 16 PDCP Duplication”.

Proposal 1: Clarify the note for #DRB UE Capability Constraint to 8 per MAC entity with Release 15 duplication

Proposal 2: Add a note for #DRB UE Capability Constraint to 4 per MAC entity with Release 16 duplication

# Conclusion

Observation 1: Minimum #DRB capability constraint could be interpreted as “16 per UE without PDCP Duplication” or “32 per UE with PDCP Duplication”.

Proposal 1: Clarify the note for #DRB UE Capability Constraint to 8 per MAC entity with Release 15 duplication

Proposal 2: Add a note for #DRB UE Capability Constraint to 4 per MAC entity with Release 16 duplication

# References

1. 3GPP TS 38.306 v16.0.0

2. R2-1804201: RAN2#101 Chairman’s Report