3GPP TSG-RAN WG2 Meeting #109bis-e													R2-2003438

Electronic, 20 April – 30 April 2020

Agenda item:	6.1.3

Source:	CATT

Title:	On Release of BAP Entities

Document for:	Discussion and Decision

1	Introduction

Previously in RAN2 it has been agreed that IAB MT and DU each has its own BAP entity. In the RRC and BAP specifications, some procedures have been captured regarding the release of BAP entity. However there are still open issues that require further discussions on the topic. More specifically the issues include

- Handling of RRC_INACTIVE state for IAB MT

  - Whether it is supported by IAB MT, and required specification changes if decided accordingly

- Detailed behavior of BAP entity/entities release

In this contribution, we look deeper into these issues and provide our views.

2	Discussions

2.1 Handling of RRC_INACTIVE state for IAB MT

The topic has been discussed in RAN2 109e meeting and the email discussions thereafter. However it seems there is no conclusion, and even the alternatives haven’t been clearly listed. We therefore believe more clarification is useful to reach a conclusion.

It is noted that whether RRC_INACTIVE (is supported by Rel-16 IAB MT) is optional or mandatory is a separate discussion which is out of the scope of this contribution.

In Table 1 we list the 3 possible alternatives.

Table 1 Alternatives of RRC_INACTIVE state handling

|  | Whether RRC_INACTIVE is supported by IAB MT | IAT MT behaviour when RRCRelease includes suspendConfig is received | Change to RRC spec |
| --- | --- | --- | --- |
| Alt. 1 | Yes | IAB-MT follows UE’s behaviour - The handling of IAB MT BAP entity upon IAB-MT entering inactive state is up to implementation. | No change seems needed. |
| Alt. 2 | Yes | IAT-MT releases its BAP entity (requires change to RRC spec), and then follow UE’s behaviour. | Add MT BAP entity release in the procedure of RRCRelease including suspendConfig |
| Alt. 3 | No | IAB MT ignores the message (i.e., UE assumes network does not send such message to it). | Clarify UE behaviour accordingly. |

From Table 1 it can be seen that the required change for Alt.1 or Alt.2 if any is marginal, and the functionality does not seem to require much complexity compared with a normal UE.

Furthermore,

- For Alt.3 it is not possible to the network to move IAB MT to INACTIVE, and it might not be future proof, considering the potential use cases for IAB.

- Alt. 1 and 2 largely follows UE procedure, and the only difference is whether to keep the BAP entity.

In this late stage of WI, it is preferable to pick one between Alt. 1 and 2.

Proposal 1 	IAB MT follows the UE behaviour if RRCRelease includes suspendConfig is received. RAN2 discuss and decide whether the IAB MT releases the IAB entity upon going into INACTIVE.

2.2 Detailed behavior of BAP entity release

Existing procedure of BAP entity release

In BAP spec the procedure is now

5.1.2 BAP entity release When upper layers request release of a BAP entity, the node shall: - release the BAP entity.

The question whether this is for MT or DU BAP was raised during the email discussion during RAN2 109e, and the understanding then was that this applies to both, which is where ‘a BAP entity’ came from.

Currently there are two cases in the RRC spec that requires BAP entity release, as listed in Table 2

Table 2 Existing RRC procedures for IAB BAP entity release

|  | Descriptions | Procedure in the TS 38.331 |
| --- | --- | --- |
| Case 1 | if bap-Config in RRCReconfiguration is sent to release | The IAB-MT shall: 1> if the bap-config is set to setup: 2> if no BAP entity is established: 3> establish a BAP entity as specified in [x]; 2> configure the BAP entity to use the bap-Address as this node’s BAP address; 2> if defaultUL-BAProutingID is included: 3> configure the BAP entity to apply the default UL BAP routing ID according to the configuration; 2> if defaultUL-BH-RLC-Channel is included 3> configure the BAP entity to apply the default UL bh-RLC-Channel according to the configuration; 1> if the bap-config is set to release: 2> release the BAP entity as specified in [x]. |
| Case 2 | upon going to RRC_IDLE | 5.3.11 UE actions upon going to RRC_IDLE 1> release all radio resources, including release of the RLC entity, the BAP entity, the MAC configuration and the associated PDCP entity and SDAP for all established RBs; |

Questions and discussions

The procedures in Table 2 seem to focus on IAB MT. However the following questions need discussions.

Q1: What are the cases for higher layer to request release of IAB DU BAP entity?

Q2: Does DU release its BAP entity when MT BAP entity is released?

Q3: When releasing MT and or DU BAP entity, which configurations are released?

In Table 3 we provide analysis to these questions.

Table 3 Discussions on BAP entity release

|  | Analysis | Suggestion |
| --- | --- | --- |
| Q1: What are the cases for higher layer to request IAB DU BAP entity release? | In TS 38.401, the following has been captured 8.9.y IAB-node release An IAB node may depart the network either in an orderly fashion, which implies that both the network and the IAB-node are aware in advance, or in a disorderly fashion (e.g. RLF with failed recovery). 8.9.y.1 IAB-node orderly release For orderly release, the IAB-MT is deregistered. The deregistration procedure is the same as the normal UE deregistration procedure. The IAB-donor-CU hands over the UEs or child IAB-nodes currently connected to the IAB-node’s cell(s) to another cell(s), or releases the UEs and may stop accepting incoming handovers or connections to the IAB-node that is about to be released. The IAB-donor-CU may also update/release the BH RLC channels in the intermediate hops. At this point, the F1 interface will be released and the corresponding SCTP associations will be removed. 8.9.y.2 IAB-node disorderly release For the disorderly release case, how to remove the IAB-node context is up to network implementation. For disorderly release, handling of BAP entity is up to implementation. For orderly release which is under CU’s control, it makes sense to release both MT and DU’s BAP entities. This is discussed further Q2. | For disorderly release it is up to implementation how to handle the BAP entities. |
| Q2: Does DU release its BAP entity when MT BAP entity is released? | For orderly release, there are then separate procedures such as - F1 removal, and - BAP release When the control plane connection to Donor CU is available, these two procedures can be done separately. We therefore do not discuss more on the two procedures’ potential implications to each other. If we focus on BAP operations, Q2 itself results from the previous agreement on having BAP entity for DU and MT. a) Implementation limitation There was also agreement that - The intention of modelling into entities is to clarify the functionality. It is not intended to be an implementation restriction. It is then reasonable not to introduce possible implementation restriction, e.g., when one of MT or DU entity is released, it shall be possible to keep or release the other. b) Use case In case of orderly release of IAB node, it is very likely that Donor CU initiates F1-removal toward IAB DU, and RRC initiates connection release for IAB MT. For the use case, it is reasonable that IAB DU also releases its BAP entity. It should be possible implementation based on a). Alternatively, it seems also possible to mandate that IAB DU shall release its BAP entity if MT does so. But whether this is necessary needs further checking. In the RRC procedure, IAB MT might enter RRC IDLE for either orderly or disorderly release, such mandated linkage between IAB DU and MT release may not be necessary for all the cases. Furthermore, considering IAB is a network node it is preferable not to add too much restriction to its configuration. | IAB DU may release its BAP entity in case of F1-removal. FFS if any stage 2 clarifications is necessary. |
| Q3: When releasing MT and or DU BAP entity, which configurations are released? | From BAP specification the following configurations are relevant. - The IAB-node’s BAP address via RRC. - Mapping from next hop BAP address to downstream egress link via F1AP. - Mapping from next hop BAP address to upstream egress link via RRC. - Mapping from upper layer traffic to BAP routing ID in BAP header via F1AP and RRC. - The BAP routing entries via F1AP. - Mapping to egress BH RLC channels via F1AP and RRC. BH RLC channels are configured via RRC on the IAB-MT function of the IAB-node and via F1AP on the IAB-DU function on the IAB-node and DU function on the IAB-donor. It is likely that more configurations will be introduced for flow control report, etc. This however does not impact the discussions here. Given the discussions in Q1 and Q2, if MT and DU always release their BAP entity together there seems to be no need to further categorize which of these configurations are related to DU BAP entity, MT BAP entity, or even none. | All the BAP configurations are released when MT and DU BAP entities are released. |

Based on the discussions of Q1, the following is proposed.

Proposal 2	For disorderly release, the handling of BAP entity is up to implementation. This follows the stage 2 specification so no change is required.

For the case of orderly release, normally IAB MT is requested by RRC to release the MT BAP entities (i.e., as shown in Table 2, or when RRCRelease includes suspendConfig is received if supported), in those cases, based on discussions of Q2 and Q3 the following are proposed.

Proposal 3	IAB DU may release its BAP entity in case of F1-removal. FFS if any stage 2 clarifications is necessary.

Proposal 4	All the BAP configurations are cleared if the BAP entities of IAB MT and DU are released.

3	Summary

Based on the discussions we have the following proposals on release of BAP entities.

Proposal 1 	IAB MT follows the UE behaviour if RRCRelease includes suspendConfig is received. RAN2 discuss and decide whether the IAB MT releases the IAB entity upon going into INACTIVE.

Proposal 2	For disorderly release, the handling of BAP entity is up to implementation. This follows the stage 2 specification so no change is required.

Proposal 3	IAB DU may release its BAP entity in case of F1-removal. FFS if any stage 2 clarifications is necessary.

Proposal 4	All the BAP configurations are cleared if the BAP entities of IAB MT and DU are released.

4	Reference

[1] TR 38.822, vf01