3GPP TSG-RAN WG2 #109bis-e	R2-20xxxxx

Electronic Meeting, April 20th – 30th 2020

Agenda Item:	6.0.1

Source:	Ericsson

Title:	[AT109bis-e][072][NR RIL] DiscMail11 + DiscMail12 (Ericsson)

Document for:	Discussion, Decision

- Introduction

This document is to kick off the following email discussion:

- [AT109bis-e][072][NR RIL] DiscMail11 + DiscMail12 (Ericsson)

Scope: Discussion and implementation of review issues.

Wanted outcome: a) Agreed RIL Status update in the email discussion report b) Agreed ASN.1/procedure text proposal included in the email discussion report.
After email discussion report is agreed, the TPs will be included in the ASN.1 Review file, for the continued ASN.1 review.

Deadline: Email discussion Stop at EOM, April 30 (short extension 1 week could be considered if needed).

The following issues are addressed in this document:

| ID | Work Item | Section | Rapporteur Comment |
| --- | --- | --- | --- |
| Z302 | DCCA | 5.3.5.3 Reception of an RRCReconfiguration by the UE |  |
| E038 | DCCA, MobEnh | 5.3.10.3 Detection of radio link failure | Discussion ongoing in WI MobEnh, need not be discussed here. |
| E008 | MDT, NR-U | 5.3.10.3 Detection of radio link failure | Discussed in WI MDT/SON session, need not be discussed here. |
| I904 | NR_unlic-Core | 5.3.10.3 Detection of radio link failure |  |
| Q010 | TEI | 5.3.10.3 Detection of radio link failure |  |
| I905 | NR_unlic-Core | 5.3.10.3 Detection of radio link failure |  |
| E055 | CLI, GEN | 5.5.3.2 Layer 3 filtering |  |
| I906 | NR_unlic-Core | 5.7.3.5 Actions related to transmission of SCGFailureInformation message |  |
| Q011 | TEI | 5.7.6.3 Reception of DLDedicatedMessageSegment by the UE |  |
| S402 | PowSave | – PDSCH-Config, maxMIMO-Layers-r16 |  |
| I679 | NR-U | – PhysicalCellGroupConfig, nfi-TotalDAI-Included-r16 etc |  |
| S657 | MIMO | – PTRS-DownlinkConfig | Discussed in MIMO session, agreement to be included in MIMO WI CR, need not be discussd here. |
| I648 | NR-U | – PUCCH-Config, resourceToAddModList |  |
| I649 | MIMO | – PUCCH-Config, spatialRelationInfoToAddModList |  |
| Q007 | 2Step RA/TEI | – RACH-ConfigCommonTwoStepRA, ra-Prioritization-r16 |  |
| Q008 | 2Step RA/TEI | – RACH-ConfigCommonTwoStepRA, ra-PrioritizationForAI-r16 |  |

- ASN.1

Z302

| Z302 | ZTE (LiuJing) | DCCA | 2 | None | There are similar description in section 5.3.5.3 (see below), we suggest to remove one of them. 2> if the RRCReconfiguration message was included in an RCResume message: 3> include the RRCReconfigurationComplete message in the nr-SCG-Response within the scg-Response in the RRCResumeComplete message; 2> if the RRCReconfiguration message was included in E-UTRA RRCConnectionResume message: 3> include the RRCReconfigurationComplete message in the E-UTRA MCG RRC message RRCConnectionResumeComplete in accordance with TS 36.313 [10], clause 5.3.3.4a; | Remove the following sentences from this section. 2> if the RRCResume message includes the mrdc-SecondaryCellGroupConfig with mrdc-SecondaryCellGroup set to eutra-SCG: 3> include in the eutra-SCG-Response the E-UTRA RRCConnectionReconfigurationComplete message in accordance with TS 36.331 [10] clause 5.3.5.3; 2> if the RRCResume message includes the mrdc-SecondaryCellGroupConfig with mrdc-SecondaryCellGroup set to nr-SCG: 3> include in the nr-SCG-Response the SCG RRCReconfigurationComplete message; |
| --- | --- | --- | --- | --- | --- | --- |

5.3.5.3	Reception of an RRCReconfiguration by the UE

<Cut>

1>	set the content of the RRCReconfigurationComplete message as follows:

2>	if the RRCReconfiguration includes the masterCellGroup containing the reportUplinkTxDirectCurrent:

3>	include the uplinkTxDirectCurrentList for each MCG serving cell with UL;

3>	include uplinkDirectCurrentBWP-SUL for each MCG serving cell configured with SUL carrier, if any, within the uplinkTxDirectCurrentList;

2>	if the RRCReconfiguration includes the secondaryCellGroup containing the reportUplinkTxDirectCurrent:

3>	include the uplinkTxDirectCurrentList for each SCG serving cell with UL;

3>	include uplinkDirectCurrentBWP-SUL for each SCG serving cell configured with SUL carrier, if any, within the uplinkTxDirectCurrentList;

2>	if the RRCReconfiguration message was included in an RRCResume message:

3>	include the RRCReconfigurationComplete message in the nr-SCG-Response within the scg-Response in the RRCResumeComplete message;

2>	if the RRCReconfiguration message was included in E-UTRA RRCConnectionResume message:

3>	include the RRCReconfigurationComplete message in the E-UTRA MCG RRC message RRCConnectionResumeComplete in accordance with TS 36.313 [10], clause 5.3.3.4a;

2>	if the RRCReconfiguration is applied due to a conditional configuration execution and included a secondaryCellGroupConfig:

3>	if the applied RRCReconfiguration message was received via SRB1:

4>	if the applied RRCReconfiguration message was received via E-UTRAN:

5>	FFS;

<cut>

Rapporteur:

We noted the text proposed to be deleted is Rel-15 text. So the proposed change in this RIL should be Rejected, and other proposal related to this can be discussed

We ask companies to comment on

- Reject the proposal in the RIL

- Discuss if other change of proceduure text is needed in this context.

Z302: Please provide your comments.

| Company | Yes/No | Reason |
| --- | --- | --- |
| ZTE | see comment | Seems we put this RIL in wrong place and proposed wrong solution. Sorry. It should be in 5.3.13.4 (we raised the issue during email disc[Post109e#37], and the rapporteur suggests to discuss it during ASN.1 review). In section 5.3.13.4, we had the following duplicated descriptions: - The yellow highlight part is duplicated with section 5.3.5.3 in TS38.331. (see above yellow part provided by the rapporteur) - The green highlight part is duplicated with section 5.3.5.3 in TS36.331. So we suggest to remove the descriptions from section 5.3.13.4. 5.3.13.4 Reception of the RRCResume by the UE **skip non-related part** 3> else if the SIB1 contains idleModeMeasurements: 4> include the idleMeasAvailable; 2> if the RRCResume message includes the mrdc-SecondaryCellGroupConfig with mrdc-SecondaryCellGroup set to eutra-SCG: 3> include in the eutra-SCG-Response the E-UTRA RRCConnectionReconfigurationComplete message in accordance with TS 36.331 [10] clause 5.3.5.3; 2> if the RRCResume message includes the mrdc-SecondaryCellGroupConfig with mrdc-SecondaryCellGroup set to nr-SCG: 3> include in the nr-SCG-Response the SCG RRCReconfigurationComplete message; 2> if the UE has logged measurements available for NR and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport: 3> include the logMeasAvailable in the RRCResumeComplete message; <cut> #TS 36.331 (for reference) 5.3.5.3 Reception of an RRCConnectionReconfiguration not including the mobilityControlInfo by the UE **skip non-related part** 2> if the received RRCConnectionReconfiguration message was included in an NR RRCResume message: 3> include the RRCConnectionReconfigurationComplete message in the NR MCG RRC message RRCResumeComplete in accordance with TS 38.331 [82], clause 5.3.13.4; <cut> |
|  | For further discussion | We think the proposed changes are not aligned with R15 but some changes are needed to align with R15. We need more time to think about it. If the green part is removed, NE-DC seems has no where to refer to. In the case of reconfiguation for NR-DC (R15), the setting of the SCG RRCReconfigurationComplete messge in the MCG message is executed when 5.3.5.3 is executed for the MCG. To align with this for resume, the setting of SCG RRCReconfigurationComplete message in the MCG message should be in 5.3.13.4 (it is indeed there) and the following should be removed from 5.3.5.3: 2> if the RRCReconfiguration message was included in an RRCResume message: 3> include the RRCReconfigurationComplete message in the nr-SCG-Response within the scg-Response in the RRCResumeComplete message; In the case of NGEN-DC, for reconfiguration, there is a statement in 5.3.5.3 when it is executed for the SCG: 1> if the UE is configured with E-UTRA nr-SecondaryCellGroupConfig (UE in (NG)EN-DC): 2> if the RRCReconfiguration message was received via E-UTRA SRB1 as specified in TS 36.331 [10]; or 2> if the RRCReconfiguration message was received via SRB3 within DLInformationTransferMRDC 3> submit the RRCReconfigurationComplete via E-UTRA embedded in E-UTRA RRC message RRCConnectionReconfigurationComplete as specified in TS 36.331 [10], clause 5.3.5.3/5.3.5.4; For resume in NGEN-DC, the statement is: 2> if the RRCReconfiguration message was included in E-UTRA RRCConnectionResume message: 3> include the RRCReconfigurationComplete message in the E-UTRA MCG RRC message RRCConnectionResumeComplete in accordance with TS 36.313 [10], clause 5.3.3.4a; In general, we find the R16 wording clearer than the R15 wording but maybe we should align R16 with R15. 2> if the RRCReconfiguration message was included in E-UTRA RRCConnectionResume message: 3> includesubmit the RRCReconfigurationComplete message via E-UTRA embedded in the E-UTRA MCG RRC message RRCConnectionResumeComplete in accordance with TS 36.313 [10], clause 5.3.3.4a; |

E038, I904, Q010, I905

| E038; Ericsson (Tony); DCCA, MobEnh; 2; R2-2003201; In the procedural text in 5.3.10.3 there are different handling on whether dapsConfig is configured or not. However, in case daspConfig is configured and there is a failure on the MCG, the MCG failure procedure should be called. Further, the related part on dasp can be merged into the existing text thus avoid unnecessary and confusing procedural text. We will bring a draft CR addressing this issue.. I904; Intel (Seau Sian); NR_unlic-Core; 2; None; Text is not aligned with the others like: upon random access problem indication from MCG MAC while neither T300, T301, T304, T311 nor T319 are running; Propose to change to: upon consistent uplink LBT failure indication from MCG MAC or upon indication from MCG MAC that consistent uplink LBT failure has occurred. Q010; Qualcomm (Masato); TEI; 2; None; This is about discarding "stored" segments.; Change as follows. 3> discard any segments of segmented RRC messages received stored according to 5.7.6.3. |  |  | 2 | None | This is about discarding "stored" segments. | Change as follows. 3> discard any segments of segmented RRC messages received stored according to 5.7.6.3 |
| --- | --- | --- | --- | --- | --- | --- |

| I905 | Intel (Seau Sian) | NR_unlic-Core | 2 | None | Text is not aligned with the others like: upon random access problem indication from SCG MAC | Propose to change to: upon consistent uplink LBT failure indication from SCG MAC or upon indication from SCG MAC that consistent uplink LBT failure has occurred |
| --- | --- | --- | --- | --- | --- | --- |

Text proposal:

5.3.10.3	Detection of radio link failure

The UE shall:

1>	if dapsConfig is configured for any DRB:

2>	upon T310 expiry in source; or

2>	upon random access problem indication from source MCG MAC; or

2>	upon indication from source MCG RLC that the maximum number of retransmissions has been reached:

3>	consider radio link failure to be detected for the source MCG i.e. source RLF;

4>	suspend all DRBs in the source;

4>	release the source connection.

1>	else:

2>	upon T310 expiry in PCell; or

2>	upon T312 expiry in PCell; or

2>	upon random access problem indication from MCG MAC while neither T300, T301, T304, T311 nor T319 are running; or

2>	upon indication from MCG RLC that the maximum number of retransmissions has been reached; or

2>	upon indication from SCG MAC that consistent uplink LBT failure has occurred, or

2>	if connected as an IAB-node, upon BH RLF indication received on BAP entity from the MCG:

3>	if the indication is from MCG RLC and CA duplication is configured and activated, and for the corresponding logical channel allowedServingCells only includes SCell(s):

4>	initiate the failure information procedure as specified in 5.7.5 to report RLC failure.

3>	else:

4>	consider radio link failure to be detected for the MCG i.e. RLF;

4>	discard any segments of segmented RRC messages received;

4>	store the following radio link failure information in the VarRLF-Report by setting its fields as follows:

<cut>

The UE shall:

1>	upon T310 expiry in PSCell; or

1>	upon T312 expiry in PSCell; or

1>	upon random access problem indication from SCG MAC; or

1>	upon indication from SCG RLC that the maximum number of retransmissions has been reached; or

- upon indication from SCG MAC that consistent uplink LBT failure has occurred, or

1>	if connected as an IAB-node, upon BH RLF failure indication received on BAP entity from the SCG;

1>

2>	if the indication is from SCG RLC and CA duplication is configured and activated; and for the corresponding logical channel allowedServingCells only includes SCell(s):

3>	initiate the failure information procedure as specified in 5.7.5 to report RLC failure.

2>	else if MCG transmission is not suspended:

3>	consider radio link failure to be detected for the SCG, i.e. SCG RLF;

3>	initiate the SCG failure information procedure as specified in 5.7.3 to report SCG radio link failure.

2>	else:

3>	if the UE is in NR-DC:

4>	initiate the connection re-establishment procedure as specified in 5.3.7;

3>	else (the UE is in (NG)EN-DC):

4>	initiate the connection re-establishment procedure as specified in TS 36.331 [10], clause 5.3.7;

E038, Rappporteur: There is open discussion in WI eMOB, proposed to await outcome and not discuss the issue here.

I904: Do companies agree Text Proposal above?

| Company | Yes/No | Reason |
| --- | --- | --- |
| Huawei, HiSilicon | Yes | This looks like an editorial change and we are fine with it. |

Q010: Do companies agree Text Proposal above?

| Company | Yes/No | Reason |
| --- | --- | --- |
| Huawei, HiSilicon | Yes | Stored seems a more accurate wording. |

I905: Do companies agree Text Proposal above?

| Company | Yes/No | Reason |
| --- | --- | --- |
| Huawei, HiSilicon | Yes | This looks like an editorial change and we are fine with it. |

E055

[Description]:

The procedural text captures the following:

… for other measurements, a = 1/2(k/4), where k is the filterCoefficient for the corresponding measurement quantity received by the quantityConfig;…

This description is used for EUTRA and CLI related measurements. The existing text was referring only to EUTRA measurements and it was clear as to which field should be used (quantityConfigEUTRA in quantityConfig). In the current specification, it is confusing as to which ‘corresponding’ field to be used.

Text Proposal:

5.5.3.2	Layer 3 filtering

The UE shall:

1>	for each cell measurement quantity, each beam measurement quantity and for each CLI measurement quantity that the UE performs measurements according to 5.5.3.1:

2>	filter the measured result, before using for evaluation of reporting criteria or for measurement reporting, by the following formula:

Fn = (1 – a)*Fn-1 + a*Mn

where

Mn is the latest received measurement result from the physical layer;

Fn is the updated filtered measurement result, that is used for evaluation of reporting criteria or for measurement reporting;

Fn-1 is the old filtered measurement result, where F0 is set to M1 when the first measurement result from the physical layer is received; and for MeasObjectNR, a = 1/2(ki/4), where ki is the filterCoefficient for the corresponding measurement quantity of the i:th QuantityConfigNR in quantityConfigNR-List, and i is indicated by quantityConfigIndex in MeasObjectNR; for measObjectEUTRA, a = 1/2(k/4), where k is the filterCoefficient for the corresponding measurement quantity received by quantityConfigEUTRA in the quantityConfig; for MeasObjectCLI, a = 1/2(k/4), where k is the filterCoefficient for the corresponding measurement quantity received by quantityConfigCLI in the quantityConfig; for MeasObjectUTRA-FDD, a = 1/2(k/4), where k is the filterCoefficient for the corresponding measurement quantity received by quantityConfigUTRA-FDD in the QuantityConfig;

2>	adapt the filter such that the time characteristics of the filter are preserved at different input rates, observing that the filterCoefficient k assumes a sample rate equal to X ms; The value of X is equivalent to one intra-frequency L1 measurement period as defined in TS 38.133 [14] assuming non-DRX operation, and depends on frequency range.

<cut>

E055: Do companies agree to to Text Proposal above?

| Company | Yes/No | Reason |
| --- | --- | --- |
| ZTE | Yes | The proposed TP looks more clear, we agree. |
| Huawei, Hisilicon | Yes | We think this is to make an accurate description and is fine to have it. |

I906

| I906 | Intel (Seau Sian) | NR_unlic-Core | 2 | None | Need to add the following for NRNR-u DC case as LBT may occur on SCG : 1> else if the UE initiates transmission of the SCGFailureInformation message due to consistent uplink LBT failures: 2> set the failureType as scg-lbtFailure. | Propose to add: 1> else if the UE initiates transmission of the SCGFailureInformation message due to consistent uplink LBT failures: 2> set the failureType as scg-lbtFailure. |
| --- | --- | --- | --- | --- | --- | --- |

5.7.3.5	Actions related to transmission of SCGFailureInformation message

The UE shall set the contents of the SCGFailureInformation message as follows:

1>	if the UE initiates transmission of the SCGFailureInformation message due to T310 expiry:

2>	set the failureType as t310-Expiry;

1>	else if the UE initiates transmission of the SCGFailureInformation message due to T312 expiry:

2>	set the failureType as t312-Expiry;

1>	else if the UE initiates transmission of the SCGFailureInformation message to provide reconfiguration with sync failure information for an SCG:

2>	set the failureType as synchReconfigFailure-SCG;

1>	else if the UE initiates transmission of the SCGFailureInformation message to provide random access problem indication from SCG MAC:

2>	set the failureType as randomAccessProblem;

1>	else if the UE initiates transmission of the SCGFailureInformation message to provide indication from SCG RLC that the maximum number of retransmissions has been reached:

2>	set the failureType as rlc-MaxNumRetx;

1>	else if the UE initiates transmission of the SCGFailureInformation message due to SRB3 IP check failure:

2>	set the failureType as srb3-IntegrityFailure;

1>	else if the UE initiates transmission of the SCGFailureInformation message due to Reconfiguration failure of NR RRC reconfiguration message:

2>	set the failureType as scg-reconfigFailure.

1>	else if the UE initiates transmission of the SCGFailureInformation message due to consistent uplink LBT failures:

2>	set the failureType as scg-lbtFailure.

1> include and set MeasResultSCG-Failure in accordance with 5.7.3.4;

1>	for each MeasObjectNR configured by a MeasConfig associated with the MCG, and for which measurement results are available:

2>	include an entry in measResultFreqList;

<cut>

I906: Do companies agree on Text Proposal above?

| Company | Yes/No | Reason |
| --- | --- | --- |
| Huawei, Hisilicon | No | We think this has already been covered in 5.7.3.3? 1> else if the UE initiates transmission of the SCGFailureInformationNR message due to Reconfiguration failure of NR RRC reconfiguration message: 2> set the failureType as scg-reconfigFailure. 1> else if the UE initiates transmission of the SCGFailureInformationNR message due to consistent uplink LBT failures: 2> set the failureType as scg-lbtFailure. |

Q011

| Q011 | Qualcomm (Masato) | TEI | 2 | None | The text can be a bit more specific what the segment is, e.g. by aligning with the ASN.1 field description. | Change as follows. 1> store the segment of the encoded DL DCCH message included in rrc-MessageSegmentContainer: |
| --- | --- | --- | --- | --- | --- | --- |

Text proposal:

5.7.6.3	Reception of DLDedicatedMessageSegment by the UE

Upon receiving DLDedicatedMessageSegment message, the UE shall:

1>	store the segment of the encoded DL DCCH message included in rrc-MessageSegmentContainer;

1>	if all segments of the message have been received:

2>	assemble the message from the received segments and process the message according to 5.3.5.3 for the RRCReconfiguration message or 5.3.13.4 for the RRCResume message;

2>	discard all segments.

Q011: Do companies agree on Text Proposal above?

| Company | Yes/No | Reason |
| --- | --- | --- |
| Huawei, HiSilicon | Yes | It is clearer to define the specific segments. |

S402

| S402 | Samsung (soenghun Kim) | PowSave | 2 | None | Need code is Need M while IE is INTEGER. Should be SetupRelease to be able to release it. | maxMIMO-Layers-r16 SetupRelease { maxMIMO-Layers } OPTIONAL, -- Need M [Comments]: |
| --- | --- | --- | --- | --- | --- | --- |

Text proposal:

PDSCH-Config information element

-- ASN1START

-- TAG-PDSCH-CONFIG-START

PDSCH-Config ::=                        SEQUENCE {

dataScramblingIdentityPDSCH             INTEGER (0..1023)                                                   OPTIONAL,   -- Need S

<CUT>

sp-ZP-CSI-RS-ResourceSetsToReleaseList  SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-ResourceSets)) OF ZP-CSI-RS-ResourceSetId

OPTIONAL,   -- Need N

p-ZP-CSI-RS-ResourceSet                 SetupRelease { ZP-CSI-RS-ResourceSet }

OPTIONAL,   -- Need M

...,

[[

maxMIMO-Layers-r16                      SetupRelease { MaxMIMO-LayersDL-r16 }                 OPTIONAL,   -- Need M

minimumSchedulingOffsetK0-r16           SetupRelease { MinSchedulingOffsetK0-Values-r16 }                   OPTIONAL,   -- Need M

prb-BundlingTypeForDCI-Format1-2-r16    CHOICE {

staticBundling-r16                      SEQUENCE {

bundleSize-r16                          ENUMERATED { n4, wideband }                                 OPTIONAL    -- Need S

<CUT>

pdsch-TimeDomainAllocationList-v16xy     SetupRelease { PDSCH-TimeDomainResourceAllocationList-v16xy }      OPTIONAL,   -- Need M

repetitionSchemeConfig-r16               SetupRelease { RepetitionSchemeConfig-r16}                         OPTIONAL    -- Need M

]]

}

RateMatchPatternGroup ::=               SEQUENCE (SIZE (1..maxNrofRateMatchPatternsPerGroup)) OF CHOICE {

cellLevel                               RateMatchPatternId,

bwpLevel                                RateMatchPatternId

}

MaxMIMO-LayersDL-r16 ::=                   INTEGER (1..8)

MinSchedulingOffsetK0-Values-r16 ::=    SEQUENCE (SIZE (1..maxNrOfMinSchedulingOffsetValues-r16)) OF INTEGER (0..maxK0-SchedulingOffset-r16)

-- TAG-PDSCH-CONFIG-STOP

-- ASN1STOP

We noted that also max-MIMO-Layers in PDSCH-ServingCellConfig could use this new-defined IE MaxMIMO-LayersDL-r16, to be consistent. But Rapporteur proposes not to do this change.

PDSCH-ServingCellConfig ::=             SEQUENCE {

codeBlockGroupTransmission              SetupRelease { PDSCH-CodeBlockGroupTransmission }       OPTIONAL,   -- Need M

xOverhead                               ENUMERATED { xOh6, xOh12, xOh18 }                       OPTIONAL,   -- Need S

nrofHARQ-ProcessesForPDSCH              ENUMERATED {n2, n4, n6, n10, n12, n16}                  OPTIONAL,   -- Need S

pucch-Cell                              ServCellIndex                                           OPTIONAL,   -- Cond SCellAddOnly

...,

[[

maxMIMO-Layers                          INTEGER (1..8)                                          OPTIONAL,  -- Need M

processingType2Enabled                  BOOLEAN                                                 OPTIONAL   -- Need M

]],

[[

pdsch-CodeBlockGroupTransmissionList-r16 SetupRelease { PDSCH-CodeBlockGroupTransmissionList-r16 }  OPTIONAL   -- Need M

]]

}

S402: Do companies agree Text Proposal above, and to not change PDSCH-ServingCellConfig?

| Company | Yes/No | Reason |
| --- | --- | --- |
| Huawei, HiSilicon | Yes | We are fine to use setuprelease for Rel-16 IEs, but we should not touch Rel-15 signaling and we agree with the rapporteur that the Rel-15 part should not be changed. |

I679

| I679 | Intel (Sudeep) |  | 2 | None | ENUMERATED true Need M cannot be released once configured. | Consider changing to Need R or BOOLEAN. |
| --- | --- | --- | --- | --- | --- | --- |

Text Proposal:

-- ASN1START

-- TAG-PHYSICALCELLGROUPCONFIG-START

PhysicalCellGroupConfig ::=         SEQUENCE {

harq-ACK-SpatialBundlingPUCCH       ENUMERATED {true}                                               OPTIONAL,   -- Need S

<cut>

harq-ACK-SpatialBundlingPUSCH-secondaryPUCCH-group-r16    ENUMERATED {true}                         OPTIONAL,   -- Cond twoPUCCHgroup

pdsch-HARQ-ACK-Codebook-secondaryPUCCH-group-r16          ENUMERATED {semiStatic, dynamic}          OPTIONAL,   -- Cond twoPUCCHgroup

p-NR-FR2-r16                                              P-Max                                     OPTIONAL,   -- Need R

p-UE-FR2-r16                                              P-Max                                     OPTIONAL,   -- Cond MCG-Only

nrdc-PCmode-FR1-r16                ENUMERATED {semi-static-mode1, semi-static-mode2, dynamic}       OPTIONAL,   -- Cond MCG-Only

nrdc-PCmode-FR2-r16                ENUMERATED {semi-static-mode1, semi-static-mode2, dynamic}       OPTIONAL,   -- Cond MCG-Only

pdsch-HARQ-ACK-Codebook-r16            ENUMERATED {enhancedDynamic, spare1}                         OPTIONAL,   -- Need R

nfi-TotalDAI-Included-r16              ENUMERATED {true}                                            OPTIONAL,   -- Need R

ul-TotalDAI-Included-r16               ENUMERATED {true}                                            OPTIONAL,   -- Need R

pdsch-HARQ-ACK-OneShotFeedback-r16     ENUMERATED {true}                                            OPTIONAL,   -- Need R

pdsch-HARQ-ACK-OneShotFeedbackNDI-r16  ENUMERATED {true}                                            OPTIONAL,   -- Need R

pdsch-HARQ-ACK-OneShotFeedbackCBG-r16  ENUMERATED {true}                                            OPTIONAL,   -- Need R

downlinkAssignmentIndexForDCI-Format0-2-r16    ENUMERATED { enabled }                               OPTIONAL,   -- Need S

downlinkAssignmentIndexForDCI-Format1-2-r16    ENUMERATED {n1, n2, n4}                              OPTIONAL,   -- Need S

pdsch-HARQ-ACK-CodebookList-r16        SetupRelease {PDSCH-HARQ-ACK-CodebookList-r16}              OPTIONAL,   -- Need M

ackNackFeedbackMode-r16                ENUMERATED {joint, separate}                                 OPTIONAL    -- Need R

]]

}

<cut>

-- TAG-PHYSICALCELLGROUPCONFIG-STOP

-- ASN1STOP

I679: Do companies agree on Text Proposal above?

| Company | Yes/No | Reason |
| --- | --- | --- |
| Huawei, HiSilicon | Yes | We agree with the principle, maybe better to check from RAN1 point of view whether it is a one shot configuration or not. |

S657

| S657 | Samsung (Seungri Jin) | MIMO | 2 | None | According to RAN1 (R1-2001478), this field is only configured for single-PDCCH based multi-TRP operation. | Add this condition in the field description or make this field as OPTIONAL with Cond sPDCCH-mTRP i.e. need the description for this condition. |
| --- | --- | --- | --- | --- | --- | --- |

Rapporteur: Discussed in MIMO session, agreement to be included in MIMO WI CR:

1.	Agree with the proposed change

a) Change the signalling of maxNrofPorts from ENUMERATED {n2} to ENUMERATED {n1,  n2}

b) add the condition when n2 can be selected in the field description: 2 PT-RS ports can only be configured for single-PDCCH based multi-TRP operation.

Rapporteur proposes not to dicuss S657 here in this email discussion.

I648

resourceToAddModList, resourceToReleaseList Lists for adding and releasing PUCCH resources applicable for the UL BWP and serving cell in which the PUCCH-Config is defined. The resources defined herein are referred to from other parts of the configuration to determine which resource the UE shall use for which report.

The Rapporteur’s understanding is that network configures either resourceToAddModList (without suffix) or resourceToAddModList-16, there is never a mix. Since max number of PUCCH resource is the same, both PUCCH-Resource (without suffix) and PUCCH-Resource-16 have a PUCCH-ResourceId (without suffix). Only one toRelease list seems needed.

Since the term “the network configures the field…” means that the network included the field in this or a previous message, the intended behaviour could be captured by adding the following to the field description:

resourceToAddModList, resourceToReleaseList Lists for adding and releasing PUCCH resources applicable for the UL BWP and serving cell in which the PUCCH-Config is defined. The resources defined herein are referred to from other parts of the configuration to determine which resource the UE shall use for which report. The network configures either resourceToAddModList (without suffix) or resourceToAddModList-r16.

I648: Please provide your comments on the discussion and Text Proposal.

| Company | Yes/No | Reason |
| --- | --- | --- |
| Huawei, HiSilicon | To be further discussed | We think it perhaps worth a discussion on whether we should have some generic principle on this issue. We have one contribution in R2-2003716 in 2.1.2 which is for searchSpacesToAddModList but can be adopted as it is for this field as this is exactly the same situation (as discussed in the paper). |

I649

spatialRelationInfoToAddModList Configuration of the spatial relation between a reference RS and PUCCH. Reference RS can be SSB/CSI-RS/SRS. If the list has more than one element, MAC-CE selects a single element (see TS 38.321 [3], clause 5.18.8 and TS 38.213 [13], clause 9.2.2).

-- ASN1START

-- TAG-PUCCH-CONFIG-START

PUCCH-Config ::=                        SEQUENCE {

resourceSetToAddModList                 SEQUENCE (SIZE (1..maxNrofPUCCH-ResourceSets)) OF PUCCH-ResourceSet   OPTIONAL, -- Need N

resourceSetToReleaseList                SEQUENCE (SIZE (1..maxNrofPUCCH-ResourceSets)) OF PUCCH-ResourceSetId OPTIONAL, -- Need N

resourceToAddModList                    SEQUENCE (SIZE (1..maxNrofPUCCH-Resources)) OF PUCCH-Resource         OPTIONAL, -- Need N

resourceToReleaseList                   SEQUENCE (SIZE (1..maxNrofPUCCH-Resources)) OF PUCCH-ResourceId       OPTIONAL, -- Need N

format1                                 SetupRelease { PUCCH-FormatConfig }                                   OPTIONAL, -- Need M

format2                                 SetupRelease { PUCCH-FormatConfig }                                   OPTIONAL, -- Need M

format3                                 SetupRelease { PUCCH-FormatConfig }                                   OPTIONAL, -- Need M

format4                                 SetupRelease { PUCCH-FormatConfig }                                   OPTIONAL, -- Need M

schedulingRequestResourceToAddModList   SEQUENCE (SIZE (1..maxNrofSR-Resources)) OF SchedulingRequestResourceConfig

OPTIONAL, -- Need N

schedulingRequestResourceToReleaseList  SEQUENCE (SIZE (1..maxNrofSR-Resources)) OF SchedulingRequestResourceId

OPTIONAL, -- Need N

multi-CSI-PUCCH-ResourceList            SEQUENCE (SIZE (1..2)) OF PUCCH-ResourceId                            OPTIONAL, -- Need M

dl-DataToUL-ACK                         SEQUENCE (SIZE (1..8)) OF INTEGER (0..15)                             OPTIONAL, -- Need M

spatialRelationInfoToAddModList         SEQUENCE (SIZE (1..maxNrofSpatialRelationInfos)) OF PUCCH-SpatialRelationInfo

OPTIONAL, -- Need N

spatialRelationInfoToReleaseList        SEQUENCE (SIZE (1..maxNrofSpatialRelationInfos)) OF PUCCH-SpatialRelationInfoId

OPTIONAL, -- Need N

pucch-PowerControl                      PUCCH-PowerControl                                                    OPTIONAL, -- Need M

...,

[[

resourceToAddModList-r16                SEQUENCE (SIZE (1..maxNrofPUCCH-Resources)) OF PUCCH-Resource-r16     OPTIONAL, -- Need N

dl-DataToUL-ACK-r16                     SEQUENCE (SIZE (1..8)) OF INTEGER (-1..15)                            OPTIONAL, -- Need M

dl-DCI-triggered-UL-ChannelAccess-CPext-r16 SEQUENCE (SIZE (1..16)) OF INTEGER (0..15)                        OPTIONAL, -- Need M

subslotLengthForPUCCH-r16               ENUMERATED {n2,n7}                                                    OPTIONAL, -- Need M

dl-DataToUL-ACK-ForDCI-Format1-2-r16    SEQUENCE (SIZE (1..8)) OF INTEGER (0..15)                             OPTIONAL, -- Need M

numberOfBitsForPUCCH-ResourceIndicatorForDCI-Format1-2-r16  INTEGER (0..3)                                    OPTIONAL, -- Need M

dmrs-UplinkTransformPrecodingPUCCH-r16  ENUMERATED {enabled}                                             OPTIONAL,  -- Cond PI2-BPSK

spatialRelationInfoToAddModList-r16     PUCCH-SpatialRelationInfoList-r16                                     OPTIONAL, -- Need N

spatialRelationInfoToReleaseList-r16    PUCCH-SpatialRelationInfoIdList-r16                                   OPTIONAL, -- Need N

resourceGroupToAddModList-r16           SEQUENCE (SIZE (1..maxNrofPUCCH-ResourceGroups-r16)) OF PUCCH-ResourceGroup-r16

OPTIONAL, -- Need N

resourceGroupToReleaseList-r16          SEQUENCE (SIZE (1..maxNrofPUCCH-ResourceGroups-r16)) OF PUCCH-ResourceGroupId-r16

OPTIONAL  -- Need N

]]

}

PUCCH-SpatialRelationInfoList-r16 ::=      SEQUENCE (SIZE (1..maxNrofSpatialRelationInfos-r16)) OF PUCCH-SpatialRelationInfo-r16

PUCCH-SpatialRelationInfoIdList-r16 ::=    SEQUENCE (SIZE (1..maxNrofSpatialRelationInfos-r16)) OF PUCCH-SpatialRelationInfoId-r16

PUCCH-ResourceGroup-r16 ::=                SEQUENCE {

pucch-ResourceGroupId-r16                  PUCCH-ResourceGroupId-r16,

resourcePerGroupList-r16                   SEQUENCE (SIZE (1..maxNrofPUCCH-ResourcesPerGroup-r16)) OF PUCCH-ResourceId

}

PUCCH-ResourceGroupId-r16 ::=              INTEGER (0..maxNrofPUCCH-ResourceGroups-1-r16)

-- TAG-PUCCH-CONFIG-STOP

-- ASN1STOP

maxNrofSpatialRelationInfos             INTEGER ::= 8

maxNrofSpatialRelationInfos-r16         INTEGER ::= 64

PUCCH-SpatialRelationInfoId ::=         INTEGER (1..maxNrofSpatialRelationInfos)

PUCCH-SpatialRelationInfoId-r16 ::=     INTEGER (1..maxNrofSpatialRelationInfos-r16)

The Rapporteur’s understanding is that

- PUCCH-SpatialRelationInfo-r16 is a true critical extension of PUCCH-SpatialRelationInfo.

- Network configures either spatialRelationInfoToAddModList (without suffix) or spatialRelationInfoToAddModList-16, there is never a mix.

- Since max number of PUCCH spatial relations is different, there is both PUCCH-SpatialRelationInfoId (without suffix) and PUCCH-SpatialRelationInfoId -16, as well as both spatialRelationInfoToReleaseList (without suffix).

- The “intermediate IEs” PUCCH-SpatialRelationInfoList-r16 and PUCCH-SpatialRelationInfoIdList-r16 are not needed, and can be deleted (to maintain consistency with the Rel-15 fields).

- E130 above can be closed with no change to the specification text.

- Since the term “the network configures the field…” means that the network included the field in this or a previous message, the intended behaviour could be captured by adding the following to the field description:

spatialRelationInfoToAddModList Configuration of the spatial relation between a reference RS and PUCCH. Reference RS can be SSB/CSI-RS/SRS. If the list has more than one element, MAC-CE selects a single element (see TS 38.321 [3], clause 5.18.8 and TS 38.213 [13], clause 9.2.2). The network configures either spatialRelationInfoToAddModList (without suffix) or spatialRelationInfoToAddModList-r16.

I649: Please provide your comments on the discussion and Text Proposal.

| Company | Yes/No | Reason |
| --- | --- | --- |
| Huawei, HiSilicon | To be further discussed | Similiarly as the above, we think it perhaps worth a discussion on whether we should have some generic principle on this issue. The meaning of the proposed text is not clear because a ToAddModList is a one shot field. We suggest to say: "That network can only configure instances of PUCCH-SpatialRelationInfo-r16 if all instances of PUCCH-SpatialRelation (without suffix) in the UE configuration are released (possibly in the same message), and vice-versa." Besides, it may be good to clarify that "spatialRelationInfoToAddModList (without suffix) can only be used to release spatial relation info created using spatialReleationInfoToAddModList (without suffix). spatialRelationInfoToAddModList-r16 can only be used to release spatial relation info created using spatialReleationInfoToAddModList-r16." |

Q007, Q008

| Q007 | Qualcomm (Masato) | TEI | 2 | None | "If not configured" can mean either RACH-ConfigCommonTwoStepRA is not included or ra-PrioritizationForAI is absent in RACH-ConfigCommonTwoStepRA. The first approach does not allow RACH prioritization to be configured only for 4-step RACH. | We should discuss if such signalling optimization is needed. If deemed necessary, make the following change, and put a suitable need code. If not configured the field is absent in RACH-ConfigCommonTwoStepRA, the UE shall use the values in the corresponding 4-step configuration if configured. |
| --- | --- | --- | --- | --- | --- | --- |
| Q008 | Qualcomm (Masato) | TEI | 2 | None | "If not configured" can mean either RACH-ConfigCommonTwoStepRA is not included or ra-Prioritization is absent in RACH-ConfigCommonTwoStepRA. The first approach does not allow RACH prioritization to be configured only for 4-step RACH. | We should discuss if such signalling optimization is needed. If deemed necessary, make the following change, and put a suitable need code. If not configured the field is absent in RACH-ConfigCommonTwoStepRA, the UE shall use the values in the corresponding 4-step configuration if configured. |

ra-PrioritizationForAccessIdentityTwoStep-r16        SEQUENCE {

ra-Prioritization-r16                                RA-Prioritization                              OPTIONAL, -- Need S

ra-PrioritizationForAI-r16                           BIT STRING (SIZE (2))                          OPTIONAL  -- Need S

}                                                                                                       OPTIONAL, -- Need R

RACH-ConfigCommonTwoStepRA field descriptions

groupB-ConfiguredTwoStepRA Preamble grouping for 2-step random access type. If the field is absent then there is only one preamble group configured and only one msgA PUSCH configuration.

msgA-CB-PreamblesPerSSB-PerSharedRO Number of contention-based preambles used for 2-step RA type from the non-CBRA 4-step type preambles associated with each SSB for RO shared with 4-step type RA. The number of preambles for 2-step RA type shall not exceed the number of preambles per SSB minus the number of contention-based preambles per SSB for 4-step type RA. The possible value range for this parameter needs to be aligned with value range for the configured SSBs per RACH occasion in SSB-perRACH-OccasionAndCB-PreamblesPerSSB in RACH-ConfigCommon. The field is only applicable for the case of shared ROs with 4-step type random access.

msgA-PRACH-RootSequenceIndex PRACH root sequence index. If the field is not configured, the UE applies the value in field prach-RootSequenceIndex in RACH-ConfigCommon in the configured BWP.

msgA-RestrictedSetConfig Configuration of an unrestricted set or one of two types of restricted sets for 2-step random access type preamble. If the field is not configured, the UE applies the value in field restrictedSetConfig in RACH-ConfigCommon in the configured BWP.

msgA-RSRP-Threshold The UE selects 2-step random access type to perform random access based on this threshold (see TS 38.321 [3], clause 5.1.1). This field is only present if both 2-step and 4-step RA type are configured for the BWP.

msgA-RSRP-ThresholdSSB UE may select the SS block and corresponding PRACH resource for path-loss estimation and (re)transmission based on SS blocks that satisfy the threshold (see TS 38.213 [13]).

msgA-RSRP-ThresholdSSB-SUL The UE selects SUL carrier to perform random access based on this threshold (see TS 38.321 [3], clause 5.1.1). The value applies to all the BWPs where 2-step RA is configured.

msgA-RSRP-ThresholdSUL The UE selects 2-step random access type to perform random access if SUL carrier is selected based on this threshold (see TS 38.321 [3], clause 5.1.1). This field is only present if both 2-step and 4-step RA type are configured for the BWP.

msgA-SSB-PerRACH-OccasionAndCB-PreamblesPerSSB The meaning of this field is twofold: the CHOICE conveys the information about the number of SSBs per RACH occasion. Value oneEight corresponds to one SSB associated with 8 RACH occasions, value oneFourth corresponds to one SSB associated with 4 RACH occasions, and so on. The ENUMERATED part indicates the number of Contention Based preambles per SSB. Value n4 corresponds to 4 Contention Based preambles per SSB, value n8 corresponds to 8 Contention Based preambles per SSB, and so on. The total number of CB preambles in a RACH occasion is given by CB-preambles-per-SSB * max(1, SSB-per-rach-occasion). If the field is not configured and both 2-step and 4-step are configured for the BWP, the UE applies the value in the field ssb-perRACH-OccasionAndCB-PreamblesPerSSB in RACH-ConfigCommon.

msgA-SSB-SharedRO-MaskIndex Indicates the subset of 4-step type ROs shared with 2-step random access type for each SSB. This field is configured when there is more than one RO per SSB. If the field is absent, and 4-step and 2-step has shared ROs, then all ROs are shared.

msgA-SubcarrierSpacing Subcarrier spacing of PRACH (see TS 38.211 [16], clause 5.3.2). Only the values 15 or 30 kHz (FR1), and 60 or 120 kHz (FR2) are applicable. The field is only present in case of 2-step only BWP, otherwise the UE applies the SCS as derived from the msgA-PRACH-ConfigurationIndex in RACH-ConfigGenericTwoStepRA in the configured BWP (see tables Table 6.3.3.1-1 and Table 6.3.3.2-2, TS 38.211 [16]). The value also applies to contention free 2-step random access type (RACH-ConfigDedicated).

msgA-TotalNumberOfRA-Preambles Indicates the total number of preambles used for contention-based and contention-free 2-step random access type when ROs for 2-step are not shared with 4-step. If the field is absent, and 2-step and 4-step does not have shared ROs, all 64 preambles are available for 2-step random access type.

ra-PrioritizationForAI Indicates whether the field ra-Prioritization-r16 applies for Access Identities. The first/leftmost bit corresponds to Access Identity 1, the next bit corresponds to Access Identity 2. Value 1 indicates that the field ra-Prioritization-r16 applies, otherwise the field does not apply. If the field is absent, the UE shall use the values in the corresponding 4-step configuration if configured.

ra-ContentionResolutionTimer The initial value for the contention resolution timer for fallback RAR in case no 4-step random access type is configured (see TS 38.321 [3], clause 5.1.5). Value sf8 corresponds to 8 subframes, value sf16 corresponds to 16 subframes, and so on.

ra-Prioritization Parameters which apply for prioritized random access procedure for specific Access Identities. If the field is absent, the UE shall use the values in the corresponding 4-step configuration if configured.

rach-ConfigGenericTwoStepRA 2-step random access type parameters for both regular random access and beam failure recovery.

These RILs are leated to 2Step-RA
The Rapporteur provided text proposal according to RILs (modified need code and modified field descriptions).

Q007, Q008: RAN2 is asked to discuss:

- If this signalling optimization is needed (alternative would be to have fields ra-Prioritization-r16 and ra-PrioritizationForAI-16 mandatory within ra-PrioritizationForAccessIdentityTwoStep-r16).

- If signaling optimization is needed, is text proposal agreeable?

| Company | Yes/No | Reason |
| --- | --- | --- |

- Conclusion

The followings are proposed:

To be agreed:

Further discussion:

- References