3GPP TSG-RAN WG2 Meeting #109bis-e	Draft_R2-2004055

Online, 20th - 30th April 2020

Title:	[Draft] LS on security of PUR for the CP solution

Response to:

Release:	Release 16

Work Item:	NB_IOTenh3-Core, LTE_eMTC5-Core

Source:	Huawei [to be RAN2]

To:	SA3

Cc:

Contact Person:

Name:	Baokun Shan

E-mail Address:	baokun.shan@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN2 has agreed to introduce transmission using PUR (Preconfigured Uplink Resource) in Rel-16 for BL UE, UEs in CE and NB-IoT UEs. Transmission using PUR allows one uplink transmission from RRC_IDLE using a preconfigured uplink resource without performing the random access procedure.

The (ng-)eNB provides the configuration of the radio resources for transmission using PUR in RRCConnectionRelease message. The configuration is valid for one or multiple PUR occasions and can span over a period of time of  one minute to several days.

For the User Plane CIoT EPS/5GS optimisations, the RRCConnectionRelease message is sent integrity protected and ciphered. However, for the Control Plane CIoT EPS/5GS optimisations, the RRCConnectionRelease message is sent without AS security.

PUR (re-)configuration includes UE-specific time and frequency resources to be used for the uplink in RRC_IDLE mode and is specific to one cell of an (ng-)eNB, i.e., if the UE changes cell, then PUR configuration is autonomously released.

RAN2 also agreed “from RAN2 point of view PUR (re-)configuration can be provided to the UE for the CP solution without AS security enabled”. Therefore, RAN2 would like to ask SA3 the following question:

- Is there any security concern about providing the PUR configuration without AS security to UEs using the Control Plane CIoT EPS/5GS optimisations?

2. Actions:

To SA3:

RAN2 respectfully asks SA3 to take above information into consideration and provide feedback on the question accordingly.

3. Date of Next TSG-RAN WG2 Meetings:

3GPP RAN2#110-e		01 - 12 Jun, 2020   	 		Online

3GPP RAN2#111 		24 - 28 Aug, 2020    			Toulouse, France