3GPP TSG-RAN WG2 #110-e	R2-xxx

- Electronic Meeting, 1st – 10th June 2020

Agenda Item:	6.1.5

Source:	Ericsson (rapporteur)

Title:	[AT110e][044][IAB] RRC CR (Ericsson)

Document for:	Discussion, Decision

# Introduction

This document captures the following email discussion:

- [AT110e][044][IAB] RRC CR (Ericsson)

Scope: FIRST Treat R2-2004607 and make agreements on everything as far as possible. Treat all Relevant RIL issues from ASN.1 review. Update RRC CR(s) to implement the solutions. When they are ready, take into account all IAB agreements and update further the RRC CR(s).

Part 1: Agreements relevant to Stage-3 from R2-2004607 and RIL issues.

Part 2: Agreed CR 38331 (Ericsson) (and 36331 if there is impact) for RP. Deadline: June 11, 0700 UTC

# Remaining RIL issues

Since most of the RRC open issues are under the scope of the other email discussions ([045], [046] and [047]), here we cover the few remaining RILs reported during the ASN.1 review.

## RIL: S001 Samsung (JuneHw)

Description: There is missing part on smtc occasion derivation for smtc3 parameters.

Proposed Change: put this. If smtc3list is present, for cells indicated in the pci-List parameter in each SSB-MTC3 element of the list in the same MeasObjectNR, the UE shall setup an additional SS block measurement timing configuration in accordance with the received periodicityAndOffset parameter (using same condition as smtc1 to identify the SFN and the subframe for SMTC occasion) in each SSB-MTC3 configuration and use the duration and ssb-ToMeasure parameters from each SSB-MTC3 configuration.

Rapporteur Comments: Agree with the proposed change and the RIL has already addressed in the DraftCR uploaded last week to [Post109bis-e][920] folder.

Other companies are welcome to provide their comments on the RIL.

| Company | Company comments |
| --- | --- |

## RIL: S002 Samsung (JuneHw)

Description: Currently smtc3 pci list is mandatory, but considering the size of cell list and possibility frequent update of mother MO due to smtc3list with need R, pci list can be further optimized to need M. so that we can save unnecessary bits.

Proposed Change: pci-List-r16            SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId,     OPTIONAL -- Need M

Rapporteur Comments: Agree with the proposed change and the RIL has already addressed in the DraftCR uploaded last week to [Post109bis-e][920] folder.

Other companies are welcome to provide their comments on the RIL.

| Company | Company comments |
| --- | --- |

## RIL: S003 Samsung (JuneHw)

Description: Currently extension of failuretype is the combination of two, using otherType in legacy field and bh-RLF in new field of –v16xy. If the network is aware of that this UE is Rel-16 IAB MT, it can further ignore the legacy field and only concern the newfield. So in legacy field, we don’t need to otherType to indicate the extension.

Proposed Change:

Rapporteur Comments: As mentioned before, this issue has been discussed with the RRC rapporteur and we followed the rapporteur guideline. The failuretype extension is a cross-WI topic, and Ericsson has submitted a 38.331 Draft CR (R2-2005176) about the corrections to failureType handling in SCGFailureInformation. So, we suggest waiting as any changes for IAB WI (on this topic) should be aligned with main room discussion/agreement.

Other companies are welcome to provide their comments on the RIL.

| Company | Company comments |
| --- | --- |

## RIL: S021 Samsung (Milos)

Description: There is no corresponding section on (or description of) smtc3.

Proposed Change: Add the relevant description.

Rapporteur Comments: It seems that this RIL highlights the same issue identified by S001, which is already addressed in the uploaded Draft CR for 38.331.

## RIL: S016 Samsung (Milos)

Description: Value should be updated according to recent RAN3 agreements

Proposed Change: RAN3 agreed to support a maximum of 2^14 different BH RLC channel IDs.

Rapporteur Comments: Agree with the proposed changes, however, RAN3 has a CB on this topic and we suggest waiting for the RAN3 CB conclusion.

Other companies are welcome to provide their comments on the RIL.

| Company | Company comments |
| --- | --- |

## RIL: H691 Huawei (Yulong)

Description: The default BAP configuration should also be used in case of IAB migrating based on the R3 new agreement. Also, it applies to the procedure of RRC resume and RRC re-establishment.

Proposed Change: In the field description of defaultUL-BAP-routingID and defaultUL-BH-RLC-Channel, delete “during IAB- node bootstrapping”.

Rapporteur Comments: Agree with the proposed change and the RIL has already addressed in the Draft CR.

Other companies are welcome to provide their comments on the RIL.

| Company | Company comments |
| --- | --- |

## RIL: H692 Huawei (Yulong)

Description: The new introduced iab-DU-CellIndex is for the index of availableCombToReleaseList. We should use new index for AvailabilityCombinationsPerCell, rather than add new index in IAB-DU-CellID-AI.

Proposed Change: Add availabilityCombinationsIndex-r16 within AvailabilityCombinationsPerCell; Change back to “IAB-DU-CellID-AI-r16 ::=      CellIdentity”  ; Change to “availableCombToReleaseList-r16   SEQUENCE (SIZE(1..maxNrofDUCells-r16)) OF iab-DU-CellIndex-r16AvailabilityCombinationsIndex-r16”.

Rapporteur Comments: We don’t have strong opinion about this RIL, however, other companies are welcome to provide their comments on the RIL.

| Company | Company comments |
| --- | --- |

## RIL: H693 Huawei (Yulong)

Description: There is no text in the current spec to clarify that BH RLC entity should deliver SDU to BAP rather than PDCP.

Proposed Change: In the description of IE BH-RLC-ChannelConfig¸ add “The IE BH-RLC-ChannelConfig is used to configure an RLC entity, a corresponding logical channel in MAC for BH RLC channel between IAB-node and its parent node. The IAB-MT shall deliver RLC SDUs received via the RLC entity of this BH RLC channel to the BAP entity.”

Rapporteur Comments: Agree with the proposed change and the RIL has already addressed in the DraftCR uploaded last week to [Post109bis-e][920] folder.

Other companies are welcome to provide their comments on the RIL.

| Company | Company comments |
| --- | --- |

## RIL: H694 Huawei (Yulong)

Description: As in the Uu configuration for RLC-BearerConfig, the bh-LogicalChannelIdentity is mandatory configured upon creation of a BH RLC channel and cannot be changed after creation. Similar Cond should be added as LCH-SetupOnly.

Proposed Change: Change to “bh-LogicalChannelIdentity-r16    BH-LogicalChannelIdentity-r16, OPTIONAL,   -- Cond LCH-SetupOnly” Add “LCH-SetupOnly   This field is mandatory present upon creation of a BH RLC channel. It is absent, Need M otherwise.”

Rapporteur Comments: We don’t have strong opinion about this RIL, however, other companies are welcome to provide their comments on the RIL.

| Company | Company comments |
| --- | --- |

## RIL: H696 Huawei (Yulong)

Description: No need of introducing TDD-UL-DL-ConfigDedicated-IAB-MT. We can just extend new IEs slotSpecificConfigurationsToAddModList-IAB-MT-r16and slotSpecificConfigurationsToReleaseList-IAB-MT-r16 in TDD-UL-DL-ConfigDedicated. As long as we clarify that IAB-MT reads the slotSpecificConfigurationsToAddModList-IAB-MT-r16 and its field descriptin that value explicit indicates explicitly how many symbols in the beginning and end of this slot are allocated to uplink and downlink. This can save the redundant definition of the whole new TDD-UL-DL-ConfigDedicated-IAB-MT.

Proposed Change: Delete TDD-UL-DL-ConfigDedicated-IAB-MT; Add  “...,  [[ slotSpecificConfigurationsToAddModList-IAB-MT-r16      SEQUENCE (SIZE (1..maxNrofSlots)) OF TDD-UL-DL-SlotConfig       OPTIONAL, -- Need N slotSpecificConfigurationsToReleaseList-IAB-MT-r16      SEQUENCE (SIZE (1..maxNrofSlots)) OF TDD-UL-DL-SlotIndex       OPTIONAL -- Need N ]] ” in TDD-UL-DL-ConfigDedicated; Add field description for slotSpecificConfigurationsToAddModList-IAB-MT as “The slotSpecificConfigurationToAddModList-IAB-MT allows overriding UL/DL allocations provided in tdd-UL-DL-configurationCommon with a limitation that effectively only flexible symbols can be overwritten in Rel-16. The value explicit in TDD-UL-DL-SlotConfig indicates explicitly how many symbols in the beginning and end of this slot are allocated to uplink and downlink”.

Rapporteur Comments: Agree with the proposed change and the RIL has already addressed in the DraftCR uploaded last week to [Post109bis-e][920] folder.

Other companies are welcome to provide their comments on the RIL

| Company | Company comments |
| --- | --- |

## Remaining RACH issue

Description: RACH Configuration for Msg1-based SI request

During the email discussion [109bis][920], there was a consensus that the IAB-MT uses the same RACH configuration for Msg1-based SI request. This configuration is included in the RACH-ConfigGeneric IE. In other words, there is no need to define a specific RACH-ConfigGeneric configuration for MSg1-based SI request for IABs and IABs and UEs will use the same configuration.

RACH-ConfigGeneric IE contains IAB specific parameters to configure the period scaling, frame offset, or S_offset. It was unclear to some companies if the three specific IAB IEs (i.e. prach-ConfigurationPeriodScaling-r16, prach-ConfigurationFrameOffset-r16 and prach-ConfigurationSOffset-r16) do apply or not for the specific case of Msg1-based SI request.

Two different proposals are suggested:

P1: For Msg1-based SI request, IAB-MTs and UEs use the same RACH-ConfigGeneric configuration and IAB-MTs do not apply the IAB specific IEs in the IE RACH-ConfigGeneric (if included they are ignored)

P2: For Msg1-based SI request, IAB-MTs and UEs use the same RACH-ConfigGeneric configuration and IAB-MTs apply the IAB specific IEs in the IE RACH-ConfigGeneric.

Please, provide your preference with justification.

| Company | P1/P2 | Company comments |
| --- | --- | --- |

## Any other RIL/issue

Description:

Proposed Change:

| Company | Company comments |
| --- | --- |

# Summary

- Based on the inputs received from companies regards the open issues, it has been agreed: