3GPP TSG RAN WG2 #110-e	draft R2-20xxxxx

Electronic meeting, 1 – 12 Jun 2020

Source: 	Huawei, HiSilicon

Title: 	[draft] Summary for Offline [010][NR15] Measurements and System Information

Agenda Item:	5.4.1.2

Document for:	Discussion and decision

# Introduction

This document contains a list of TDocs covered in the following offline discussion:

- [AT110e][010][NR15] Measurements and System Information (Huawei)

Scope: Treat R2-2004363 R2-2005419, R2-2005420, R2-2005421, R2-2005422, R2-2005392, (proponents are responsible to explain and drive)

Part 1: Decision whether to make corrections or not, identify agreeable corrections. Deadline: June 4, 0700 UTC.

Part 2: For agreeable parts, continuation to agree CRs. Deadline: June 10, 0700 UTC

Companies are invited to share their views on each TDoc submitted.

# Discussion

## R2-2004363 LS on UE reporting criteria (contact: Nokia)

Q1: Do you think any RAN2 action is needed for this LS? If YES, please clarify.

| Company | Any action needed? (YES/NO) | Comments |
| --- | --- | --- |
| Huawei | NO | Before receiving the RAN4 reply LS, RAN2 has defined the MN-SN coordination on measId, with Stage 3 and Stage 2 CRs agreed. |

## R2-2005419, R2-2005420 36331 CR on inter-RAT SFTD measurements

Q2: Do you agree with the changes in R2-2005419/R2-2005420?

| Company | YES/NO | Comments |
| --- | --- | --- |
| Huawei | YES | Motivation for the change: 1) The current text is inconsistent with RAN4 spec; 2) Although relying on explicit deletion by the NW is also feasible, autonomous removing the measId is a more robust solution from UE’s perspective. Compared with the version (R2-2003734/R2-2003735) submitted to the previous meeting, the clarification was made in a note and only applies when the network does not configure the UE to remove the measId, therefore NBC problems were avoided. |

## R2-2005421, R2-2005422 38331 CR on inter-RAT SFTD measurements

Q2: Do you agree with the changes in R2-2005421/R2-2005422?

| Company | YES/NO | Comments |
| --- | --- | --- |
| Huawei | YES | In the 109bis-e meeting, the 1st and 3rd change were agreed R2-2003701 Correction to inter-RAT SFTD measurements Huawei, HiSilicon CR Rel-15 38.331 15.9.0 1578 - F NR_newRAT-Core R2-2003702 Correction to inter-RAT SFTD measurements Huawei, HiSilicon CR Rel-16 38.331 16.0.0 1579 - A NR_newRAT-Core - [010] 1st and 3rd changes are agreed - [010] revised However, we failed to provide a revised version due to some mistake. R2-2005421/R2-2005422 keeps the 1st and 3rd changes of R2-2003701/R2-2003702 and the 2nd change was removed. |

## R2-2005392 Corrections to SIB1 Processing

Q2: Do you agree with the changes in R2-2005392?

| Company | YES/NO | Comments |
| --- | --- | --- |
| Huawei | YES | The changes look reasonable to us. |

# Conclusion

To be added.