3GPP RAN WG2 Meeting #110-e	R2-2006300
June 1st – 12th, 2020

Agenda Item:	6.19

Source:	InterDigital

Title:	Summary of MPE mitigation in FR2

Document for:	Discussion, Decision

# Introduction

This is the summary report for tdocs submitted for P-MPR reporting to mitigate MPE in FR2:

- [AT110e][030][Other] FR2 MPE (Interdigital) Scope: Treat discussion papers R2-2004341, R2-2004906, R2-2004932, R2-2005126, R2-2005138, R2-2004386, R2-2004650, R2-2004778 (proponents are responsible to explain and drive) Part 1: Identify agreeable changes. If needed after a first round of email discussion, can be revisited on-line. Rapporteur can set additional check-points. Deadline for part 1: June 8th 13:00 UTC Part 2: For agreeable parts, continuation to agree CRs. Deadline: EOM

RAN4 has sent an LS to RAN2 asking to introduce MAC-CE that reports the P-MPR when P-MPR is higher than a configurable threshold [1]:

RAN4 has agreed further details on the Rel-16 FR2 MPE enhancement signalling, which is used to mitigate RLF due to sudden and unpredictable large back off caused UE’s actions to ensure compliance with MPE regulation. In addition to the previous details provided for the Rel-16 FR2 MPE enhancement signalling RAN4 would like to ask RAN2 to take the following additional details into account when developing MAC-CE based signalling for the FR2 MPE enhancements: - Network configured threshold for event-triggered FR2 MPE P-MPR reporting is defined based P-MPR being higher than a configurable threshold. Whether an additionally relative threshold will be defined is still under discussion in RAN4 and RAN4 will inform RAN2 the outcome in the following meeting - P-MPR reporting range and reporting granularity will be defined in the next RAN4 meeting using [2…5] bits. RAN4 will inform RAN2 the exact reporting range and reporting granularity in its next meeting. - P-MPR is reported by the UE after or on the grant and the exact details are up to UE implementation. 2. Actions: To RAN WG2 group. ACTION: RAN4 asks RAN2 group to take the above-mentioned information into account when defining Rel-16 FR2 MPE signalling

Offline discussion “[AT109bis-e][041][NR16 Other] MPE enhancements FR2” was started last meeting in RAN2#109bis-e, but the consensus was to wait for further progress in R4.

# MPE Reporting in L2/3

The work item on NR RF Requirement Enhancements for FR2 in [21]Error: Reference source not found includes an objective to avoid RLFs or Connection Release caused by applying large P-MPR due to compliance to regulatory exposure requirements in FR2. The network needs to be aware of any applied power backoff by the UE in order to schedule the UE with the appropriate number of RBs subsequently.

It can be useful to get a common understanding of the reporting procedure and what is required to be reported. An LS from RAN4 may also arrive this meeting, which should clarify a number of open issues, including the number of bits used for the granularity of the reported P-MPR.

## Triggering of MPE reporting

Per RAN4’s agreement, FR2 MPE P-MPR reporting is event triggered based on P-MPR exceeding a network configured threshold for event-triggered. This is based on the absolute P-MPR value and is thus different from the relative P-MPR reporting specified for PHR, whereby the UE triggers a new PHR if the P-MPR difference relative to the last reported value is larger than a configured threshold. When the UE’s exposure limit exceeds MPE, the UE can apply power backoff over UL transmissions in different serving cells in the cell group, i.e. per MAC entity. The following MPE P-MPR reporting triggers are thus possible:

- Option 1: UE triggers MPE reporting if at least one cell in the MAC entity with a P-MPR ≥ a configurable threshold (per cell). This is proposed by [4, 7, 8, and 18]

- Option 2: UE triggers MPE reporting if cumulative P-MPR applied in all cells (i.e. the sum) in the MAC entity ≥ a configurable threshold.

- Option 3: Reuse PHR trigger on relative PMPR to the last reported value: trigger if the P-MPR change comparing to last report ≥ a configurable threshold. Proposed by [7 and 13].

- Option 4: UE triggers MPE reporting if P-MPR change drops comparing to last report < a configurable threshold. Proposed in [8]. This can be helpful for the network to know that corrective action worked, i.e. UE goes back to normal operation.

Question 1: which option(s) do you prefer for triggering MPE reporting to the network?

| Company | Preferred option(s) | Additional comments |
| --- | --- | --- |
| Ericsson | 3 | If possible to reuse PHR, we think should strive for this. And so far we have not seen any showstopper for this. We think that the relative-trigger is enough and will sufficiently cover the use case suggested by the fixed trigger condition (as per Option 1). |
| Nokia, Nokia Shanghai Bell | 1 or 1+4 | The RAN4 LS [1] specifically stated the following: “Network configured threshold for event-triggered FR2 MPE P-MPR reporting is defined based P-MPR being higher than a configurable threshold. Whether an additionally relative threshold will be defined is still under discussion in RAN4 and RAN4 will inform RAN2 the outcome in the following meeting”. Hence, Option 1 is what RAN4 has requested so at least that needs to be specified. Option 2 does not work: With multi-panel UE, MPE event may only affect some cells, but network needs to still know about the MPE event. The P-MPR is always related to the cell that is used for transmission, not for the other cells. Option 3 is under discussion in RAN4 (as indicated in [1]) and cannot be the only option. Whether this is additionally included can be decided once RAN4 has decided on the inclusion of additional relative threshold. Option 4 is useful in addition to option 1, indicating network when MPE event recovers. Generally, we think network must know both when the MPE event starts and stops to apply necessary actions - This is similar to reportOnLeave in Ax measurement events. |
| Qualcomm | 2 | We think triggering and reporting may have different granularities, i.e. - MPE requirement obviously is defined per UE, even for UEs with multiple panels. Therefore, triggering should be based on cumulative P-MPR summed over all cells in a MAC entity; - When it comes to reporting, P-MPR should be reported per cell, because network needs to know which cells/panels that MPE violation has occurred and how UE applies backoff to different cells, so that it can schedule accordingly. For Option 3, let us wait for RAN4’s agreement. |
| ZTE | 1, or 3 if RAN4 is confirmed | For option 1/2, we share the same view with NOKIA, option 1 is the correct understanding, the transmission is always in a granularity of per cell. Option 3 is still under RAN4 discussion |
| OPPO | 1 | Only option 1 is requested in RAN4 LS. |
| Vivo | 1 | We should at least support Option 1 as requested by RAN4. Regarding other trigger conditions, we can probably wait for the inputs from RAN4. |
| MedaiTek | 1 | Based on RAN4 agreement, option 1 should be supported. For option 3, we can waits for RAN4 input. For other options, we are open to discuss the benefit/necessity. |
| Apple | 1 and 3 | Option 1 is stated in the RAN4 LS, and Option 3 has been agreed in RAN4 agreed WF (R4-2008479). |
| Interdigital | 1 | Per RAN4’s agreement. |
| Intel | 1 and 3 | we also support RAN4 agreement. |

Summary of responses

- 8/10 support option 1 (already agreed by RAN4)

- 4/10 support option 3, if agreed by RAN4.

- 1 company supports option 2

- 1 company supports option 4

Proposal 1: UE triggers MPE reporting if at least one cell in the MAC entity with a P-MPR ≥ a configurable threshold (per cell). (8/10)

Rapporteur comment: this proposal is just to implement RAN4’s agreement. Other options can be revisited based on further input from R4.

## P-MPR reporting for multiple cells

When the UE’s exposure limit exceeds MPE, how the UE applies power backoff over UL transmissions in different serving cells is up to the UE. The UE may also apply power backoff on one or more cells. The following options are thus possible for the design of the MAC CE:

- Option 1: bitmap of cells is included as header of the MAC CE (e.g. as in the PHR MAC CE format). Number of octets is proportional to the number of indicated cells with P-MPR applied. Proposed in [2, 4, 7, 13, and 17]

- Option 2: MAC CE reports P-MPR for a single cell (fixed size MAC CE): a single cell index is included per P-MPR report [8 and 19]

- Option 3: MAC CE reports P-MPR for all serving cells, including cells for which P-MPR = 0 (fixed size MAC CE). This can be for 8 cells or 32 cells (1 or 4 bytes), depending on the configured number of uplink serving cells.

Question 2: which option do you prefer for the format of the MAC CE to indicate the cell?

| Company | Preferred option | Additional comments |
| --- | --- | --- |
| Ericsson | ? | If we reuse the PHR MAC CE, this is already handled by current spec and RAN2 doesn’t need to discuss this. |
| Nokia, Nokia Shanghai Bell | Option 2 | Just to clarify: Option 2 means a MAC CE may be triggered for multiple cells, but each cell will have its own MAC CE. In practice the P-MPR applies for the current UL transmission, and most cells will not utilize UL. Hence, in practice the bitmap will be unnecessary as in most cases UE will always apply the P-MPR for one cell only. |
| Qualcomm | - | To us, the answer to this question depends on whether we agree to introduce a new MAC CE for P-MPR, or use enhanced PHR MAC CE, i.e. - If a new MAC CE is to be introduced, then we can discuss whether to adopt option 1 or option 2; - If enhanced PHR MAC CE is used, then Option 3 is a reasonable design. |
| ZTE | - | Agree with Ericsson and Qualcomm. The key issue is whether the PHR MAC CE can be used or not. |
| OPPO | - | Rely on the new MAC CE for P-MPR reporting or reuse PHR MAC CE. Only when a new MAC CE is introduced for P-MPR reporting, the question is valid. If so, only FR2 serving cells will be included. |
| vivo |  | We have no strong preference on whether to define a new MAC CE or reuse the PHR MAC CE. This depends on how many bits are required by RAN4 for the P-MPR report. |
| MediaTek | - | We share same view with ZTE. |
| Apple | - | Agree with Ericsson. If we reuse PHR MAC CE, we donot need to discuss this question. |
| Interdigital | 2 | Assuming the UE triggers a MAC CE for each cell for which MPE P-MPR is reported. |

Summary of responses

Most companies think this depends on whether a new MAC CE is defined or the PHR MAC CE is reused. If PHR MAC CE is re-used, the issue does not need to be discussed.

Rapporteur comment: RAN4 has not concluded on the number of bits for reported MPE P-MPR, which is important input to decide whether to design a new MAC CE reuse PHR MAC CE. This issue can be postponed until we have further clarity on the MAC CE design. No proposal is made for now. MAC CRs can be looked into next meeting.

## New MAC CE vs. re-use PHR MAC CE

This can be made clear if more input arrives from RAN4 during this discussion. One limitation of the PHR MAC CE is it only has 2 reserved bits, so a new MAC CE can be a better design if RAN4 agrees to report the P-MPR granularity with more than 2 bits. The following options are proposed:

- Option 1: Introduce a new MAC CE to report MPE-related P-MPR [4, 8, 18]. This allows flexibility for more than 2 bits of P-MPR and allows the reporting procedure to be separate from the legacy PHR reporting procedure, including separate configuration of the prohibit timer and threshold.

- Option 2: Reuse PHR framework and PHR MAC CE: replace the 2 reserved bits with P-MPR for MPE [2, 7, 13]. This should be straight forward if RAN4 agrees to a 2-bit granularity for P-MPR. This applies for both single and multiple entry PHR.

- Option 3: Enhance PHR MAC CE to possibly accommodate more than 2 bits of P-MRP granularity [2, 7, 17]. This applies for both single and multiple entry PHR.

[2] further explains the specified behavior that for current PHR MAC CE, i.e. where PcMax is reported if V=0 and is omitted otherwise.

Question 3: which option do you prefer for the MAC CE type used to report MPE-related P-MPR?

| Company | Preferred option | Additional comments |
| --- | --- | --- |
| Ericsson | 2 or 3 Prefer 2. | We could actually use option 2 also if RAN4 requires three bits since if MPE is configured the P-bit can be used as well. |
| Nokia, Nokia Shanghai Bell | Option 1 | First, we would like to note that RAN4 does NOT determine the signalling format: That’s the task of RAN2. However, RAN4 is responsible for indicating to RAN2 what are the conditions under which the MAC CE report is triggered – and on those, there have been communications. So far RAN4 has indicated 2-5 bits (As per agreed RAN4 WF in R4-200573: “Compromise is encouraged between Option 1 (5 bits) and Option 2 (2 bits), i.e. 2 to 5 bits”), so RAN2 has to prepared that >2 bits will be used, as was already requested in the RAN4 LS [1]. Therefore, option 2 cannot work due to the RAN4 LS. Second, the problem with reusing PHR MAC CE is that we end up discussing how the PHR triggering could impact FR2 MPE triggering: The purpose and periodicity of PHR may be completely different from FR2 MPE: MPE is, by nature, an event-triggerered condition. It’s also not reasonable to limit MPE triggering e.g. due to PHR prohibit timer running (see also Q4). Third, another problem with reusing PHR MAC CE is that mixing two procedures into one MAC CE may create both complex procedural text as well as hard-to-detect error cases. |
| Qualcomm | 2 or 3 (depend on RAN4) | In our view, after P-MPR is triggered, PHR should be reported to network too, to better assist power control. If this is agreeable, then it makes sense that we combine P-MPR and PHR in a single MAC CE. The exact format of this enhanced PHR MAC CE will depend on the granularity of P-MPR decided by RAN4. |
| ZTE | Option 3/2 | We have same sympathy with Qualcomm. The p,cmax shall be reported to NW along with the P-MPR value for NW to control the power. |
| OPPO | - | Before we decide this, we should confirm some questions with RAN2 as described in [2][3]. We propose to send LS to RAN4 first, then we decide which MAC CE is possible. |
| vivo | Option 1 | Using a new MAC CE would be more future-proof. We also consider that only reporting the P-MPR without reporting the PH value may not be very helpful. However we can probably require the UE to report the PHR MAC CE along with the P-MPR MAC CE. |
| MediaTek | - | No strong view. However, we think both P-MPR and PHR are related to power control, so we think it is reasonable to go for option 2 or option 3 unless we find great complexity in combining the two procedures. |
| Apple | 2 or 3 (prefer 2) | We share Qualcomm’s view. As RAN4 agreed, at least to solve RLF PHR information is needed in addition to PMPR for FR2. |
| Interdigital | 1 | Given the triggering condition is different, a new MAC CE is preferred in order to not tamper with the PHR procedure. Further, a new MAC CE can accommodate more than 2 bits for P-MPR, thus giving flexibility for whichever reporting granularity is decided by R4. |
| Intel | Option 3 | PHR/P_cmax,c information along with P-MPR would be useful in more correct uplink scheduling. |

Summary of responses

- 5/10 companies prefer to enhance the PHR MAC CE to include the MPE P-MPR

- 3/10 companies prefer to use a new MAC CE for reporting MPE P-MPR

- 2/10 companies have not expressed a strong preference.

Rapporteur comment: RAN4 has not concluded on the number of bits for reported MPE P-MPR, which is important input to decide whether to design a new MAC CE reuse PHR MAC CE. No sufficient consensus is achieved in this R2 discussion either. This issue can be postponed until we have further clarity on the MAC CE design. No proposal is made for now. MAC CRs can be looked into next meeting.

For reusing the PHR MAC CE, one potential issue discussed in [2] is whether new MPE-related P-MPR triggers apply to legacy PHR MAC CEs (e.g. for P-MPR reporting caused by SAR for cells in FR1).

Applicable only for options 2 or 3 in Q3:
Question 3(a): For new triggers agreed (per Q1, e.g. triggering based on absolute P-MPR), do you agree that such trigger should not apply to legacy PHR (e.g. for SAR P-MPR reporting in FR1)?

| Company | Reply (y/n) | Additional comments |
| --- | --- | --- |
| Ericsson | ? | We think that the relative trigger is enough. The absolute trigger is sufficiently covered by the relative trigger. |
| Nokia, Nokia Shanghai Bell | Yes | What RAN4 requested was only for FR2 MPE event determination. We shouldn’t start specifying MPE for FR1 as FR1 and FR2 regulatory requirements are different. |
| Qualcomm | Yes | No need to apply absolute-threshold based trigger to PHR. |
| ZTE | Yes | I guess there is no reason for us to change anything for PHR reporting |
| OPPO | ? | If PHR MAC is reused for P-MPR reporting, we think the answer for this question is yes. |
| vivo |  | No strong preference. We’d rather consider that the PH value should be reported along with the P-MPR. |
| MediaTek | Yes | No need to change legacy PHR trigger. |
| Apple | Yes | No need to change legacy PHR trigger. |
| Interdigital | Yes | if the PHR MAC CE is reused to report MPE P-MPR, a PHR triggered by absolute P-MPR exceeding the threshold should only be applicable for MPE related P-MPR (rather than legacy P-MPR cause by SAR for example). |
| Intel | Yes | It is for simplicity. |

Summary of responses

- 8/10 companies agree that if the PHR MAC CE is reused to report MPE P-MPR, legacy PHR reporting and triggering is not affected by the absolute P-MPR triggering, and the absolute P-MPR trigger should only be applicable for FR2 MPE related P-MPR (rather than legacy P-MPR cause by SAR for example).

Rapporteur comment: This issue can be handled by proper configuration of MPE reporting functionality (see proposal 2) in the appropriate cell groups. Details can be looked into if the PHR MAC CE is reused. No proposal is made for now.

[2, 17] also discuss whether the enhanced PHR MAC CE can be triggered by legacy PHR triggers, in addition to the MPE-related P-MPR trigger, and whether legacy PHR MAC CE can also include the P-MPR due to legacy triggers of PHR reporting. In other words, can the UE report MPE-related P-MPR in a PHR MAC CE triggered by legacy triggers (e.g. pathloss change, activation of Scell, periodic reporting etc), even if the P-MPR is less than the configured threshold.

Applicable only for options 2 or 3 in Q3:
Question 3(b): Do you agree that PHR MAC CE with enhanced structure (which includes MPE P-MPR) can only be triggered by P-MPR events agreed by R4?

| Company | Reply (y/n) | Additional comments |
| --- | --- | --- |
| Ericsson | N | It seems the proposal are focusing on some optimization? We think the gNB does not necessarily know which trigger caused the UE to transmit the PHR MAC CE (the periodic trigger could perhaps be guessed, but that is a special case). And that is of course fine. So, if we go the PHR MAC CE-approach we will only need to add a new MPE-field and the old triggers take care of the rest. If the UE is configured to report MPE-indications the field has to contain something and the UE cannot omit the field (the bits will be there). Perhaps we have misunderstood the proposal, but it seems to hint towards some optimization. |
| Nokia, Nokia Shanghai Bell | Y | If we go this way, then yes, it should only apply for the FR2 MPE events. |
| Qualcomm | N | We share similar view with Ericsson, i.e. enhanced PHR MAC CE can be triggered by either P-MPR or legacy PH conditions. But only when it is triggered by P-MPR, the enhanced PHR MAC CE needs to include P-MPR information. Network can tell what is included and what is not by the MPE indication fields in the MAC CE. |
| Zte | Yes | It is beneficial for NW to realize which event is triggered for this MAC CE (i.e for PHR or P-MPR). |
| OPPO | N? |  |
| vivo | N | It would be simpler to configure the UE to report a certain PHR format. |
| MediaTek | N | We share same view with Qualcomm. NW can realize which event triggers this PHR MAC CE transmission by checking whether the field for P-MPR is present or not. So, the restriction proposed in Q3 is not needed. |
| Apple | N | We shar Ericsson’s view. |
| Interdigital | Y | MPE P-MPR should only be reported if the triggers agreed by R4 occur (i.e. the FR2 triggers). |
| Intel | Y | The discussion is a bit confusing but we should avoid using different format but P-MPR reporting can be included only when it is triggered by new MPE triggers. |

Summary of responses

- 6/10 companies think that if the PHR MAC CE is reused to report MPE P-MPR, then the MPE P-MPR can be reported by any PHR MAC CE regardless of the triggering event type.

- 4 companies expressed that P-MPR reporting should only apply for FR2 MPE triggers, and otherwise the network may not know the event that caused triggering of the PHR MAC CE.

Rapporteur comment: Details can be looked into if the PHR MAC CE is reused. This discussion can be postponed for now.

Configuration of MPE P-MPR reporting function:

If the PHR MAC CE is reused, the network needs to support it. [2, 7] point out that a R-15 gNB may not decode an enhanced PHR MAC CE for example. [2, 7, 13] thus propose to add an explicit RRC configuration, whereby the UE reports MPE related P-MPR only when such parameter is configured. Such RRC parameter can be per UE configuration, i.e. only from Rel-16 compatible cells.

Applicable only for options 2 or 3 in Q3:
Question 3(c): Do you agree to limit MPE P-MPR reporting to only when configured to report it?

| Company | Reply (y/n) | Additional comments |
| --- | --- | --- |
| Ericsson | Y | UE shall of course not signal anything to the gNB which the UE hasn’t been configured to signal. That applies also here. |
| Nokia, Nokia Shanghai Bell | Y | Especially if we reuse PHR and the format changes, legacy network will not be able to comprehend the new format. So as with all new features, the usage is configured by network. |
| Qualcomm | Y |  |
| ZTE | Y |  |
| OPPO | Y |  |
| vivo | Y |  |
| MediaTek | Y |  |
| Apple | Y |  |
| Interdigital | Y |  |
| Intel | Y |  |

Summary of responses

All companies agree to add an explicit RRC configuration, whereby the UE reports MPE related P-MPR only when such parameter is configured

Proposal 2: Support a per MAC entity RRC configuration, whereby the MAC entity reports MPE related P-MPR only when such parameter or IE is configured (10/10)

## RRC Parameters

RAN4 has indicated in the LS the need for a configured threshold for the purpose of absolute triggering and a prohibit timer. RAN4 further tentatively agreed that periodic PMPR reporting is not introduced [22].

Reporting prohibit timer:

To configure the prohibit timer, the following options are proposed:

- Option 1: Reuse the value configured for the PHR prohibit timer [7, 13]

- Option 2: A separate timer is configured for MPE reporting procedure per MAC entity [8, 18]

Question 4: which option do you prefer for the configuration of the prohibit timer for MPE reporting?

| Company | Preferred option | Additional comments |
| --- | --- | --- |
| Ericsson | 1 | If we go with the PHR MAC CE approach, everything is already in place and we don’t need to disucss this further. |
| Nokia, Nokia Shanghai Bell | Option 2 | We cannot reuse existing PHR prohibit timer for several reasons: - RAN4 specifically requested to define prohibit timer for MPE based on the absolute threshold triggering as per the RAN4 LS R4-2002916: “A prohibit timer is enabled to be configured by network to trigger the P-MPR reporting”. - PHR having been sent should never prohibit sending FR2 MPE information (MPE event is not connected to PHR triggering, and RAN4 request for prohibit timer was specifically for the MPE event) - The values for PHR and MPE prohibition timers could be very different – using the same timer would require to always use the smallest value, which will just increase MAC CE signalling (it’s quite likely MPE prohibit timer is longer than PHR one, so using only PHR value would mean UE sends the MPE information more frequently since network has to prioritize reception of MPE information to avoid even asking UE to use the FR2 cell using the high P-MPR) However, for value range we are open to consider similar value range as for PHR prohibit timer. |
| Qualcomm | Option2 | We think it is necessary to have a separate prohibit timer for P-MPR, as they may have different latency requirement for reporting. And we do not want the scenario where a previously reported PHR blocks reporting of newly triggered P-MPR. |
| ZTE | Option 2/1 | It relies on the outcome of question 3(b), if enhanced PHR MAC CE is only used for P-MPR, the prohibit timer for P-MPR reporting can be separated from PHR reporting. Otherwise, they should have one prohibit Timer. |
| OPPO | ? | It relies on which MAC CE is designed to report P-MPR. |
| Vivo | Option 2 | If we use a new MAC CE for reporting the P-MPR, probably we need to define a new timer. |
| MediaTek | Option 2 | P-MPR and legacy PHR triggers are for different purposes, and thus it makes sense to apply separate prohibit timer. Otherwise, P-MPR info may be blocked by previous PHR report for legacy PHR triggers. |
| Apple | Option 1 | Reusing whole PHR framework is simple and sufficient. |
| Interdigital | Option 2 | Agree with Nokia and Qualcomm. The reporting requirements are different. |
| Intel | Option1 | If we go with enhancing PHR MAC CE, one prohibit timer should be sufficient. |

Summary of responses

- 5/10 support option 2, given legacy PHR and MPE triggers are for different purposes and have different reporting requirements. A reported PHR should not prohibit sending FR2 MPE.

- 3/10 support option 1, given it’s already there for PHR.

- 2 companies think we should wait till we decide on whether and how to re-use PHR MAC CE.

Proposal 3: For P-MPR reporting prohibit timer, a separate value is configured for MPE reporting procedure per MAC entity (5/10).

Rapporteur comment: This proposal can be viewed independently of the decision of whether a new MAC CE or PHR MAC CE is used to report MPE P-MPR. if we decide to reuse PHR MAC CE, this RRC value applies only for PHR MAC CEs triggered by MPE, and a legacy PHR already reported should not prohibit a new MPE reporting MAC CE from being reported.

[8] further proposes to stop the prohibit timer if P-MPR drops below a threshold for a previously reported P-MPR. This is relevant if the trigger in Option 4 for question 1 is agreed.

Question 5: Do you support stopping the prohibit timer if P-MPR drops below a threshold for a previously reported P-MPR?

| Company | Reply (y/n) | Additional comments |
| --- | --- | --- |
| Ericsson | N | We think the point of the prohibit timer is to ensure that the UE does not retransmit the MAC CE until the timer has expired (by itself). |
| Nokia, Nokia Shanghai Bell | Y | Network needs to know both when the MPE even starts and when it stops. For example, network might retain the FR2 cell but deactivate it, and then activate it again once the MPE event has stopped. Alternatively, it could just stop scheduling the cell while waiting to see if the MPE event continues. Without the information on when the P-MPR drops to acceptable levels, network will have few means to know when the FR2 cell has recovered. |
| Qualcomm | Y | We share the same view as Nokia |
| ZTE | N | The prohibit Timer is used for not overtriggering the P-MPR MAC CE, it seems no reason to stop it which may revert its original intention. |
| OPPO | N | We share same view with Ericsson. |
| vivo | N |  |
| MediaTek | N |  |
| Apple | N |  |
| Interdigital | N |  |
| Intel | N |  |

Summary of responses

8/9 do not support stopping the prohibit timer if P-MPR drops below a threshold for a previously reported P-MPR. No proposal is made.

P-MPR threshold for absolute triggering:

To configure the threshold for absolute P-MPR triggering, the following options are proposed:

- Option 1: A separate value is configured for MPE reporting procedure per MAC entity [8, 18]

- Option 2: Reuse value configured for PHR in phr-Tx-PowerFactorChange dB [7, 13]

Question 6: which option do you prefer for the configuration of the P-MPR threshold for absolute triggering?

| Company | Preferred option | Additional comments |
| --- | --- | --- |
| Ericsson | Neither | We think the absolute trigger-condition is sufficiently covered by the relative trigger, and hence is not needed. |
| Nokia, Nokia Shanghai Bell | Option 1 | Option 2 doesn’t work: the PHR value is meant for relative threshold, and is meant for sending PHR. The values are likely very much lower than those used fror MPE reporting. There’s no real reason to tie the PHR triggering with MPE event triggering – their uses cases are completely different. Further, option 2 is against the RAN4 LS request, asking for absolute P-MPR value as per : “Network configured threshold for event-triggered FR2 MPE P-MPR reporting is defined based P-MPR being higher than a configurable threshold.” RAN4 is only discussing additions on top of this, but no changes to this. |
| Qualcomm | Option 1 | We share the same view as Nokia |
| ZTE | Option 1 |  |
| OPPO | Option 1 |  |
| vivo | Option 1 |  |
| MediaTek | Option 1 |  |
| Apple | Option 1 |  |
| Interdigital | Option 1 |  |
| Intel | Option1 |  |

Summary of responses

9/10 support option 1.

Proposal 4: For P-MPR threshold for absolute triggering, a separate value is configured for MPE reporting procedure per MAC entity (9/10).

## Other Details

Need to trigger SR

An open issue in discussed in offline 041 in R2#109-bis-e is whether there is a need to signal a new SR when MPE is triggered and UE has no grant to multiplex the MAC CE. R4 agreed that P-MPR is reported by the UE after or on the grant and the exact details are up to UE implementation. The new MAC CE can be dynamic in nature during dynamic scheduling, i.e. similar to the PHR MAC CE. [8] however points out that this may cause reporting delays for the network. [8] proposes to trigger SR if the P-MPR value is larger than a configured threshold for this SR triggering, as a mean to provide an early warning to the network.

Question 7: Do you agree to trigger SR if the P-MPR value > a configured threshold?

| Company | Reply (y/n) | Additional comments |
| --- | --- | --- |
| Ericsson | N | If it was not needed for normal PHR MAC CE reporting, it is not needed here either. |
| Nokia, Nokia Shanghai Bell | Y | It was requested by RAN4 that the MPE report is sent as fast as possible: Normally, a MAC CE doesn’t trigger BSR, so no SR is triggered, either.This can cause delays to send the MPE report. For example, PHR report never triggers SR. We would also note that this is exactly the same behaviour as was already used for BFR and LBT MAC CEs in Rel-16 already based on similar reasoning (i.e. sending the information as fast as possible). |
| Qualcomm | N | RAN4 already agreed that no urgent signaling is required for P-MPR. So we can following the same PHR procedure for P-MPR. So there is no need to trigger SR to request UL grant for P-PMR reporting. |
| ZTE | N |  |
| OPPO | N | Cannot see the necessary if no UL grant. |
| vivo | N |  |
| MediaTek | N |  |
| Apple | N |  |
| Interdigital | N | R4 agreed that P-MPR is reported by the UE after or on the grant and the exact details are up to UE implementation. The reported MPE should be dynamic in nature, i.e. during dynamic scheduling. |
| Intel | N |  |

Summary of responses

9/10 do not support triggering a SR if the P-MPR value > a configured threshold. No proposal is made.

eLCID allocation

If a new MAC CE is introduced, [4] proposes to use a one-octet eLCID for the to save the signalling overhead.

Question 8: If a new MAC CE is introduced, do you agree to use one-octet eLCID?

| Company | Reply (y/n) | Additional comments whether eLCID set1 (below 64) or set2 (above 64) is used |
| --- | --- | --- |
| Ericsson | Y | We assume we are invited to answer this question even if we propose to go the PHR MAC CE approach: This can go in the new range of LCIDs so that we save the legacy LCIDs for overhead critical MAC CEs. Of course, if we go with the PHR MAC CE approach, this question is not applicable. |
| Nokia, Nokia Shanghai Bell | Y | We are fine to use eLCID but have no strong view – also LCID would work. |
| Qualcomm | N | We assume that either we agreed to introduce a new P-MPR MAC CE or enhance the legacy PHR MAC CE, that MAC CE will need to have a new LCID and it will have variable length. Then we should stick to the principle that a new MAC CE with variable length should get its LCID from the new range. |
| ZTE | Y | If new MAC CE is introduced with new LCID is introduced, we agree with that eLCID is applied to this MAC CE. |
| OPPO | N |  |
| vivo | Y |  |
| MediaTek | Y | Assume the P-MPR trigger events does not happen frequently, it make sense to use eLCID. |
| Apple | N |  |
| Interdigital | Y |  |
| Intel | Y |  |

Summary of responses

- 7/10 companies agree to use one-octet eLCID.

- 2/10 companies disagree.

- 1 company answer “N”, though meant they prefer to put this new MAC CE in the new eLCID space

Rapporteur comment: This issue can be postponed until we have further clarity on whether a new MAC CE is introduced. No proposal is made for now.

LCP multiplexing priority

If a new MAC CE is introduced, [4] proposes to use the same LCP multiplexing priority as the PHR MAC CE.

Question 9: If a new MAC CE is introduced, do you agree to use the same LCP multiplexing priority for this MAC CE? If not clarify what priority should be used (if you support a new MAC CE).

| Company | Reply (y/n) | Additional comments |
| --- | --- | --- |
| Ericsson | Y | We assume we are invited to answer this question even if we propose to go the PHR MAC CE approach. |
| Nokia, Nokia Shanghai Bell | N | We think the MPE report should have higher priority than PHR since it’s triggered due to regulatory MPE requirements. |
| Qualcomm | Y |  |
| Zte | Y |  |
| OPPO | Y |  |
| vivo | Y |  |
| MediaTek | Y |  |
| Apple | Y |  |
| Interdigital | Y |  |
| Intel | Y |  |

Summary of responses

- 9/10 companies agree to use the same LCP multiplexing priority for this MAC CE

Rapporteur comment: This issue can be postponed until we have further clarity on whether a new MAC CE is introduced. No proposal is made for now.

UE capability:

[2, 4, 8, 13] propose that MPE-related P-MPR reporting should be a per-UE optional capability. [4, 13] think this can also be frequency agnostic, though the MPE P-MPR reporting is only for FR2. [4] prefers to wait for R4 before making decisions on capability.

Question 10: Do you agree that FR2 MPE-related P-MPR reporting is an optional per-UE capability?

| Company | Reply (y/n) | Additional comments |
| --- | --- | --- |
| Ericsson | Y |  |
| Nokia, Nokia Shanghai Bell | Y | This was already discussed in RA2#109bis-e and all companies thought a per-UE capability is sufficient. |
| Qualcomm | Y |  |
| ZTE | Y |  |
| OPPO | Y |  |
| vivo | Y |  |
| MediaTek | Y |  |
| Apple | Y |  |
| Interdigital | Y |  |
| Intel | Y | In RAN4 feature list, RAN4 agreed to have per UE. |

Summary of responses

- All companies agree that that FR2 MPE-related P-MPR reporting is an optional per-UE capability

Proposal 5: FR2 MPE-related P-MPR reporting is an optional per-UE capability (10/10).

# Conclusion

RAN2 should discuss the above and agree to the following:

Proposal 1: UE triggers MPE reporting if at least one cell in the MAC entity with a P-MPR ≥ a configurable threshold (per cell). (8/10)

Rapporteur comment: this proposal is just to implement RAN4’s agreement. Other options can be revisited based on further input from R4.

Proposal 2: Support a per MAC entity RRC configuration, whereby the MAC entity reports MPE related P-MPR only when such parameter or IE is configured (10/10)

Proposal 3: For P-MPR reporting prohibit timer, a separate value is configured for MPE reporting procedure per MAC entity (5/10).

Rapporteur comment: This proposal can be viewed independently of the decision of whether a new MAC CE or PHR MAC CE is used to report MPE P-MPR. if we decide to reuse PHR MAC CE, this RRC value applies only for PHR MAC CEs triggered by MPE, and a legacy PHR already reported should not prohibit a new MPE reporting MAC CE from being reported.

Proposal 4: For P-MPR threshold for absolute triggering, a separate value is configured for MPE reporting procedure per MAC entity (9/10).

Proposal 5: FR2 MPE-related P-MPR reporting is an optional per-UE capability (10/10).

In addition, the following issues were discussed and postponed due to lack of input from R4:

Postponed issues:

2.2	P-MPR reporting for multiple cells

Summary of responses

Most companies think this depends on whether a new MAC CE is defined or the PHR MAC CE is reused. If PHR MAC CE is re-used, the issue does not need to be discussed.

Rapporteur comment: RAN4 has not concluded on the number of bits for reported MPE P-MPR, which is important input to decide whether to design a new MAC CE reuse PHR MAC CE. This issue can be postponed until we have further clarity on the MAC CE design. No proposal is made for now. MAC CRs can be looked into next meeting.

2.3	New MAC CE vs. re-use PHR MAC CE

Summary of responses

- 5/10 companies prefer to enhance the PHR MAC CE to include the MPE P-MPR

- 3/10 companies prefer to use a new MAC CE for reporting MPE P-MPR

- 2/10 companies have not expressed a strong preference.

Rapporteur comment: RAN4 has not concluded on the number of bits for reported MPE P-MPR, which is important input to decide whether to design a new MAC CE reuse PHR MAC CE. No sufficient consensus is achieved in this R2 discussion either. This issue can be postponed until we have further clarity on the MAC CE design. No proposal is made for now. MAC CRs can be looked into next meeting.

Q3(a) (on whether FR2 P-MPR triggers apply to legacy PHR):

- 8/10 companies agree that if the PHR MAC CE is reused to report MPE P-MPR, legacy PHR reporting and triggering is not affected by the absolute P-MPR triggering, and the absolute P-MPR trigger should only be applicable for FR2 MPE related P-MPR (rather than legacy P-MPR cause by SAR for example).

Rapporteur comment: This issue can be handled by proper configuration of MPE reporting functionality (see proposal 2) in the appropriate cell groups. Details can be looked into if the PHR MAC CE is reused. No proposal is made for now.

Q3(b) (on whether MPE P-MPR is reported in PHR triggered by legacy non-FR2 triggers):

- 6/10 companies think that if the PHR MAC CE is reused to report MPE P-MPR, then the MPE P-MPR can be reported by any PHR MAC CE regardless of the triggering event type.

- 4 companies expressed that P-MPR reporting should only apply for FR2 MPE triggers, and otherwise the network may not know the event that caused triggering of the PHR MAC CE.

Rapporteur comment: Details can be looked into if the PHR MAC CE is reused. This discussion can be postponed for now.

2.5	Other Details

Need to trigger SR

9/10 do not support triggering a SR if the P-MPR value > a configured threshold. No proposal is made.

eLCID allocation

- 7/10 companies agree to use one-octet eLCID.

- 2/10 companies disagree.

- 1 company answer “N”, though meant they prefer to put this new MAC CE in the new eLCID space

Rapporteur comment: This issue can be postponed until we have further clarity on whether a new MAC CE is introduced. No proposal is made for now.

LCP multiplexing priority

- 9/10 companies agree to use the same LCP multiplexing priority for this MAC CE

Rapporteur comment: This issue can be postponed until we have further clarity on whether a new MAC CE is introduced. No proposal is made for now.

# References

- R2-2004341, LS on MPE enhancements (R4-2005670; contact: Nokia), RAN4

- R2-2004386, Discussion on UE FR2 P-MPR reporting,	OPPO, Huawei

- R2-2004387, Draft Response LS on UE FR2 P-MPR reporting,	OPPO

- R2-2004650, Discussion on the support of P-MPR report,	vivo

- R2-2004651, Draft CR on supporting the P-MPR report,	vivo

- R2-2004652, Draft CR on supporting the P-MPR report,	vivo

- R2-2004778, P-MPR Reporting, 	Apple

- R2-2004906, UE FR2 MPE enhancements and solutions,	 Nokia, Nokia Shanghai Bell

- R2-2004907, Introduction of MPE reporting for FR2,	Nokia, Nokia Shanghai Bell

- R2-2004908, Introduction of MPE reporting for FR2,	Nokia, Nokia Shanghai Bell

- R2-2004909, Introduction of MPE reporting for FR2,	Nokia, Nokia Shanghai Bell

- R2-2004910, Introduction of MPE reporting for FR2,	Nokia, Nokia Shanghai Bell

- R2-2004932, Discussion on MPE enhancements,	Ericsson

- R2-2004936, Implementing MPE enhancements,	Ericsson

- R2-2004938, Implementing MPE enhancements,	Ericsson

- R2-2004939, Implementing MPE enhancements,	Ericsson

- R2-2005126, Considerations on MPE enhancement FR2,	ZTE, Sanechips

- R2-2005138, L2/3 aspects of MPE mitigation,	InterDigital

- R2-2005141, Addition of MPE reporting,	InterDigital

- R2-2005151, Addition of MPE reporting, 	InterDigital

- RP-192227, “Revised WID of NR RF Requirement Enhancements for FR2”, Nokia, 3GPP TSG RAN Meeting #85, Newport Beach, US, Sept 16-20, 2019

- R4-200xxxx, “Email discussion summary for [95e] [121] NR_RF_FR2_req_enh_Part_1”, Oppo