3GPP RAN WG2 Meeting #110e	Draft_R2-200xxxx
June 01th– 12th, 2020

Agenda Item:	6.2.2

Source:	Lenovo

Title:	Summary of NR-U User plane

Document for:	Discussion, Decision

# Introduction

This is the summary report for tdocs submitted to the NR-U user plane agenda items (6.2.2):

# Random Access

## 2-step RACH

[2] points out that according to current specified behavior, UE needs to monitor the PDCCH for Random Access Response identified by the C-RNTI even for cases when MSGA PUSCH wasn’t transmitted due to LBT failure and C-RNTI MAC CE was included in the MSGA MAC PDU. Accordingly, the proposal is to relax the UE behaviour, i.e. UE doesn’t need to monitor PDCCH addressed to C-RNTI for cases when MSGA PUSCH wasn’t transmitted due to LBT failure. Even though the observation made in [2] is correct, the proposed change seems to be an optimization.

Question 1: Do companies see a need to change the current UE behavior such, that UE does not need to monitor the PDCCH identified by C-RNTI when LBT fails for msgA PUSCH transmission

| Company | Reply (y/n) | Additional comments |
| --- | --- | --- |
| LG | No | It’s already discussed in RAN#109 meeting. Agreements … 7. : a UE in connected mode monitors PDCCH addressed to C-RNTI in addition to the MsgB-RNTI, if LBT fails only for the payload part of MsgA (no spec changes required) …. |
| Ericsson | No | Agree with LG, RAN2 has already concluded this issue in RAN2#109. |
| Qualcomm | Yes | There is no point for the UE to monitor C-RNTI and a reasonable UE implementation should be expected not to do that. Naturally, there shouldn’t be a test for this either. So, the question is whether to make the spec compliant with what will practically happen in the field. It is also fine to have a Note that a UE implementation is allowed to not monitor C-RNTI in this case. |
| OPPO | No |  |
| Samsung | N | BTW, the question from the rapporteur seems inaccurate, as UE has to monitor PDCCH addressed to C-RNTI regardless of whether random access is ongoing or not. The question should be updated to 'when LBT fails for msgA PUSCH transmission, whether the reception of PDCCH addressed to C-RNTI completes the Random Access procedure'. Coming to the updated question, this case happens mainly for BFR, and UE in BFR would not receive (unintended) PDCCH from the previous beam in most cases, so to complete the RA procedure (as in the current specification) would be okay. |
| vivo | No | We agree with LG. Since no critical issue is found, we prefer not to revert the achieved agreement at this stage. Besides, if we adopt the proposed solution, the CONNECTED UE cannot monitor the C-RNTI PDCCH for other data scheduling (e.g. DL data) during the RA procedure, which causes unnecessary transmission delay. What’s worse, it also goes against the UE behavior defined in RAN1 spec (i.e. a CONNECTED UE with C-RNTI monitors C-RNTI PDCCH in the SS in a slot where the UE monitors RA-RNTI/MsgB-RNTI). |
| Huawei | Yes | There is no need for the UE to monitor C-RNTI when C-RNTI cannot be sent. |
| Spreadtrum | No | We agree with LG，the issue had been discussed in RAN2# 109. It seems that we have no strong reason to revert the previous agreement. |
| Intel | No | Agree with LG. It can be left to UE implementation if it wants to optimize this case. |
| ZTE | No | First of all, the statement that “UE need not monitor C-RNTI if MSGA PUSCH LBT fails” is not correct! In fact UE has to monitor C-RNTI if it is for CFRA. Further the UE also needs to monitor C-RNTI in case of connected mode during active time regardless of the fact that MSGA PUSCH fails (e.g. RACH due to out of sync case etc). So, even if we have to specify some exceptional case, it will have to be a real corner case and we don’t really think there is any need to reopen this discussion which has already been done once as pointed out above by LG. |
| Nokia | No | Already discussed and concluded. |
| Lenovo | No | Agree with others. |
| MediaTek | No | Agree with other companies that this was already discussed in previous meetings and would be an optimization. |
| InterDigital | No | Agreed with others. The issue was already discussed and concluded. UE needs to monitor C-RNTI in connected mode anyway. |
| Charter Communications | No |  |
| Panasonic | No | RAN2 has already made this agreement in #109 meeting. |
| Fujitsu | No | Agree with LG, this was outcome of email discussion [501]. The email summary in Section 6.2.2 of R2-2001911. |

# LBT related issues

## Discussion on LBT Failure Detection and Recovery During HO with DAPS and CHO

According to the NR-U discussion on the consistent LBT failure detection in the RAN2#109bis meeting, RAN2 made the following agreements [1]:

Agreements: 5 Reply to RAN4 that UL LBT failure detection/recovery is applicable per current specifications to RA in R15-based handover, R15 SN addition/change, and PSCell addition, given the UE is in connected mode. LBT failure detection/recovery is not applicable per current specifications in RRC setup, resume, re-establishment, or release with redirection, as the UE does not have lbt-FailureRecoveryConfig during those procedures 6 Reply to RAN4 that no enhancements are planned in R-16 for UL LBT failure detection and recovery during handover, RRC setup, resume, re-establishment, or release with redirection. However, RAN2 will check if there is any technical issues with DAPS and CHO.

### DAPS HO

In case of DAPS HO, the UE does not detach from source cell upon receiving the RRCReconfiguration message but continues the DL reception from source gNB until the release of source cell as well as continues the UL transmissions to source gNB until successful completion of RACH procedure to target gNB. From UE perspective when being configured with DAPS Handover the UE has two MAC entities: one for the source cell and one for the target cell.

As observed by the contributions the current specification support LBT failure detection and recovery procedure at source cell and target cell for the DAPS HO case. Basically the LBT failure detection and recovery mechanism (lbt-FailureRecoveryConfig-r16) can be configured for both MAC entities during a DAPS HO. Since UE will transmit UL data to source gNB and target gNB simultaneously during the DAPS HO, consistent UL LBT failure may occur in both source and target cell during DAPS HO.

In [12,13] it is pointed out, that the current LBT failure detection procedure may force the UE to perform RACH in the source cell – for cases when multiple BWPs with RACH are configured – while attempting to connect to the target cell, i.e. parallel RACH procedures in source and target cell.

In order to avoid e.g. parallel RACH procedures in source and target cell [13, 21] propose to suspend LBT failure detection and recovery procedure in the source cell during DAPS HO, while for cases when the DAPS handover fails and the UE stays in the source cell, the UE shall resume LBT failure detection and recovery in the source cell. [12] instead proposes to disable BWP switching for the LBT failure detection and recovery procedure at the source cell, which can be achieved by gNB configuration, i.e. configuring RACH resources only at the current BWP.

The following options are possible:

- Option 1: suspend LBT failure detection and recovery at source cell during DAPS HO

- Option 2: disable BWP switching for the LBT failure detection and recovery procedure at source cell

- Option 3: legacy LBT failure detection and recovery procedure at source cell, i.e. no change required

Question 2: Which option do you prefer?

| Company | Preferred option | Additional comments |
| --- | --- | --- |
| LG | Option 3 | We don’t see this is issue. For case of parallel RACH, it’s a case that each RACH is performed in its MAC entity (i.e. one in source MAC, the other in target MAC). It is similar with two Ras in MCG MAC and SCG MAC. |
| Ericsson | Option 1 | Support parallel RACH procedures, would mean that the UE needs to split its transmission power between the source and the target cell. As specified in 38.211 clause 7.2, the PRACH transmission power is determined according tot the below formula. [dBm], UE must be configured with two Pcmax settings, one for the source cell, another cell for the target cell. Although such UE behaviors would be much like dual connectivity, RAN1 would still need to spend extra standardization efforts to support power split. This would not be feasible for Rel-16 given limited time is left. Option 2 disables the BWP switch in the source cell. The gNB is required to only configured RACH resource in one BWP, which limits the configuration flexibility of the gNB. An alternative would be to not require configuring only one BWP with RA resources, but instead require that LBT RLF is triggered immediately if consistent LBT failure is detected in one BWP and then combine this with a yes on Question 3 below. Option 3 is incomplete since it doesn’t cover the case where the UE has detected consistent LBT failure in a BWP, while there are other BWPs available. In this case, the UE triggers RACH to switch to another BWP. Therefore, Option 1 is the best option, which minimizes the standardization efforts. Meanwhile, the source gNB can detect by itself (see [13]) if the UE is experiencing consistent LBT failure in its active BWP in the source cell, upon which, the source gNB orders to the UE to switch to another BWP in the source cell to continue data transmission/reception or the source gNB may stop transmissions to the UE. |
| Qualcomm | Option 2 | We should align LBT failure handling with DL based RLM for DAPS. For RLM during DAPS, the UE will detect failure at the source but will not perform recovery (i.e.re-establishment). The UE will use the detection to stop transmission/reception with the source. The analogous for LBT case is to detect failure upon which communication with the source is stopped but there is no BWP switching or RACH attempt to report the failure. Option 1 is similar to what was done for Rel-14 LTE MBB, which is also acceptable, but it is better to have consistent behavior for NR Rel-16. Note that if DL LBT fails for SSB, this will cause RLF and the UE will behave as mentioned above and DL and UL LBT failure should have the same outcome. Option 3 is not acceptable. The underlying principle of DAPS was that UE holds on to the source (which is likely degrading due to HO) as long as possible (or until target releases it whichever comes first) but does not try to recover that link. The UE instead tries to spend its resources at the target as much as possible and that is why there is prioritization of target UL transmissions over source. RACH attempts at the source while the UE is communicating with the target, even if the UE is dual UL capable, will waste UE power which can be used for target transmission. |
| OPPO | Option 3 | We don’t see the issue on supporting LBT detection and recovery for source cell. |
| Samsung | Option 3 | - |
| vivo | Option 3 | For simplicity, we prefer to align the UE behavior of LBT failure detection and recovery in different cases. Note that both Option 1 and 2 cannot solve the power split issue due to parallel RA procedures. This is because the RA procedure at the source cell can be initiated by other RA triggers (e.g. BFR). |
| Huawei | Option3 | We don’t see parallel RACH as an issue for 4arget4ng the current UE behavior for continuing the LBT detection at the source MCG, even if some capability is defined for simultaneous UL transmission between source MCG and 4arget MCG in DAPS. |
| Spreadtrum | Option 3 | We don’t see any issue on LBT detection and recovery during DAPS .In the mobility WI, the parallel rand access procedures in source cell and in target cell are permitted from RAN2 perspective and RAN1 can treat this situation. |
| Intel | Option 3 | We do not see the need to change anything here. Option 1 and 2 can be done via network implementation when the network configures DAPS configuration. |
| ZTE | Option 3 | We also don’t understand the issue with parallel RACH as mentioned by other companies above. So, if there is no issue, we should not try to fix it. |
| Nokia | Option 3 | RACH can be triggered for other purpose in the source as well. We do not see need to change LBT failure detection due to DAPS to avoid RACH in source. It’s rare case anyway. |
| Lenovo | Option 3 | Power control aspects for DAPS HO have been already discussed in RAN1 (TS38.213 section 15). Also RACH may be triggered for other reason as mentioned by others. Therefore we don’t see any NR-U (LBT) specific issue here. |
| MediaTek | Option 3 | We do not see any reason for optimizations in this case. RACH in source cell could happen because of other reasons and is not restricted to DAPS HO. Parallel RACH can also happen in other scenarios, such as DC. However they are performed in different MAC entities, not in a single MAC entity. If required, the network can avoid autonomous BWP switching by the UE (due to consistent LBT failures) by configuring only one UL BWP with RACH occasions. |
| InterDigital | Option 3 | There can be parallel RACHs but in different MAC entities, and this is not specific to NR-U. |
| Charter Communications | Option 3 | Support of parallel RACH at the source and target cells does not necessarily mean PRACH transmission at the same time hence splitting the power. In fact it’d be a rare occasion if that happens. Therefore, it seems sticking with the legacy LBT failure detection and recovery procedure at source cell would be fine. |
| Panasonic | Option 3 | We do not see any strong reason to change behavior here. |
| Fujitsu | Option 3 | We don’t see the issue. We agree that RAN1 spec needs to be updated considering power split between UL transmission in source MAC and UL transmission in target MAC to support DAPS HO. However, it requires no extra standardization efforts for parallel RACHs if DAPS HO is supported by RAN1 spec. For now, the parallel RA procedure is allowed for which we see no problem. This means that there seems no problem with the parallel RA procedure which is triggered by LBT failure. If the parallel RA procedure is problematic, it is better to be discussed in FeMOB WI. Besides, there are many triggers for RA. If RAN2 agrees that parallel RACH procedures during DAPS HO are not expected, RA in source MAC should be suspended instead of LBT detection and recovery. |

It should be noted that according to current TS38.331 specification (section 5.3.10.3), consistent uplink LBT failures indication from MCG MAC is not considered as source radio link failure condition. Therefore, even if source MCG MAC indicates consistent uplink LBT failures to the RRC, DRB(s) with DAPS configured will not be suspended and the source connection is not released.

In [11,12,16,17,19,20,22] it is proposed, that if DAPS HO is applied for a NR-U cell, the detection of radio link failure should also consider the case of consistent LBT failure at the source cell. UE triggers source RLF upon indication of consistent UL LBT failures, so that the all the DRBs are suspended in the source and the source connection is released.

Question 3: Do companies support that for DAPS HO, upon indication of consistent uplink LBT failures from source MCG MAC UE declares RLF, i.e. the UE should suspend all DRBs in the source and release the source connection, i.e. same behaviour as for source RLF?

| Company | Reply (y/n) | Additional comments |
| --- | --- | --- |
| LG | Yes | It can be achieved by adding the indication to triggers of source RLF in the current specification. Then, re-establishment can be triggered at T304 expiry. |
| Ericsson | No | It is better to suspend LBT failure detection and recovery procedure in the source cell in case of DAPS HO. The source gNB can detect by itself (see [13]) if the UE is experiencing consistent LBT failure in its active BWP in the source cell, upon which, the source gNB orders to the UE to switch to another BWP in the source cell to continue data transmission/reception or it stops transmitting to the UE. In case the source gNB has detected that the UE has experienced consistent LBT failure in all configured BWPs in the source cell, the source gNB can send a RRC reconfiguration to order the UE to suspend all DRBs in the source and release the source connection. |
| Qualcomm | Yes | We should have consistent behavior for all RLF causes at the source. |
| OPPO | Yes |  |
| Samsung | Y | - |
| vivo | Yes | We think that consistent UL LBT failure can be considered as a kind of RLF. Thus, the legacy principle can be reused for the source cell where consistent UL LBT failure is detected. |
| Huawei, HiSilicon | Yes |  |
| Spreadtrum | Yes |  |
| Intel | Yes |  |
| ZTE | Yes |  |
| Nokia | Yes |  |
| Lenovo | Yes |  |
| MediaTek | Yes | The existing actions for other RLF cases in 38.331 5.3.10 can be applied for resolving this issue. |
| InterDigital | Yes |  |
| Charter Communications | Yes |  |
| Panasonic | Yes | In order to solve the issue, we also think the lagecy RLF principle can be applied here. |
| Fujitsu | y | It is consistent with normal operation case (i.e. existing RLF behavior in source). |

According to the submitted contribution, the majority doesn’t seem to see a need to change the behaviour for cases when consistent LBT failure is detected at the target cell. For the target MCG, the behaviour should be same as for legacy non-DAPS handover.

[15] proposes as an enhancement, that the information on consistent UL LBT failures occurring during Random Access in target cell should be reported to source gNB for RRM purpose.

In [20,21] it is proposed that the UE should consider T304 timer as expired upon indication of consistent UL LBT failure in target cell.

The following options are possible:

- Option 1: UE considers RLF to be detected upon detection of consistent UL LBT failures at the target cell

- Option 2: UE doesn’t consider RLF to be detected upon detection of consistent UL LBT failures at the target cell, i.e. rely on T304 timer

- Option 3: UE considers T304 timer as expired upon detection of consistent UL LBT failures at the target cell

Question 4: Which option do you prefer for handling of consistent UL LBT failures detected at the target cell for DAPS HO?

| Company | Preferred option | Additional comments |
| --- | --- | --- |
| LG | Option 2 |  |
| Ericsson | Option 2 | Option 3 goes against prevous agreement, but may be ok if companies insist. |
| Qualcomm | Option 3 | Note that we also need to fix this for legacy HO as the current CR has Option 1. Assuming Option 1 is not acceptable to anyone, we need to make changes anyway and it is better to do the more optimal solution. |
| OPPO | Option 2 |  |
| Samsung | Option 3 but (see comments) | Agree that same procedure is applied for DAPS and non-DAPS case. Option 2 is not aligned with non-DAPS HO. Option 1 is captured in RRC specification for non-DAPS HO. However, there is an issue with this procedure (raised by RIL S058) and is discussed in CP email discussion #501. So we can follow the outcome of discussion #501 for DAPS HO. |
| vivo | Option 1 | Consistent UL LBT failure should be considered as a kind of RLF. Thus, the legacy principle for the SCG RLF can be reused. |
| Huawei, HiSilicon | Option2 | T304 can already handle this while if we have UL LBT detection in place in this scenario, it would be a duplciation of functionality. |
| Spreadtrum | Option 3 | We also think the information on consistent UL LBT failures occurring during Random Access in target cell should be reported to source gNB for RRM purpose. |
| Intel | Option 2 during DAPS HO Option 1 when DAPS is configured | During the DAPS HO, it will just depend on the T304 expiry, just like regular handover. After DAPS is configured, the consistent LBT failure detection and recovery mechanism is used and if consistent LBT failure is detected, it will trigger RLF. Our understanding is that eMOB WI may still be updating the DAPS configuration part during RLF and we should ensure that we are aligned with eMOB behavior during DAPS HO and after DAPS is configured. |
| ZTE | Option 2 |  |
| Nokia | - | All options works. Same should be applied to both non-DAPS case and DAPS for target. If UE continues trying in the target when consistent LBT failure happens, then option 2. For non-HO case, re-establishment is triggered when consistent LBT failure happens, no strong reason to do differently for HO since otherwise the UE will be just waiting in the target until the channel is available. But no problem with that either since it should be anyway rare case and it seem to be the simplest option. If not to continue in target, with DAPS and source not failed, target should be released with both option 1 and 3; while if source also failed or non-DAPS HO case, re-establishment from the source for option 3 or from the target for option 1. For non-DAPS HO, it has been captured RLF is triggered and re-establishment performed from target (should be no issue since the target cell is anyway prepared), no issue to go for option 1 for HO with DAPS as well. Option 3 is simple to model by applying the same actions as T304 expiry. If to adopt it, should be for both non-DAPS and DAPS. |
| Lenovo | Option 2/3 | Even though we agreed in the last meeting that no enhancements are necessary/done for legacy HO case, we see some benefit of Option 3 - triggering an early HOF in case of LBT falure. However Option 2 would be also sufficient. In our view we should have same behavior for both non-DAPS case and DAPS for target. Since non-DAPS case will be treated in CP session first, we should align behavior for DAPS HO case then. |
| MediaTek | Option 3 | Depending on the configuration (e.g. the timer values), consistent LBT failure condition can be detected on the target cell before the T304 expiry. It is unlikely that the conditions on the target cell wrt LBT will improve after consistent LBT failure is detected by the UE and before the expiry of T304. It is therefore better to take the same actions as T304 expiry i.e. HO failure earlier as it may speed up the recovery. |
| InterDigital | Option 3 | Assuming delay is an issue for recovery, e.g. whent T304 is large. Otherwise Option 2 is okay. |
| Charter Communications | Option 2 |  |
| Panasonic | Option 2 |  |
| Fujitsu | Option 2 | We prefer option 2. For DAPS HO, detection of RLF is performed in source only, i.e. no RLF detection in target. In order to allow fallback to source when DAPS handover fails but RLF is not detected in the source PCell, we prefer to keep align with current mechanism regarding RLF detection for consistent UL LBT failure indication, in target (which requires no change to specification). It means no RLF is declared including upon receiving consistent UL LBT failure indication in target during DAPS HO. For both option 1 and option 3, the reason is unclear. Given that CFRA is dominant in DAPS due to latency reduction instead of CBRA, the network can configure proper UL BWP that is not heavily suffering from LBT failure by HO command. Therefore, in most cases, LBT failure seems rare case and T304 expiry is dominant case. |

### CHO/CPC

All contributing companies [11,12,13,14,16,17,18,19,23] conclude, that UL LBT failure and recovery mechanism can be supported in CHO and CPC without additional specification change. Basically for the CHO case UE will handle LBT detection and recovery as during a normal handover, which was agreed by RAN2 in RAN2#109bis. If the consistent LBT failure occurred in source cell before one candidate cell met the execution condition, the UE can perform the recovery in source cell as in normal connected state, e.g. UE will perform BWP switch and/or trigger the RLF. For cases when the execution condition is met, configuration for that selected candidate CHO cell will contain the UL LBT failure and recovery mechanism just like in regular handover command. Hence CHO inherently supports LBT failure detection and recovery mechanism for target cell, if it is configured. Also for the CPC procedure no enhancements are required, since UL LBT failure detection/recovery is applicable per current specifications to RA in PSCell addition.

Question 5: Do companies agree that UL LBT failure and recovery mechanism can be supported in CHO and CPC without additional specification change?

| Company | Reply (y/n) | Additional comments |
| --- | --- | --- |
| LG | Yes |  |
| Ericsson | Yes |  |
| Qualcomm | Yes |  |
| OPPO | Yes |  |
| Samsung | Y | - |
| vivo | Yes | According to the current RRC specification, the CHO and CPC can be inherently supported for NR-U. |
| Huawei, HiSilicon | Yes |  |
| Spreadtrum | Yes |  |
| Intel | Yes |  |
| ZTE | Yes |  |
| Nokia | Yes |  |
| Lenovo | Yes |  |
| MediaTek | Yes |  |
| InterDigital | Yes |  |
| Charter Communications | Yes |  |
| Panasonic | Yes |  |
| Fujitsu | y |  |

### Fast MCG link recovery

In R16 CA/DC enhancement, fast MCG link recovery is supported which means UE can be configured to report the MCG failure information to the MN via the SCG without performing connection re-establishment. [18] raises the question whether UE should report MCG failure instead of performing RRC re-establishment upon detecting consistent UL LBT failures for cases when fast MCG link recovery is configured.

Question 6: Do companies support that UE reports MCG failure instead of performing RRC re-establishment upon detecting consistent UL LBT failures for cases when fast MCG link recovery is configured?

| Company | Reply (y/n) | Additional comments |
| --- | --- | --- |
| LG | Yes |  |
| Ericsson | Yes |  |
| Qualcomm | Yes |  |
| OPPO | Yes |  |
| Samsung | Y but (see comments) | This can be applied as for other MCG failure cases (except for handover scenario) |
| vivo | Yes |  |
| Huawei, HiSilicon | Yes |  |
| Spreadtrum | Yes |  |
| Intel | Yes |  |
| ZTE | Yes |  |
| Nokia | Yes |  |
| Lenovo | Yes |  |
| MediaTek | Yes |  |
| InterDigital | Yes |  |
| Charter Communications | Yes |  |
| Panasonic | Yes |  |
| Fujitsu | y |  |

## UL LBT failure MAC CE

In [3] it is pointed out, that according to the current specifications it’s not clear when UE shall use the one-octet LBT failure MAC CE respectively the four-octet LBT failure MAC CE. Similar to the BFR case it is proposed to specify the conditions for choosing the corresponding LBT failure MAC CE format. More specifically it’s suggested that for cases when the highest ServCellIndex of this MAC entity's SCell for which LBT failure is detected is less than 8 UE uses the one-octet LBT failure MAC CE format, otherwise the four-octet format is used.

Question 7: Do companies agree to clearly specify the conditions when to use the one-octet LBT failure MAC CE or the four-octet LBT failure MAC CE, i.e. one-octet format is used when the highest ServCellIndex of this MAC entity's SCell for which LBT failure is detected is less than 8, otherwise four-octet format is used?

| Company | Reply (y/n) | Additional comments |
| --- | --- | --- |
| LG | Yes |  |
| Ericsson | Yes | This makes sense, since the same behaviors have been already agreed for BFR MAC CE in Rel-16. |
| Qualcomm | Yes |  |
| OPPO | Yes |  |
| Samsung | Y | The changes are merely for the consistency, as for all other MAC CEs from UE. |
| vivo | Yes | It makes the UE behavior clearer. |
| Huawei, HiSilicon | Yes |  |
| Spreadtrum | Yes |  |
| Intel | Yes | Follow BFR |
| ZTE | No strong view |  |
| Nokia | Yes |  |
| Lenovo | Yes |  |
| MediaTek | Yes |  |
| InterDigital | Yes |  |
| Charter Communications | Yes |  |
| Panasonic | Yes |  |
| Fujitsu | y |  |

## LBT Counter

[4] makes the observation, that the variable LBT_COUNTER should be per Serving Cell. However, from current specification it’s not clear though, that LBT_COUNTER is maintained per Serving Cell. Rapporteur agrees that from past discussions of the LBT failure detection procedure, the common understanding was that the LBT_COUNTER is per Serving Cell.

Question 8: Do companies agree that LBT_COUNTER is per Serving cell and that specification should be correspondingly updated to clearly capture this?

| Company | Reply (Y/N) | Additional comments |
| --- | --- | --- |
| LG | Yes / No | Yes, we agree that LBT_COUNTER is per serving cell. However, we have doubt on need of spec. update. Because the current specification for LBT_COUNTER is already as clear as that of BFI_COUNTER is. |
| Ericsson | Yes/No | Agree with LG |
| Qualcomm |  | This sentence in the CR seems good enough: For each activated Serving Cell configured with lbt-FailureRecoveryConfig, the MAC entity shall: |
| OPPO | Yes | We agree it’s per serving cell, it would be good to update. The proposed change in [4] is ok for us. |
| Samsung | Y | (minor correction) |
| vivo | Yes / No | Same view as LG. |
| Huawei, HiSilicon | Yes | We consider the revision as benificial for the integrity of the spec, e.g., with a LBT_COUNTERs (in plural) for alignment with counter per serving cell. |
| Spreadtrum | Yes/No | Agree with LG |
| Intel | Yes/No | But this is already clear. |
| ZTE | Yes | We are okay with this minor clarification. |
| Nokia | Yes |  |
| Lenovo | Yes/No | Agree with LG |
| MediaTek | Yes, and | The counter is per serving cell, and this should be made clear in the specifications to avoid misinterpretations. The confusion arises because the lbt-FailureRecoveryConfig is defined per cell group rather than per serving cell. We think that lbt-FailureRecoveryConfig should also be configured per serving cell. For example, it could be included in BWP-UplinkDedicated, similar to beamFailureRecoveryConfig. With this change, for the case where a subset of the cells in the cell group are operating in shared spectrum, the configuration can be provided on per cell basis. |
| InterDigital | Yes | This is a useful clarification. |
| Charter Communications |  | Agree that the counter is per serving cell. However, as others mentioned this can be asserted from already from the spec. |
| Panasonic | Yes/No | Agree with LG. |
| Fujitsu | Yes, but | The MAC needs to be updated, but we suggest another TP. As in the current MAC, it says that “LBT_COUNTER: counter for LBT failure indication which is initially set to 0”. The fundamental unclarity on LBT_COUNTER in TS38.321 seems to come from whether “LBT failure indication” is per Serving Cell or not. Once the “LBT failure indication” is clarified to be per Serving Cell, the unclarity of LBT_COUNTER is solved. And in fact, most companies thought that “LBT failure indication” is per Serving Cell in the last meeting, so that the clarification seems ok. The suggested text is shown below, which is similar with beam failure indication: 5.21.1 General The lower layer may perform an LBT procedure, see TS 37.213 [18], according to which a transmission is not performed if the channel is identified as being occupied. When lower layer performs an LBT procedure before a transmission and the transmission is not performed, an LBT failure indication on the Serving Cell(s) is sent to the MAC entity from lower layers. Unless otherwise specified, when LBT procedure is performed, actions related to "is transmitted" and "transmission is performed" shall not be performed if an LBT failure indication is received from lower layers. |

# Configured Grant

## Initial “Pending” status of a HARQ process

For NR-U a new status, i.e. “pending/not pending”, associated with a HARQ process was introduced in order to allow for autonomous retransmissions for cases where CG PUSCH transmission couldn’t be performed due to LBT failure. Whether a HARQ process is considered as pending or as not pending depends on whether a corresponding transmission has been performed or not. In [5] it’s identified that the initial status [pending/not pending] of a HARQ process upon activation/configuration of a configured grant is currently not clearly specified, which leads to a situation that UE is not able to perform the very first (initial) transmission on a HARQ process according to current MAC procedures.

Question 9: Do companies agree, that all HARQ processes associated with a Configured grant shall be considered as not pending upon activation/configuration of the Configured grant?

| Company | Reply (Y/N) | Additional comments |
| --- | --- | --- |
| LG | Yes | Upon activation/configuration of the configured grant, that HARQ process is not pending because no MAC PDU is expected to be obtained for that HARQ process. |
| Ericsson | Yes | Agree with LG. |
| Qualcomm | Yes |  |
| OPPO | Yes |  |
| Samsung | Y | No strong view but this seems quite obvious, so not sure whether it should be captured explicitly. Can follow the majority. |
| vivo | Yes |  |
| Huawei, HiSilicon | Yes |  |
| Spreadtrum | Yes |  |
| Intel | Yes |  |
| ZTE | Yes | We agree to set the initial “not pending” status of HARQ processes when they are associated with configured grant for the first time. Considering the HARQ processes may be shared for multiple configured grants, the HARQ process used for configured grant type 2 may be used for configured grant type 1. Hence we think it is reasonable that the initial “not pending” status is set upon configuration of the configured grant, not upon activation. In addition, since some HARQ processes may have been configured/used, only some HARQ processes which are not associated with other configured grants should be set the initial “not pending” status. |
| Nokia | Yes |  |
| Lenovo | Yes |  |
| MediaTek | Yes |  |
| InterDigital | Yes |  |
| Charter Communications | Yes |  |
| Panasonic | Yes |  |
| Fujitsu | Y | The meaning of “pending” is clear. For example, the meaning of SR “pending” has been specified for a long time in TS38.321, where there have been no unclarity and misunderstanding i.e. “SR is pending” occurs after “SR is triggered”. This should be the same for HARQ process i.e. “HARQ process is pending” occurs after “HARQ process is generated”. |

[5] proposes to specify in TS38.331 (field description of harq-ProcID-Offset) that all HARQ processes associated with a CG grant are considered as not pending upon activation/configuration of the configured grant. Rapporteur thinks, that since the [pending/not pending] status of a HARQ process is a MAC internal status, it may be more suitable to specify the intial status of a HARQ process in TS38.321 (section 5.4.2.2). Therefore the following options are possible:

- Option 1: 	Specify in TS38.331 (field description of harq-ProcID-Offset) that initial status of HARQ
 processes associated with a CG grant is “not pending” upon configuration/activation of a configured grant

- Option 2: 	Specify in TS38.321 that initial status of a HARQ process associated with a configured grant is not pending upon configuration/activation of a configured grant (if cg-RetransmissionTimer  is configured for the HARQ process)

- Option 3:	No specification change required (spec is already clear on this)

Question 10: Which option do you prefer for specifying the initial status [pending/not pending] of a HARQ process?

| Company | Preferred option | Additional comments |
| --- | --- | --- |
| LG | Option 3 | Based on the below texts, the intention of “pending” upon LBT failure indication is read to focus on indicating unsuccessful transmission by LBT failure, and it is not difficult to consider the initial status of a HARQ process as “not pending”. R2-2005331 Corrections of NR operating with shared spectrum channel access in 38.321 (Ericsson, Nokia) 5.4.2.1 HARQ Entity … 5> if the transmission is performed and LBT failure indication is not received from lower layers: 6> consider the identified HARQ process as not pending. 5> else: 6> consider the identified HARQ process as pending. … TS 38.321 V16.0.0 5.4.2.2 HARQ process … When cg-RetransmissionTimer is configured and the HARQ entity obtains a MAC PDU to transmit, the corresponding HARQ process is considered to be pending. A pending HARQ process is pending until a transmission is performed on that HARQ process or until the HARQ process is flushed. … |
| Ericsson | Option 3 | Option 2 is acceptable and better than option 1. |
| Qualcomm | Option 2/3 | Option 1 is not good; we should not use MAC internal variables in RRC descriptions. |
| OPPO | Option 3 | If any clarification is needed on this, we prefer to make it in MAC instead of RRC. |
| Samsung | Option 3 | (can follow majority views as said above) |
| vivo | Option 2 | Agree with rapporteur. |
| Huawei, HiSilicon | Option2 | We are fine with either a 14efinition of the initial status by MAC or RRC. The issue with not defining this is on the very first transmission. Without a definition of the HARQ stasus, at the very first UL transmission, the UE can take the status as either pending or not pending. In UL grant reception, the following conditions need to be examined. 2> else if the cg-RetransmissionTimer for the corresponding HARQ process is configured and not running, then for the corresponding HARQ process: 3> if the configuredGrantTimer is not running, and the HARQ process is not pending (i.e. new transmission): 4> consider the NDI bit to have been toggled; 4> deliver the configured uplink grant and the associated HARQ information to the HARQ entity. 3> else if the previous uplink grant delivered to the HARQ entity for the same HARQ process was a configured uplink grant (i.e. retransmission on configured grant): 4> deliver the configured uplink grant and the associated HARQ information to the HARQ entity. Consider, for the very first transmission, if the status of the HARQ process is pending, the UE would not consider the NDI to be toggled and perform retransmission, while the HARQ buffer is actually empty |
| Spreadtrum | Option 3 | Option2 is better than option1 if some clarification is needed. |
| Intel | Option 3 | Agree with LG. We think it is already clear in the specification based on the pending and not pending definition. |
| ZTE | Option 2 |  |
| Nokia | Option 3 |  |
| Lenovo | Option 2/3 | No strong view . |
| MediaTek | Option 2 | It should be clarified in the MAC spec. |
| InterDigital | Option 3 | The MAC text mentioned by LG is sufficient, but option 2 is okay if further clarification is needed.. |
| Charter Communications | Option 3 |  |
| Panasonic | Option 3 |  |
| Fujitsu | Option 3 | We agree with LG that current text is already clear on this. Also see Q9. In addition, according to description in 5.4.2.2 that “When cg-RetransmissionTimer is configured and the HARQ entity obtains a MAC PDU to transmit, the corresponding HARQ process is considered to be pending”, the HARQ process is considered to be pending if the HARQ entity obtains a MAC PDU to transmit. In case of activation/configuration of the configured grant, the HARQ process will not be considered to be pending since the HARQ entity has not obtain a MAC PDU to transmit. |

# Conclusion

RAN2 should discuss the above and agree to the following:

[TBA]

# References

- R2-2003804, Session minutes for NR-U, Power Savings, NTN and 2-step RACH, RAN2 Session chair (InterDigital)

- R2-2004975, Remaining issue on 2-step random access, Huawei

- R2-2004425, Clarification on when to use one-octet or four-octet LBT failure MAC CE, Samsung

- R2-2004426, Clarification on the variable LBT_COUNTER, Samsung

- R2-2005054, Text proposal on pending status for HARQ process in NRU, Huawei

- R2-2005330, UE declaring beam failure due to LBT failures during active TCI switching, Ericsson

- R2-2004671, Beam failure declaration due to TC switching failure, Intel Corporation

- R2-2004616, UE declaring beam failure due to LBT failures during active TCI switching, ZTE Corporation, Sanechips

- R2-2004598, Discussion about LBT failure and beam failure relation, Nokia, Nokia Shanghai Bell

- R2-2004420, LBT Impacts on the TCI State Switching, Vivo

- R2-2005713, Consideration on DAPS and CHO in NR-U, ZTE Corporation, Sanechips

- R2-2005550, Handling of UL LBT failure for DAPS and CHO, Qualcomm Incorporated

- R2-2005329, LBT failure detection and recovery for DAPS and CHO, Ericsson

- R2-2005050, Consistent LBT failure in CHO,	 Spreadtrum Communications

- R2-2005049, Consistent LBT failure in DAPS, Spreadtrum Communications

- R2-2004974, LBT failure recovery for DAPS and CHO, Huawei, HiSilicon

- R2-2004659, UL LBT failures interactions with DAPS and CHO, Intel Corporation

- R2-2004549, Remaining issues on UL LBT failure, OPPO

- R2-2004419, Discussion on LBT Failure Detection/ Recovery During HO with DAPS and CHO, vivo

- R2-2004529, Consistent LBT Failure Handling during Handover, Samsung Electronics Co., Ltd

- R2-2004694, On Applicability of DAPS Handover in NR-U	, MediaTek Inc.

- R2-2005617, Discussion on issues with DAPS in NR-U, LG Electronics Deutschland

- R2-2004696, On Conditional Handover in NR-U, MediaTek Inc.