3GPP TSG-RAN WG2 Meeting #110-e		R2-2005855

Electronic, 1st Jun. – 12th June. 2020

Title:	[Draft] LS to RAN4 on RRM relaxation in power saving

Response to:	-

Release:	Rel-16

Work Item:	NR_UE_pow_sav-Core

Source:	vivo [to be RAN2]

To:	RAN4

Cc:

- Contact Person:

- Name:	                Chenli

E-mail Address:	 Chenli5g@vivo.com

Attachment:	             R2-2006008.zip (Report of [Post109bis-e][940]PowSav RRM open)

- 1. Overall Description:

RAN2 have discussed NR power saving RRM measurement related issues and the following agreements have been achieved:

- The description for parameter combineRelaxedMeasCondition in TS 38.304 can be (baseline) as described in the attached paper in R2-2006008.  Definition can be updated during CR revision and we will align with RAN4.

- For Proposal 3 in R2-2006008: case A – remove option 3, Case C and G – remove option 2 and 3

- ASK RAN4 in general LS:  The parameter highPriorityMeasRelax is kept and defined as follows as a baseline:

If highPriorityMeasRelax is configured and set to True, and the criterion is fulfilled,

  - the UE can perform relaxed measurement for higher priority frequency. How to relax measurement for higher priority frequency is up to the conclusion of RAN4

Otherwise (i.e. highPriorityMeasRelax is not configured,)

  - The UE doesn’t stop measurements for higher priority frequency and the UE should continue searching according to Thigherprioritysearch.  Update if RAN4 has concerns.

- Network configuration should ensure that SSearchThresholdP ≤ SIntraSearchP, and SSearchThresholdQ ≤ SIntraSearchQ, as well as, SSearchThresholdP ≤ SnonIntraSearchP, and SSearchThresholdQ ≤ SnonIntraSearchQ.

- Capture the following restriction in the field description in TS 38.331: Network configuration should ensure that SSearchThresholdP ≤ SIntraSearchP, and SSearchThresholdQ ≤ SIntraSearchQ, as well as, SSearchThresholdP ≤ SnonIntraSearchP, and SSearchThresholdQ ≤ SnonIntraSearchQ.  Capture: The network configures A smaller or equal to B

- Capture in TS 38.304 that “Less than 1 hour have passed since measurements for cell selection/reselection were last performed”. How to capture it can be discussed in CR discussion.  Let RAN4 know cell selection is added here.

- RAN2 understands that RRM measurement relaxation method with scaling factor should be captured in RAN4 specification (TS 38.133). And RAN2 specification (TS 38.304) should refer to the relevant section in TS 38.133.

- RAN2 understands the use case, in which the UE may choose to not perform any NR inter/intra-frequencies or inter-RAT frequencies measurements, should be captured in TS 38.304

- It is up to RAN4 to conclude the measurement relaxation method to be adopted when both criteria are fulfilled if combineRelaxedMeasCondition is not configured.

- 2. Actions:

- To RAN WG4

- RAN2 kindly asks RAN4 to

- take the above information into account and

- provide feedback, if RAN4 have any concern on the above agreements.

- 3. Date of Next RAN2 Meetings:

TSG RAN WG2 Meeting #111-e		24 – 28 Aug. 2020				e-Meeting

TSG RAN WG2 Meeting #111bis-e		12 – 16 Oct. 2020				e-Meeting