3GPP TSG-RAN WG2 Meeting #111 electronic	R2-2006564

E-Meeting,August 17th - 28th, 2020

Source:	vivo

Title:	Identify positioning integrity use case and KPIs

Agenda Item:	8.11.3.1

Document for:	Discussion and Decision

Introduction

At the RAN#86 meeting, the study item on NR Positioning Enhancements RP-200928[1] was approved. From RAN2’s perspective, the SI includes the following objectives:

Study solutions necessary to support integrity and reliability of assistance data and position information: [RAN2] - Identify positioning integrity KPIs and relevant use cases. - Identify the error sources, threat models, occurrence rates and failure modes requiring positioning integrity validation and reporting. - Study methodologies for network-assisted and UE-assisted integrity.

In this contribution, we present our views about the positioning integrity KPIs and relevant use cases.

- Discussion

In TS22.872[2], integrity is defined as

-	Integrity: measure of the trust in the accuracy of the position-related data provided by the positioning system and the ability to provide timely and valid warnings to the UE and/or the user when the positioning system does not fulfill the condition for intended operation.

Integrity is one of the criteria to evaluate GNSS performance, which was firstly introduced in the aviation field. It is a measure of trust which can be placed in the correctness of the information supplied by the total system. This system is called Safety-of-Life (SoL) system. In an SoL system, when the system is not available to achieve the integrity requirements, a warning should be provided to the user(pilot).

In this document, we first analyze the KPIs and use cases for GNSS in the current civil aviation system. Secondly, we identify the use cases that needed to introduce integrity and KPIs which are also appropriate for the wireless system and give an example of how these KPIs work for NR use cases.

KPIs and use cases for the aviation system

For aviation integrity[3], there are four KPIs defined as following:

Alert Limit: The alert limit for a given parameter measurement is the error tolerance not to be exceeded without issuing an alert.

Time to Alert: The maximum allowable time elapsed from the onset of the navigation system being out of tolerance until the equipment enunciates the alert.

Integrity Risk: Probability that, at any moment, the position error exceeds the Alert Limit.

Protection Level: Statistical bound error computed to guarantee that the probability of the absolute position error exceeding said number is smaller than or equal to the target integrity risk.

Let’s suppose the positioning result  X

X=Xv+Ex

where

Xv is the real location.

Ex.is the random error, Ex~N(μ,σ2) and the probability density function (PDF)[4] is

fExe=1σ2∙πe-12e-μσ2

In this case, we assumed that the PL (Protection Level) is Cx and should make sure the

P|Ex|>Cx≤IR(integrity risk)

The probability P|Ex|>Cxis shown as below function and figure 1

P|Ex|>Cx= 2c∞fExede

Figure 1 probability density function

The probability that P|Ex|>Cx. can be seen as IR(Integrity Risk), which is defined as the probability that Ex exceeds the confidence interval Cx.

Similar to the PL, the Alarm Limit ( AL ) is the same value, such as Cx' , which ask the probability of the region (|Ex|>Cx') within a probability P.

It is noted that the value of AL and PL determines the availability of the integrity system

- If ( PL >AL ), the estimate of integrity cannot be assessed, then the integrity system is unavailable;

  - And if (Ex<AL<PL),  a false alarm is generated even though Ex<AL

- If ( PL <AL), the integrity of the estimate is said to be available.

The availability of the integrity system would be analyzed in detail combing with Figure 2.

Figure 2 The Stanford diagram of PE

When PL > AL, the system is not available and an alert is triggered.(PE-1,PE-2,PE-3).

When PL< AL, the system is available and under integrity protection. (PE-4, PE-5, PE-6, and PE-6 is a dangerous risk)

A use case of the civil aviation system of ICAO(International Civil Aviation Organization) is introduced as an example. It has many typical operations, such as APV-I(Approach procedure with vertical guidance) and APV-II is designed for landing, EN-route is used for aircraft flying on the ocean or continent. And the KPI is different for those different operations.

A typical operation EN-route is described in Figure 3.

Figure 3 Integrity protection applied in ICAO

The integrity system for EN-route is for protecting the aircraft fly in an area of the route(within the blue line in figure 3), and if the aircraft flies out of the boundary(orange line), an alert is triggered.

the target integrity risk(TIR) is set very low as 1*10-7, so the integrity is high as (1 - 1*10-7) with high light at table 3.7.2.4.1 in ICAO - ANNEX 10 VOL I [5] below.

The Protect Level(PL) is a statistic value calculated by target integrity risk from probability density function which could be taken as a protected route under which the aircraft is safety protected, this is the blue line in figure 3. Its value depends on the TIR calculated by the integrity system.

The Alert Limit(AL) is an absolute value given by application which could be taken as a boundary route that aircraft will be in danger if exceeds the boundary, indicated as an orange line in figure 3. It contains a horizontal alert limit and vertical alert limit which are defined in the table as well. The horizontal alert limit when EN-route is 7.4km when flying on the ocean and 40m when flying on land using APV.The vertical alert limit is not defined when En-route flying and it is 20.0m when in APV-II high light in the table.

The Time to Alert(TTA) is the maximum period when the aircraft first detects its route exceeds the AL until the time it alerts. In the table, it is 5min if the aircraft is in En-route condition like flying on the ocean. It is critical at 6s when the aircraft flying on land[5].

Observation 1: Alert Limit, Integrity Risk, Protection Level and Time to Alert are KPIs used for the GNSS system in aviation.

Observation 2: the value of Alert Limit, Integrity Risk, Protection Level, and Time to Alert are KPIs is different for the different operation in aviation.

## Use cases and KPIs for NR

Centimeter-level positioning is introduced in NR. And integrity is becoming more and more important nowadays, for example, in many application areas, integrity is one of the key requirements and even defined as the industrial standards. Therefore, for commercial use cases requirement in NR, integrity needs to be introduced.

But for the NR system, there are many use cases and different from aviation use cases. In this section, we should identify which use cases in NR is needed to introduce integrity and those KPIs.

The use cases of positioning are defined at TS22.872[2] as below

Table 6.1-1 – Use cases synthesis

| Use cases |  | Potential requirements per use cases |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Environment of Use | Position Accuracy | Velocity | Avail. | Update rate or interval | TTFF(Time to First Fix) | Latency | Other KPI |
| 5.2.1 | Bike sharing | 5G positioning service area - Outdoor | 2m Horizontal |  | 90 % |  | 10s | 1s |  |
|  |  | Enhanced positioning area - Outdoor | 0.2m Horizontal |  | 99 % |  | 10s | 1s |  |
| 5.2.2 | Augmented Reality | Outdoor - 5G positioning service area | 1-3m Horizontal 0.1-3m Vertical | 2 m/s 10deg. | 80 % | 1 - 10 Hz | 10s | 1s | Low Energy |
| 5.2.3 | Wearables | 5G positioning service area - -Outdoor/Indoor | 2m Horizontal 1-3m Vertical |  | 90 % | 30s - 300s | 10s |  | Power saving mode |
|  |  | 5G positioning service area - -Outdoor/Indoor | 2m Horizontal 1-3m Vertical |  | 99 % | 1s - 30s | 10s | 1s | Normal mode |
| 5.2.4 | Advertisement push | 5G positioning service area - -Outdoor/Indoor | 3m Horizontal 3m Vertical |  | 90 % |  |  | 60s |  |
| 5.2.5 | Flow management | Enhanced positioning- Outdoor/Indoor | 10m Horizontal |  | 80 % | 10s | 10s |  |  |
| 5.3.1 | Person and medical equipment location in Hospital | Enhanced positioning- Outdoor/Indoor | 3m Horizontal 2m Vertical |  | 99 % |  |  | 60s |  |
| 5.3.2 | Patient location (outside Hospital) | 5G positioning service area Outdoor/Indoor | 10m Horizontal 3m Vertical (floor) |  | 99 % |  |  |  |  |
| 5.3.3 | Trolley | Enhanced positioning- Outdoor/Indoor | 0.5m Horizontal 1-3m Vertical |  | 99 % |  |  | 20ms |  |
| 5.3.4 | Waste management | 5G positioning service area - Outdoor | 3m Horizontal |  | 99 % | 2h - 1 day |  | 60s | Very low energy (15 years) |
| 5.4.1 | Emergency call | 5G positioning service area Outdoor/Indoor | 50m Horizontal 3m Vertical |  | 95 % |  | 30s | 60s | Reliability/ Confidence |
| 5.4.2 | Accurate Positioning for First Responders | Outdoor | 1m Horizontal, 0.3 m Vertical |  | 98 % |  | 10s | 5s | MCX Confidence Event-triggered report |
|  |  | Indoor | 1m Horizontal, 2 m Vertical |  | 95 % |  | 10s | 1s |  |
| 5.4.3 | Alerting nearby emergency responders | 5G positioning service area Outdoor/Indoor | 50m Horizontal 3m Vertical (floor) |  | 99% |  | 10s |  | Privacy, scalability, cross operator |
| 5.4.4 | Emergency equipment loc. outside hospitals | 5G positioning service area Outdoor/Indoor | 10m Horizontal 3m Vertical (floor) |  | 95% |  | 10s |  | Extended sleep periods |
| 5.5.1 | Traffic Monitoring & Control | 5G positioning service area - Outdoor | 1-3m Horizontal 2.5m Vertical |  | 95 % | 10 Hz | 10s | 30ms | Antispoofing Antitampering |
| 5.5.2 | Road User Charging | 5G positioning service area - Outdoor Enhanced positioning-Tunnels | <1m (across track) 3m (along track) | 2 m/s | 99 % | 1 Hz | 10s |  | Antispoofing Antitampering |
| 5.6.1 | Asset tracking and management | 5G positioning service area - Outdoor | 10-30m Horizontal | 5 m/s | 99 % | 300s-1day |  |  | 20 mJ/fix (average), Antispoofing, Antitampering, support for "out of coverage" positioning |
|  |  | Enhanced positioning - Outdoor | 1m Horizontal |  | 99 % | 1s | 1s in enhanced positioning area |  |  |
| 5.7.1 | UAV (Data analysis) | 5G positioning service area - Outdoor | 0.1m Horizontal 0.1m Vertical | 0.5 m/s 2 deg. | 99 % |  | 10s |  | Low Energy, Antispoofing, Antitampering |
| 5.7.2 | UAV (Remote control) | 5G positioning service area - Outdoor | 0.5m Horizontal 0.3m Vertical |  | 99 % |  |  | 150ms | Antispoofing Antitampering |
|  |  | Enhanced positioning area - Outdoor | 0.5m Horizontal 0.1m Vertical |  | 99.9 % |  |  | 150ms | Antispoofing Antitampering |
| 5.8.1 | Support multiple different location service | 5G positioning service area - Outdoor | 2m Horizontal |  | 90 % |  | 10s | 1s | Management of different KPI and positioning services |
|  |  | Enhanced positioning area - Indoor | 0.1m Horizontal |  | 99 % |  | 10s | 1s |  |
| 5.8.2 | Support location method negotiation | 5G positioning service area Outdoor/Indoor |  |  |  |  |  |  | Support + negotiation of positioning methods (incl. hybrid) |
| Note: most use cases also feature potential requirements on modes of operation, intended for the UE, the Network or for the 5G system. |  |  |  |  |  |  |  |  |  |

Time to First Fix (TTFF) describes the time and process required for a GPS device to acquire enough satellite signals to provide accurate navigational information. The word "fix" here means "position."

Before analyzing, it is worth noted that the avail (80% to 99%) is not the integrity and 20%(1-80%) to 1%(1-99%) is not the TIR in the table. On the same hand, the accuracy is not the alert limit. To avoiding misunderstanding, let’s explain accuracy and integrity.

The integrity in the aviation field is defined from 99.999% to 99.9999999%. The intention behind this is to keep the probability of hazardous situations extremely low, which means the plane always flies in the route(within PL or AL scope). It will warn if it exceeds the boundary. And the probability of warning is very low, such as 1-99.9999999% to 1- 99.999%. While accuracy and availability are used to illustrate the one-shot positioning result. For example, for 5.4.1 Emergency call in this table, the availability is 95%, and accuracy is 50m, it represents that the positioning system output a positioning result (such as (22°34' 14''N, 114° 3' 18''E)), and the probability is 95% that the error of the real location and (22°34' 14''N, 114° 3' 18''E) is less than 50 m.

One example of the NR positioning use case introduces the integrity KPIs and how to do these integrity KPIs work is 5.5.1 traffic control:

Figure 4 integrity example of traffic monitoring & control

The accuracy is 1-3m in this case. For fault-tolerance purposes, the alert limit can’t be set at exactly 3m, otherwise, 5% dangerous of risk is out of control. One of the purposes of introducing integrity is to improve confidence in the estimated position[6]. If we move the tolerance accuracy to 4m, we may find that 99.999% of the position falls into the [ -4, 4 ] area. If we move it to 5m, the integrity may be higher at 99.99999%. But higher is not better. High integrity also means high false alarm described in session 2.1 in this document (Ex<AL<PL).

Figure 5 false alarm

In this example, if the integrity is 99.999%, so the target integrity risk is 0.001%, by a large amount of data analysis we could calculate the PL is 4m. Protect Level(PL) value depends on the integrity requirement and the methodology we do positioning.

If the traffic Monitoring & control system requires AL as 5m:

        - AL>PL, the integrity system is available.

If the traffic Monitoring system requires AL as 3m:

        - AL<PL, the integrity system is not available.

According to the above cases, we have found that the value of PL needs a large amount of data to confirm and to get these redundancy data, the user may need to follow some regular route to move. Currently, integrity is used at Safety-of-Life (SoL) system and Automotive Safety Integrity Levels (ASILs) systems that are for aircraft, road, highway, and rail, the common ground are they all follow some regular route and possible to collect redundancy data.

Base on the analysis above, then we combine the use cases in table 6.1-1 in TS22.872[2] and IIoT positioning Service Performance Requirements in 8.1.7 of TS 22.804[7], list the use cases that adopt to introduce integrity.

Table 2.2-1

| Use case | Regular route | Date collection | Integrity Adaptable |
| --- | --- | --- | --- |
| Bike sharing | Fixed parking area | Yes | only adopt to parking operation |
| Augmented Reality | NO | NO | NO |
| Wearables | NO | NO | NO |
| Advertisement push | NO | NO | NO |
| Flow management | NO | NO | NO |
| Person and medical equipment location in Hospital | NO | NO | NO |
| Patient location (outside Hospital) | NO | NO | NO |
| Trolley | YES | YES | YES |
| Waste management | YES | YES | YES |
| Emergency call | NO | NO | NO |
| Accurate Positioning for First Responders | NO | NO | NO |
| Alerting nearby emergency responders | NO | NO | NO |
| Emergency equipment loc. outside hospitals | YES | YES | YES |
| Traffic Monitoring & Control | YES | YES | YES |
| Road User Charging | YES | YES | YES |
| Asset tracking and management (out of coverage" positioning) | YES | YES | YES |
| UAV (Data analysis) | part operation | YES | Part operation |
| UAV (Remote control) | part operation | YES | Part operation |
| Mobile control panels with safety functions in smart factories (within factory danger zones) | YES | YES | YES |
| Mobile control panels with safety functions ( non-danger zones | YES | YES | YES |
| Augmented reality in smart factories | NO | NO | NO |
| Process automation – plant asset management | YES | YES | YES |
| Inbound logistics for manufacturing (for driving trajectories (if supported by further sensors like camera, GNSS, IMU) of autonomous driving systems) ) | YES | YES | YES |
| Inbound logistics for manufacturing (for storage of goods) | YES | YES | YES |
| Flexible, modular assembly area in smart factories (for autonomous vehicles (only for monitoring proposes)) | YES | YES | YES |
| Flexible, modular assembly area in smart factories (for tracking of tools at the work-place location) | YES | YES | YES |

From the above analysis, we observe that almost all the use cases in IIOT can apply integrity. They have the same characteristic:

- follow a regular route or a fixed area(in a factory, on the assembly line, in a warehouse)

- high accuracy requirement

- possible to collect analysis data by factory automatic system.

Only part of the positioning use cases can apply integrity:

- Trolley

- Traffic Monitoring & Control

- Road User Charging

- UAV

- Out of coverage positioning

They are all following the same rule as above. All the KPIs in session 2.1 are adaptable to the above use cases. Thereafter, IIOT, V2X, automation, and related navigation use cases should be treated priority.

Observation 3: Integrity is adaptable to almost all the use cases in IIOT.

Proposal 1: Integrity is adaptable to the use case which follows the regular routes and is possible to collect user data.

Proposal 2: Alert Limit, Integrity Risk, Protection Level, and Time to Alert should be introduced as integrity KPIs for NR positioning.

Proposal 3: IIOT, V2X, automation-related navigation use cases should be treated priority.

- Conclusion

Based on the discussion, we observe and propose the following:

Observation 1: Alert Limit, Integrity Risk, Protection Level and Time to Alert are KPIs used for the GNSS system in aviation.

Observation 2: Integrity needs to be introduced for NR positioning, but only for use cases follow regular routes, and the routes data can be collected.

Proposal 1: Integrity is adaptable to the use case which follows the regular routes and is possible to collect user data.

Proposal 2: Alert Limit, Integrity Risk, Protection Level, and Time to Alert should be introduced as integrity KPIs for NR positioning.

Proposal 3: IIOT, V2X, automation-related navigation use cases should be treated priority.

- Reference

- RP-200928-(rev.fr.RP-193237)-clean

- TS22.872 “Study on positioning use cases” V16.1.0,2018-09

- GNSS Position Integrity in Urban Environments: A Review of Literature

- https://en.wikipedia.org/wiki/Probability_density_function

- ICAO - ANNEX 10 VOL I. Aeronautical Telecommunications - Volume I Radio Navigation Aids

- R2-2006541 TP for Study on Positioning Integrity and Reliability

- TS22.804 “Study on Communication for Automation in Vertical Domains”, V16.3.0,2020-07

- REPORT ON THE PERFORMANCE AND LEVEL OF INTEGRITY FOR SAFETY AND LIABILITY CRITICAL MULTI-APPLICATIONS\

- https://en.wikipedia.org/wiki/Chebyshev%27s_inequality

- RP-200797 Support of NR Industrial Internet of Things (IoT)

- The Stanford – ESA Integrity Diagram: Focusing on SBAS Integrity

- RP-190855 Motivation for SI on Integrity support for NR Positioning

- The SBAS Integrity Concept Standardised by ICAO: Application to EGNOS