3GPP TSG-RAN WG2 Meeting #111-e	R2-2006580

Electronic, 17th – 28th Aug, 2020

Agenda Item:	8.11.3.2

Source:	Huawei, HiSilicon

Title:	Discussion on positioning integrity validation and reporting

Document for: Discussion

Introduction

The NR positioning study item for Rel-17 in [1] was agreed in RANP#86. The objective of the SI for RAN2 on the integrity and reliability issue includes the following:

2. Study solutions necessary to support integrity and reliability of assistance data and position information: [RAN2]

- Identify positioning integrity KPIs and relevant use cases.

- Identify the error sources, threat models, occurrence rates and failure modes requiring positioning integrity validation and reporting.

- Study methodologies for network-assisted and UE-assisted integrity.

NOTE 4: Objective 2 is applicable to both, RAT-dependent and RAT-independent positioning methods.

According to the definition of positioning integrity in TS 22.872 [2] and some related academic papers [3][4], integrity can be interpreted from two aspects:

1) Integrity is a measure of the trust in the accuracy of the position-related data.

2) Integrity also includes the ability of the positioning system to provide timely warnings in the case of unacceptable accuracy of the delivered performance indicators.

For the first aspect (i.e. item 2a), the KPIs together with relevant use cases are captured in another discussion paper from us [5]. This contribution mainly focuses on the second aspect (i.e. item 2b), i.e., integrity validation and timely & valid reporting in the case of unfulfilled integrity requirements. First, we introduce serval factors of positioning integrity validation, including the error sources, threat models, occurrence rates and failure modes. Then we provide our views on the positioning integrity reporting.

Backgrounds and State-of-art

Integrity KPIs

As discussed in our another discussion paper [5], the key positioning integrity KPIs include Alert Limit (AL), Time to Alert (TTA), Integrity Risk (IR) and Protection Level (PL). The definition and explanation for these KPIs are summarized in Table 2.1- 1.

- Table 2.1- 1 Positioning Integrity KPIs

| KPI | Definition | Calculation (if any) |
| --- | --- | --- |
| Alert Limit (AL) | The alert limit is the maximum allowable position error beyond which the system should be declared unavailable for the intended application. | Service-specific and prescribed |
| Time to Alert (TTA) | The maximum allowable elapsed time from the onset of a positioning failure until the equipment announces the alert. | Service-specific and prescribed |
| Integrity Risk (IR) | The probability of providing a signal that is out of tolerance without warning the user in a given period of time. | Service-specific and prescribed |
| Protection Level (PL) | A statistical error bound computed so as to guarantee that the probability of the absolute position error exceeding the said number is smaller than or equal to the target integrity risk. | Obtained with the methods (i.e. calculation, derivation, statistic) varies from different assumptions or applications [6]. |

Integrity Failure

An integrity failure is an integrity event that lasts for longer than the TTA and with no alarm raised within the TTA. There are two typical integrity events, namely, misleading information (MI) events and hazardously misleading information (HMI) events:

- MI events: Misleading information events occurs when being the system declared available, the position error (PE) exceeds the protection level but not the alert limit, i.e. PL < PE < AL.

- HMI events: Hazardously misleading information events occurs when, being the system declared available, the position error exceeds the alert limit, i.e. PL < AL < PE.

To explain the concepts of different statistical bounds (i.e. AL, PL) and their relations with position error, the Stanford diagram is widely considered as a good illustrative tool, which is presented in Figure 2.2- 1.

- Figure 2.2- 1 Illustration of Stanford diagram

As shown in Figure 2.2- 1, we can have several observations:

- The diagonal axis separates those samples in which the PE is covered by the PL (i.e. PE < PL), above the diagonal, from those, below the diagonal, in which the PL fails to cover the PE (i.e. PE > PL). The proximity of the cloud of sample points to the diagonal gives an idea of the achieved level of safety, as any point above the diagonal but very close to it indicates that an integrity event (MI/HMI events) was close to occur.

- System Availability: Taking the maximum allowable error bounding (i.e. AL) into account, all the sample points can be categorized into two operation modes: system unavailable (i.e. PL > AL, the top half ) and system available (i.e. PL < AL, the bottom half ).

- System Operations under available system: Considering the numerical relationship between PE and PL/AL, the “system available” mode can be subdivided into nominal, misleading and hazardously misleading operations. This also applies to the “system unavailable” mode.

Considering different statistical bounds of position error, i.e. AL and PL, we have the following cases in total:

- System unavailable mode: PL > AL;

- System available mode: PL < AL or PL = AL

  - Nominal operations: PE < PL < AL;

  - Misleading operations: PL < PE < AL;

  - Hazardously misleading: PL < AL < PE.

Since the position error is not observable, the positioning system only has real-time access to the PL, so the decision of alert is done by comparing the prescribed AL and the PL (obtained by calculation, derivation, statistic, etc.), more precisely:

- If PL > AL, the alert is triggered;

- If PL < AL or PL = AL, the alert is not triggered.

Integrity Validation and Reporting

For now, there has been a large amount of literature providing a quite mature and sound study of positioning integrity for GNSS aviation, while little study can be found for other positioning methods. Therefore, the study of integrity validation and reporting for general positioning systems can refer to the corresponding study of GNSS aviation systems.

As illustrated in [7], the integrity validation and reporting mainly includes two parts. First, the PL is calculated by the detection and identification of error sources, to determine whether to trigger an alert or not. Second, if an alert is triggered (e.g. when PL > AL), error detection and determination with the potential cause of positioning errors may be initiated, with the help of threat models and failure modes. An illustrative example is given below.

- The detection and identification of error sources to derive PL

Step 1: Define equivalent user conditions to replicate the conditions and environment for the users.

- e.g., exploiting prior knowledge of the error sources.

Step 2: Determine the error distributions by error modeling.

Step 3: Determine PL.

Examine a very large number (approaching infinity) of hypothetical users, and calculate the prob. of exceeding the PL for each ensemble (a collection of users), in order to derive PL.

- The integrity requirement should be met for the most difficult case.

- Mind the dependencies between error sources.

A key step for integrity validation and reporting is to have real-time access to the PL. The PL can be obtained by different methods (i.e. calculation, derivation, and statistic) depending on different assumptions or applications.

- The identification of threat models

The threat models must describe all known conditions that could cause the true error to exceed the predicted confidence bounds. Having a comprehensive list of threat models is essential to achieving the required level of safety. According to [7], threat models can be defined as:

“Anticipated events that the system must protect the user against, and conditions during which it must provide reliably safe confidence bounds”.

The corresponding requirements are illustrated as follows:

    - Each threat model must describe the specific nature of the threat, its magnitude, and its likelihood (i.e., occurrence rate).

    - The threat models must be comprehensive in describing all reasonable conditions.

    - Restricting the scope of the threats is necessary for creating a practical system since it is not possible to create a system that can protect against every conceivable threat.

- The identification of failure modes

An error is the variation or deviation from the original value and becomes a failure when it crosses a designated limit, while a failure mode is an error that has the potential to become a failure (PL>AL). According to [8], failure mode can be defined as:

“The specific manner or way by which a failure occurs in terms of failure of the part, component, function, equipment, subsystem, or system under investigation”.

For the failure modes of positioning integrity, the “failure” refers to “integrity failure”. An example of failure modes for GPS system [9] is given in Table 2.3- 1, which may be helpful to extend to general positioning systems.

Table 2.3- 1 GPS Failure modes [9]

|  | Explanation | Causes (Error Sources) | Failure modes |
| --- | --- | --- | --- |
| System level | System level failures are those that occur within the space segment, the control segment, and the interface between the two (i.e. data transmission). | E.g., Weakness in satellite design and algorithms within the Master Control Station (MCS) environment | 1) Erroneous clock behavior; 2) Incorrect modelling and malfunction of the MCS; 3) Satellite payload performance failures; 4) Failures related to satellite orbits; 5) Space vehicle system related performance failures; 6) RF related performance failures |
| Operational environment | These failures are mainly due to interference (intended and unintended) and the effects of the media along the signal path. | The primary signal characteristic that makes GPS vulnerable to interference is the low power of the signal. | 1) Intended interference: jamming, spoofing 2) Unintended RF interference; 3) Sudden changes in the signal propagation properties: - The ionosphere surrounding the Earth refracts radio signals; - The troposphere has the effect of bending and refracting (delaying) the navigation signal. - Multipath errors result from refection of the navigation signal off surfaces. |
| User receiver | These failures relate to the end user and the end-user equipment, i.e., receiver and receiver software. | Failures related to humans include the lack of adequate training, over-reliance on a single navigation system etc. | 1) Receiver/user related performance failures: - Power system failure or power fluctuations; - Software incompatibilities (year/week rollovers); - Receiver unit overheating; - Instabilities in the quartz frequency standards; - Receiver interface outages; - Receiver outages related to excessive electromagnetic activities (lightning etc.); - Hardware incompatibilities if the GPS unit is coupled with other means of navigation (i.e. INS, compasses, external clocks, air data, navigation data bases etc.); - Processing algorithm errors; - GPS receivers comprise complex hardware and software which are vulnerable to failure; - Hard-wired and incorrect RAIM parameters have been used in certified receivers. 2) Human related failures. |

Based on the above analysis, the error sources have a great influence on the identification and specification of threat models and failure modes. So we have the following observation:

Identification of error sources can be considered as a starting point of the study of integrity validation and reporting.

Discussion

According to the backgrounds and state-of-art, integrity validation and reporting is performed on the premise of comprehensive error analysis. The detection and identification of error sources may involve dividing the positioning process into different functional parts and analyzing which part the error may come from. Considering different positioning methods and scenarios, the error sources may be different. As a starting point, the analysis of error sources should be conducted separately for RAT-dependent and RAT-independent positioning systems considering the huge difference between cellular networks and satellite networks. So we have the following proposal.

Study the potential error sources for each positioning method of RAT-dependent and RAT-independent positioning separately.

Error Source

3.1.1 RAT-dependent Positioning

For RAT-dependent positioning, the error sources may vary for different positioning methods, including timing-based positioning methods (e.g. DL-TDOA, UL-TDOA, Multi-RTT), angle-based positioning methods (e.g. DL-AoD, UL-AoA), etc.

  - For timing based positioning, the positioning error may be caused by [10]

    - timing measurement error (TOA measurement error, reporting delay etc.),

    - UE clock drifting within and across PRS occasions,

    - reference station/TRP synchronization (RTD error, RTD drift, etc.),

    - radio environment (NLOS delay bias, multipath, etc.),

    - measurement geometry (GDOP, etc.),

    - cell data base accuracy (TRP coordinates, antenna height), etc..

  - For angle measurements based positioning, the positioning error may be caused by

    - angle measurement error

    - gNB antenna calibration (phase distortion among antenna elements, conformity of antenna radiation pattern, etc.),

    - radio environment (NLOS angle spread, etc.),

    - measurement geometry,

    - cell data base accuracy, etc.

The error sources for RAT-dependent positioning can be studied separately for timing-based positioning methods (e.g. DL-TDOA, UL-TDOA, Multi-RTT), and angle-based positioning methods (e.g. DL-AoD, UL-AoA).

For timing based positioning, the error sources may include time measurement error, UE clock drifting within and across PRS occasions, reference station/TRP synchronization, radio environment, measurement geometry, cell data base accuracy, etc.

For angle measurements based positioning, the error sources may include angle measurement error, gNB antenna calibration, radio environment, measurement geometry, cell data base accuracy, etc.

LS to RAN1 to study the error sources that influence the accuracy of RAT-dependent positioning methods.

3.1.2 RAT-independent Positioning

For RAT-independent positioning (e.g. A-GNSS Positioning), the ranging error may result from satellite clock error, satellite ephemeris error, ionospheric error, tropospheric delay, multipath, receiver noise, satellite geometry (GDOP), etc. [9][11][12]. The reason why these are considered as the error sources are as follows:

- Satellite clock error: Due to the satellite motion and earth rotation, satellite clock jumps would lead to excessive pseudo-range deviation.

- Ionospheric error: The ionosphere surrounding the Earth refracts radio signals in the L1, L2 and the proposed L5 band. Therefore, small-scale (spatial and temporal) electron density fluctuations especially in periods of high solar activity may affect the GPS signals significantly causing non-integrity or non-availability situations.

- Tropospheric delay: The troposphere has the effect of bending and refracting (delaying) the navigation signal. The delay due to the troposphere can cause huge ranging error, e.g. 2 to 25m.

- Satellite ephemeris error: The ephemeris of a satellite is a mathematical description of its orbit. The high precision satellite orbital data is necessary for a receiver to calculate the satellite’s exact position in space at any given time.

- Multipath error: Resulting from reflection of the navigation signal off surfaces, multipath errors would disturb the code and carrier tracking loop.

- Receiver noise: The receiver noise (a.k.a. random measurement noise) would affect the code and carrier measurements.

- Satellite geometry: GDOP (Geometry Dilution of Precision) is used to assess the impact of satellite geometry on the final accuracy, under the premise of measurement error.

Based on the above analysis and discussion, we have the following proposals concerning the study of error sources for RAT-independent positioning systems.

The error sources for RAT-independent positioning may include satellite clock error, satellite ephemeris error, ionospheric error, tropospheric delay, multipath, receiver noise, satellite geometry (GDOP), etc.

LS to RAN1 to study the error sources of RAT-Independent positioning methods.

Threat Models and Failure Modes

According to Observation 2, the error sources should be identified first and then progress on the threat models and failure modes. Based on the discussion in Section 3.1, the potential error sources for each positioning method are different due to the different positioning mechanisms. To precisely describe a whole picture of threat models and failure modes, the characteristics of error sources for each positioning methods should be studied separately.

For threat model, study the characteristics of each error source, e.g. the occurrence rate and magnitude in the threat models for each positioning methods (for both RAT-dependent and RAT-independent positioning).

Failure modes can be studied in terms of different parts or components, e.g. system level, operational environment, and user receiver.

Identify the failure modes of each positioning methods (for both RAT-dependent and RAT-independent positioning).

Conclusion

In this contribution, we first provide an introduction of positioning integrity validation, including the error sources, threat models, occurrence rates and failure modes. Then we discuss several issues related to the positioning integrity reporting for RAT-dependent and RAT-independent positioning, respectively. Based on our analysis and discussion, we have the following observations and proposals.

- A key step for integrity validation and reporting is to have real-time access to the PL. The PL can be obtained by different methods (i.e. calculation, derivation, and statistic) depending on different assumptions or applications.

- Identification of error sources can be considered as a starting point of integrity validation and reporting.

- Study the potential error sources for each positioning method of RAT-dependent and RAT-independent positioning separately.

- The error sources for RAT-dependent positioning can be studied separately for timing-based positioning methods (e.g. DL-TDOA, UL-TDOA, Multi-RTT), and angle-based positioning methods (e.g. DL-AoD, UL-AoA).

- For timing based positioning, the error sources may include time measurement error, UE clock drifting within and across PRS occasions, reference station/TRP synchronization, radio environment, measurement geometry, cell data base accuracy, etc.

- For angle measurements based positioning, the error sources may include angle measurement error, gNB antenna calibration, radio environment, measurement geometry, cell data base accuracy, etc.

- LS to RAN1 to study the error sources that influence the accuracy of RAT-dependent positioning methods.

- The error sources for RAT-independent positioning may include satellite clock error, satellite ephemeris error, ionospheric error, tropospheric delay, multipath, receiver noise, satellite geometry (GDOP), etc.

- LS to RAN1 to study the error sources of RAT-Independent positioning methods.

- Study the characteristics of each error source, e.g. the occurrence rate and magnitude in the threat models for each positioning methods (for both RAT-dependent and RAT-independent positioning).

- Failure modes can be studied in terms of different parts or components, e.g. system level, operational environment, and user receiver.

- Identify the failure modes of each positioning methods (for both RAT-dependent and RAT-independent positioning).

- References

- 3GPP RP-193237, “New SID on NR Positioning Enhancements,” Qualcomm, RAN#86, Sitges, Spain, Dec. 9-12, 2019.

- 3GPP TR 22.872, “Study on positioning use cases”, Stage 1 (Release 16), V16.1.0, August 2018.

- R.J. Kelly, and J.M. Davis, “Required Navigation Performance (RNP) for Precision Approach and Landing with GNSS Application”, Journal of the Institute of Navigation, Vol. 41, No. 1, pp. 1-30, 1994.

- P. B. Ober, Integrity prediction and monitoring of navigation systems. Integricom Publishers Leiden, 2003, vol. 1.

- R2-2006579, “Discussion on positioning integrity KPIs and relevant use cases”, RAN2#111-e, August 17- 28, 2020.

- N. Zhu, J. Marais, D. Bétaille and M. Berbineau, “GNSS Position Integrity in Urban Environments: A Review of Literature, ” in IEEE Transactions on Intelligent Transportation Systems, vol. 19, no. 9, pp. 2762-2778, Sept. 2018.

- T. Walter, P. Enge, and B. DeCleene, “Integrity Lessons from the WAAS Integrity Performance Panel (WIPP),” in Proceedings of the 2003 National Technical Meeting of The Institute of Navigation, Anaheim, CA, 2003, pp. 183–194.

- Langford, J. W. (1995). Logistics: Principles and Applications. McGraw Hill. p. 488.

- Washington Y. Ochieng and Knut Sauer, “GPS Integrity and Potential Impact on Aviation Safety”, The Journal of Navigation, 2003, 51-65.

- Sven Fischer, “Observed Time Difference of Arrival (OTDOA) Positioning in 3GPP LTE”, June 6, 2014.

- 3GPP TS 37.355, “LTE Positioning Protocol (LPP)”, Release 16, V16.1.0, July 2020.

- Pratap Misra and Per Enge, “Global Positioning System: Signals, Measurements, and Performance”, Second Edition, 2011.