3GPP TSG-RAN WG2 Meeting #111 electronic	R2-2006595

Online, August 17th - 28th, 2020

Source:	CATT

Title:	Discussion on Mobility with Service Continuity in RRC_CONNECTED

Agenda Item:	8.1.2.2

Document for:	Discussion and Decision

Introduction

One of the objectives of WI [1] is to specify support for basic mobility with service continuity.

Mobility with the Service continuity for MBS includes mobility of UE in RRC_CONNECTED state and mobility of UE in RRC_IDLE/RRC_INACTIVE state. We focus on UE in RRC_CONNECTED state in this contribution, the issues for UE in RRC_IDLE/RRC_INACTIVE state will be addressed in another contribution [7].

In this contribution, we would like to discuss the key issues in different mobility scenarios for UE in RRC_CONNECTED state, and then we also propose some possible solutions for the identified issues.

Discussion

  - Scenarios of mobility with service continuity

According to [2], the following service continuity scenarios were identified with highest priority for MBMS services over SC-PTM,

Scenario 1: Moving from SC-PTM to SC-PTM For users receiving a group call over SC-PTM in the source cell, the target cell provides the same group call over SC-PTM. Scenario 2: Moving from SC-PTM to unicast For users receiving a group call over SC-PTM in the source cell, the target cell provides the same group call over unicast, because the service is not provided in the target cell using SC-PTM.

Service continuity scenarios for MBS in NR are similar as that in SC-PTM.

UE could move from source cell (supporting the MBS service or not supporting the MBS service) to target cell (supporting the MBS service or not supporting the MBS service), these possible scenarios are depicted in the Table 1 below.

| Target cell Source cell | Cell Supporting the MBS service | Cell Not supporting the MBS service |
| --- | --- | --- |
| Cell supporting the MBS service | Scenarios 1: Mobility from source cell supporting MBS to target cell supporting MBS For UE receiving a specific MBS service in the source cell, the same MBS service is provided in target cell. Mechanism for Service continuity should be considered for this scenario 1. | Scenarios 2: Mobility from source cell supporting the MBS service to target cell not supporting the MBS service For UE receiving a specific MBS service in the source cell, the same MBS service is not provided in target cell. UE need unicast PDU session to continue the reception of MBS in target cell. Mechanism for Service continuity should be considered for scenario 2. Notes: According to [4], it has been clarified in clause 5.8 that service continuity for broadcast service when the UE moves from a NG-RAN node that supports MBS to a NG-RAN node that does not support MBS will not be addressed within Rel-17 timeframe |
| Cell not supporting the MBS service | Scenarios 3: Mobility from source cell not supporting the MBS service to target cell supporting the MBS service For this scenario, legacy handover procedure applies during the mobility of UE in RRC_CONNECTED. Then UE applies the normal MBS session join procedure to receive the MBS service through MBS session after handover to the target cell. No special solution is needed to ensure the service continuity for scenario 3. | Scenarios 4: Mobility from source cell not supporting the MBS service to target cell not supporting the MBS service For this scenario, UE receives MBS traffic data on unicast PDU session both in source cell and target cell. Legacy handover will apply during the mobility of UE in RRC_CONNECTED. The service continuity for this scenario could be ensured by the legacy handover procedure completely. No special solution is needed to ensure the service continuity for scenario 4. |

Table 1 Scenarios for mobility with service continuity

Observation 1: Solution for service continuity is need for,

Scenarios 1: Mobility between cells supporting the MBS service

Scenarios 2: Mobility from source cell supporting the MBS service to target cell not supporting the MBS service.

Proposal 1:  Consider the solutions for Service continuity for mobility between cells supporting the MBS service and mobility from source cell supporting the MBS service to target cell not supporting the MBS service.

  - Mobility between cells supporting the MBS service

The general procedure for UE Mobility from cell supporting the MBS service to cell not supporting the MBS service in RRC_CONNECTED state is shown as below based on the understanding to the mechanism in SC-PTM and solutions agreed in SA2 SI for MBS as captured in [4],

Figure 1 Mobility from cell supporting MBS service to cell supporting MBS service

Issue 1: Necessity of reporting interested MBS services by UE in RRC_CONNECTED state (like in SC-PTM) to NG-RAN so that the NG-RAN can assist the UE to stay or get on the right carrier frequency during handover.

In SC-PTM, UEs in RRC_CONNECTED state will send the MBMSInterestIndication message so that the source cell can assist the UE to stay or get on the right carrier frequency during handover.

To facilitate the NG-RAN choose a target cell which support the MBS service UE receiving, There should be a way for source cell to be aware of the MBS service UE receiving. It could be acquired from CN, or from the counting mechanism we have discussed in [6],or  by reusing the interest indication mechanism in SC-PTM，as shown in step 1 in Figure 1 above.

Proposal 2: Discuss the necessity of reporting interested MBS services by UE in RRC_CONNECTED state to NG-RAN so that the NG-RAN can assist the UE to stay on the cell supporting the MBS service during handover.

Issue 2: Potential RAN2 impact on mobility with service continuity with the assumption that N3 multicast tunnel could be established on demand in MBS capable target cell.

As discussed in [6], MBS capable NG-RAN node can request the establishment of the N3 multicast tunnel according to the solutions agreed in SA2 SI for MBS which are captured in [4].

If MBS session could be established on demand in NG-RAN, NG-RAN/5GC can request N3 multicast tunnel establishment or MBS session resource setup towards the target RAN (see solution 27, 11/12, etc.) dynamically, as shown in step 6 in Figure 1above, thereby enabling multicast transport. With this mechanism, RAN3 impact such as impact to NG, Xn interface could be expected. Though we may also need to investigate the potential impact to RAN2.

Proposal 3: Discuss the potential RAN2 impact on mobility with service continuity based on the assumption that N3 multicast tunnel could be established on demand in MBS capable target cell.

Issue 3: Method to deliver the MBS bearer configuration of the target cell to UE during handover.

Another issue to be addressed it the method to deliver the MBS bearer configuration of the target cell to UE.

The MBS bear configuration are always get from SC-MCCH in SC-PTM. It is a simple way but may cause service interruption.

In SC-PTM, there were discussions that SC-PTM control info of the target cell could be provided to the UE by handover command to minimize the service interruption time.

For MBS in NR, we should reconsider whether to deliver the MBS bearer configuration to the UE by RRC signaling due to the new requirement and new architecture.

Proposal 3:RAN2 to discuss the method of delivering the MBS bearer configuration of the target cell to UE.

Issue 4: Handling of packet loss during Mobility between cells supporting MBS.

As there could be concern on packet lossless delivery of the MBS services during Mobility between cells supporting MBS. We would like to discuss the cause of packet loss during handover firstly.

The cause for Packet loss could be considered from two perspectives:

1. Packet missed by UE due to service interruption during UE handover

As there is interruption time of MBS data reception when UE handovers from source cell to target cell, Some MBS packet may be missed by UE.

2. Unsynchronized scheduling of the same MBS service on different cells

For the Packet loss caused by unsynchronized scheduling of the same MBS service on different cells

With the assumption that the MBS traffic data transmission in source cell and target cell should be to some extent synchronized, i.e. sending same content of the same MBMS service at the same time as much as possible, there are still some factors which may cause the unsynchronized scheduling between cells. The relevant factors will be elaborated as follows:

(1)  Unsynchronized scheduling algorithm between source cell and target cell.

As the MBS scheduling is to be performed by each cell independently, the same service may not be transmitted over the radio link at the same time from adjacent cells. This may cause problems in enabling service continuity when the UE is moving between two adjacent cells which provide the same MBS service.

Refer to [5], there was already an evaluation and discussion on the possible packet loss due to the unsynchronized SC-PTM scheduling between two adjacent cells during the standardization of SC-PTM solution. The conclusion is that the possible data loss due to the unsynchronized SC-PTM scheduling over the radio link between two adjacent cells is not a serious problem to address, as shown in Figure 2 below. The potential packet loss could be avoided/minimized by network implementation. Eventually there is no solution specified for packet loss elimination in SC-PTM.

Figure 2 unsynchronized scheduling in SC-PTM

In case of MBS in NR, the possible packet loss due to the unsynchronized scheduling over radio link between source cell and target cell should also not be a serious problem if the similar scheduling mechanism is employed as that in SC-PTM.

(2) Unsynchronized MBS scheduling due to different retransmission scheduled on PTM resources in different cells

One of the objectives of WI [1] is to specify required changes to improve reliability of Broadcast/Multicast service, e.g. by UL feedback. Additional unsynchronized scheduling between cells could be introduced if retransmission over PTM resource in each cell occurs.

Proposal 4:  Discuss whether packet loss of MBS traffic during mobility due to different transmission progress between cells is a serious problem to address.

In case that RAN2 concludes that packet loss due to different progress in different cells is a problem to address, RAN2 needs to consider solutions to address it.

Solution 1: Synchronized delivery over the radio between cells could be provided with employing SFN operation at NG-RAN side.

As stated in [1], SFN could be implemented at network side without standardization.

SFN provides synchronized delivery of user plane packets over the air from different cells. No standardized support specifically for SFN, is provided in this WI. Any SFN operation is transparent to the UE, and any related synchronization is left to network implementation.

Solution 1 is up to network implementation, no standardization impact.

Solution 2:RAN2 avoids choosing the solution with retransmission on PTM resources.

As already analyzed, different retransmission on PTM resource in different cells could result in the different progress of the MBS data delivery in source cell and target cell. Therefore not using retransmission on PTM resource could be helpful to avoid packet loss.So RAN2 can consider not to choose the solution with retransmission on PTM resources when make the solution  to  improve reliability as required in WI [1],

- Specify required changes to improve reliability of Broadcast/Multicast service, e.g. by UL feedback. The level of reliability should be based on the requirements of the application/service provided.[RAN1, RAN2]

Solution 3: NG-RAN detects the packet loss through UL feedback from UE and uses unicast radio bearer to transmit the missed packet to UE.

With option 3, the target cell could use a unicast radio bearer for UE in RRC_CONNECTED to transmit the missed packet due to the different progress of MBS data delivery in cells. However this solution will rely on UL feedback per UE. Besides, there will a significant increase of the overhead on radio resource if the number of UEs moving between cells with different progress of MBS data delivery increases.

Proposal 5:  Discuss the possible solutions to avoid packet loss in case it is considered necessary.

  - Mobility from cell supporting the MBS service to cell not supporting the MBS service

The general procedure for Mobility from cell supporting the MBS service to cell not supporting the MBS service is shown as below based on the understanding to the mechanism in SC-PTM and solutions in agreed in SA2 SI for MBS as captured in [4],

Figure 3 Mobility from cell supporting MBS service to cell not supporting MBS service

To continue the reception of the same MBS service on the target cell not supporting MBS, UE should have a unicast bearer to carry the MBS traffic data on the target cell to ensure the service continuity.

To facilitate UE to request unicast reception of the service, UE need to be aware of whether MBS service which UE is receiving is supported on target cell. The availability in neighbor cells of each MBMS service is indicated to UE in SC-PTM. As specified in [3],

For each MBMS service provided using SC-PTM, E-UTRAN indicates in the SC-MCCH the list of neighbour cells providing this MBMS service so that the UE can request unicast reception of the service before changing to a cell not providing the MBMS service using SC-PTM.

When it comes to MBS in NR, the similar neighbor cell configuration may also need to help UE to check whether the target cell support the receiving MBS service and then whether it is necessary to request unicast PDU session upon handover to a cell not supporting the related MBS service.

Proposal 6: Discuss whether the mechanism in SC-PTM could be the baseline solution for mobility from cell supporting MBS to cell not supporting MBS.

However, according to solutions agreed in SA2 SI for MBS as captured in [4], a MBS session could be associated to a unicast PDU session as shown in above picture. If there is already a PDU session associated to MBS session when UE in RRC_CONNECTED state is receiving MBS in source cell, there will be no need for UE to establish a new PDU session upon handover to the target cell not supporting MBS. The service continuity could be achieved over the existing PDU session for MBS service reception in the target cell as shown in step 6b in Figure 3 above.

Proposal 7: Discuss the potential RAN2 impact for service continuity with mobility from cell supporting MBS to cell not supporting MBS based on the assumption that there will always be an association between MBS session and PDU session.

Conclusion

According to the analysis in section 2, we propose:

Proposal 1:  Consider the solutions for Service continuity for mobility between cells supporting the MBS service and mobility from source cell supporting the MBS service to target cell not supporting the MBS service.

Proposal 2: Discuss the necessity of reporting interested MBS services by UE in RRC_CONNECTED state to NG-RAN so that the NG-RAN can assist the UE to stay on the cell supporting the MBS service during handover.

Proposal 3: Discuss the potential RAN2 impact on mobility with service continuity based on the assumption that N3 multicast tunnel could be established on demand in MBS capable target cell.

Proposal 4:  Discuss whether packet loss of MBS traffic during mobility due to different transmission progress between cells is a serious problem to address.

Proposal 5:  Discuss the possible solutions to avoid packet loss in case it is considered necessary.

Proposal 6: Discuss whether the mechanism in SC-PTM could be the baseline solution for mobility from cell supporting MBS to cell not supporting MBS.

Proposal 7: Discuss the potential RAN2 impact for service continuity with mobility from cell supporting MBS to cell not supporting MBS based on the assumption that there will always be an association between MBS session and PDU session.

Reference

- RP-201038,WID revision: NR Multicast and Broadcast Services;Huawei, HiSilicon

- 36.890,Study on Support of single-cell point-to-multipoint transmission in LTE,v13.0.0

- 36.300, (E-UTRAN);Overall description; Stage 2，v16.2.0

- 23.757,Study on architectural enhancements for 5G multicast-broadcast services,v0.4.0

- R2-152460, Summary of email discussion [89bis#22][LTE/SC-PTM] Service Continuity for SC-PTM;Huawei (Rapporteur)

- R2-2006594, Discussion on Dynamic PTM and PTP Switch with Service Continuity;CATT

- R2-2006597, Consideration on Idle and Inactive mode UEs;CATT

- 36.331, Radio Resource Control (RRC); Protocol specification,v16.1.1