3GPP TSG-RAN WG2 Meeting #111-e	R2-2006893

# E-meeting, 17th – 28th August 2020

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 1752 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on autonomous gap related UE capabilities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips, Ericsson, Nokia, Nokia Shanghai Bell, MediaTek Inc, Qualcomm Incorporated, Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_enh-Core |  |  |  |  |  |  | Date: |  | 2020-08-06 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to Rel-16 RAN4 feature list, for autonomous gap based CGI reading capablity, RAN4 asks RAN2 to introduce UE capability for all possible scenarios. 9-6; CGI reading of an NR neighbour cell; 1) Support of autonomous gap-based CGI reading of an NR neighbour cell for EN-DC, NR SA, LTE SA, NR-DC, NE-DC. 9-7; CGI reading of an E-UTRA neighbour cell; 1) Support of autonomous gap-based CGI reading of an E-UTRA neighbour cell for EN-DC, NR SA, LTE SA, NR-DC, NE-DC. The orginal agreed CR (R2-2002147, R2-2002278) has introduced the partial capabilities for NR SA and (NG)EN-DC cases, and the latest Rel-16 version also added more capabilities for NR-DC and NE-DC cases. However, there is still some remaining issues need to be solved: - Need to clarify whether TDD/FDD or FR1/FR2 differentiation is corresponding to the duplex/FR mode of serving cell or target measured cell; (note: the previous agreed CR based on the assumption of using target measured cell) - Need to clarify whether to differentiate TDD/FDD and FR1/FR2 capabilities for each scenario; (The current spec adopts different principles for EN-DC and NE-DC/NR-DC) This CR is provided to solve above issues, and complete the Rel-16 autonomous gap capabilities. |
| Summary of change: |  | - For autonomous gap based CGI reporting capabilities that need TDD/FDD or FR1/FR2 differentiation, clarify it corresponds to the duplex mode or FR mode of target measured cell; - Update the autonomous gap capabilities as below: - Measure EUTRAN cell in NR SA (no TDD/FDD diff) - Measure EUTRAN cell in NR-DC (no TDD/FDD diff) - Measure EUTRAN cell in NE-DC (no TDD/FDD diff) Note: above capabilities are defined within MeasAndMobParametersCommon structure. - Measure NR cell in NR SA (with FR1/FR2 diff, no TDD/FDD diff) - Meausre NR cell in (NG)EN-DC (with FR1/FR2 diff, no TDD/FDD diff) - Measure NR cell in NR-DC (with FR1/FR2 diff, no TDD/FDD diff) - Measure NR cell in NE-DC (with FR1/FR2 diff, no TDD/FDD diff) Note: above capabilities are defined within MeasAndMobParametersFRX-Diff structure. Impact analysis Impacted 5G architecture options: NR SA, (NG)EN-DC, NE-DC, NR-DC Impacted functionality: UE capability for autonomous gap based CGI reading Inter-operability: - Update of Rel-16 autonomous gap related UE capabilities, since the ASN.1 change is non-backward compatible, if Rel-16 UE implements according to the CR and the network does not, or the network implements according to the CR and Rel-16 UE does not, there will be decoding failure issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Rel-16 autonomous gap based CGI reading capabilities are unclear and incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  | TS38.306 CR0368 |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | CR |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of changes

6.3.3	UE capability information elements

**** ignore non-related part ****

#### –	MeasAndMobParameters

The IE MeasAndMobParameters is used to convey UE capabilities related to measurements for radio resource management (RRM), radio link monitoring (RLM) and mobility (e.g. handover).

MeasAndMobParameters information element

-- ASN1START

-- TAG-MEASANDMOBPARAMETERS-START

MeasAndMobParameters ::=                    SEQUENCE {

measAndMobParametersCommon              MeasAndMobParametersCommon              OPTIONAL,

measAndMobParametersXDD-Diff                MeasAndMobParametersXDD-Diff        OPTIONAL,

measAndMobParametersFRX-Diff                MeasAndMobParametersFRX-Diff        OPTIONAL

}

MeasAndMobParametersCommon ::=          SEQUENCE {

supportedGapPattern                     BIT STRING (SIZE (22))                  OPTIONAL,

ssb-RLM                                 ENUMERATED {supported}                  OPTIONAL,

ssb-AndCSI-RS-RLM                       ENUMERATED {supported}                  OPTIONAL,

...,

[[

eventB-MeasAndReport                    ENUMERATED {supported}                  OPTIONAL,

handoverFDD-TDD                         ENUMERATED {supported}                  OPTIONAL,

eutra-CGI-Reporting                     ENUMERATED {supported}                  OPTIONAL,

nr-CGI-Reporting                        ENUMERATED {supported}                  OPTIONAL

]],

[[

independentGapConfig                    ENUMERATED {supported}                  OPTIONAL,

periodicEUTRA-MeasAndReport             ENUMERATED {supported}                  OPTIONAL,

handoverFR1-FR2                         ENUMERATED {supported}                  OPTIONAL,

maxNumberCSI-RS-RRM-RS-SINR             ENUMERATED {n4, n8, n16, n32, n64, n96} OPTIONAL

]],

[[

nr-CGI-Reporting-ENDC                   ENUMERATED {supported}                  OPTIONAL

]],

[[

eutra-CGI-Reporting-NEDC                ENUMERATED {supported}                  OPTIONAL,

eutra-CGI-Reporting-NRDC                ENUMERATED {supported}                  OPTIONAL,

nr-CGI-Reporting-NEDC                   ENUMERATED {supported}                  OPTIONAL,

nr-CGI-Reporting-NRDC                   ENUMERATED {supported}                  OPTIONAL

]],

[[

reportAddNeighMeasForPeriodic-r16       ENUMERATED {supported}                  OPTIONAL,

condHandoverParametersCommon-r16        SEQUENCE {

condHandoverFDD-TDD-r16                  ENUMERATED {supported}              OPTIONAL,

condHandoverFR1-FR2-r16                  ENUMERATED {supported}              OPTIONAL

}                                                                               OPTIONAL,

nr-NeedForGap-Reporting-r16             ENUMERATED {supported}                  OPTIONAL,

supportedGapPattern-NRonly-r16          BIT STRING (SIZE (10))                  OPTIONAL,

supportedGapPattern-NRonly-NEDC-r16     ENUMERATED {supported}                  OPTIONAL,

maxNumberCLI-RSSI-r16                   ENUMERATED {n8, n16, n32, n64}          OPTIONAL,

maxNumberCLI-SRS-RSRP-r16               ENUMERATED {n4, n8, n16, n32}           OPTIONAL,

maxNumberPerSlotCLI-SRS-RSRP-r16        ENUMERATED {n2, n4, n8}                 OPTIONAL,

mfbi-IAB-r16                            ENUMERATED {supported}                  OPTIONAL,

multipleNS-And-Pmax-IAB-r16             ENUMERATED {supported}                  OPTIONAL,

nr-CGI-Reporting-NPN-r16                ENUMERATED {supported}                  OPTIONAL,

idleInactiveEUTRA-MeasReport-r16        ENUMERATED {supported}                  OPTIONAL,

idleInactive-ValidityArea-r16           ENUMERATED {supported}                  OPTIONAL,

eutra-AutonomousGaps-r16                ENUMERATED {supported}                  OPTIONAL,

eutra-AutonomousGaps-NEDC-r16           ENUMERATED {supported}                  OPTIONAL,

eutra-AutonomousGaps-NRDC-r16           ENUMERATED {supported}                  OPTIONAL

]]

}

MeasAndMobParametersXDD-Diff ::=            SEQUENCE {

intraAndInterF-MeasAndReport        ENUMERATED {supported}                      OPTIONAL,

eventA-MeasAndReport                ENUMERATED {supported}                      OPTIONAL,

...,

[[

handoverInterF                      ENUMERATED {supported}                      OPTIONAL,

handoverLTE-EPC                     ENUMERATED {supported}                      OPTIONAL,

handoverLTE-5GC                     ENUMERATED {supported}                      OPTIONAL

]],

[[

sftd-MeasNR-Neigh                   ENUMERATED {supported}                      OPTIONAL,

sftd-MeasNR-Neigh-DRX               ENUMERATED {supported}                      OPTIONAL

]],

[[

condHandoverParametersXDD-Diff-r16  SEQUENCE {

condHandover-r16                    ENUMERATED {supported}                  OPTIONAL,

condHandoverFailure-r16             ENUMERATED {supported}                  OPTIONAL,

condHandoverTwoTriggerEvents-r16    ENUMERATED {supported}                  OPTIONAL

}                                                                               OPTIONAL,

pcellT312-r16                       ENUMERATED {supported}                      OPTIONAL,

handoverIntraF-IAB-r16              ENUMERATED {supported}                      OPTIONAL,

handoverUTRA-FDD-r16                ENUMERATED {supported}                      OPTIONAL

]]

}

MeasAndMobParametersFRX-Diff ::=            SEQUENCE {

ss-SINR-Meas                                    ENUMERATED {supported}              OPTIONAL,

csi-RSRP-AndRSRQ-MeasWithSSB                    ENUMERATED {supported}              OPTIONAL,

csi-RSRP-AndRSRQ-MeasWithoutSSB                 ENUMERATED {supported}              OPTIONAL,

csi-SINR-Meas                                   ENUMERATED {supported}              OPTIONAL,

csi-RS-RLM                                      ENUMERATED {supported}              OPTIONAL,

...,

[[

handoverInterF                                  ENUMERATED {supported}              OPTIONAL,

handoverLTE-EPC                                 ENUMERATED {supported}              OPTIONAL,

handoverLTE-5GC                                 ENUMERATED {supported}              OPTIONAL

]],

[[

maxNumberResource-CSI-RS-RLM                    ENUMERATED {n2, n4, n6, n8}         OPTIONAL

]],

[[

simultaneousRxDataSSB-DiffNumerology            ENUMERATED {supported}              OPTIONAL

]],

[[

nr-AutonomousGaps-r16                           ENUMERATED {supported}              OPTIONAL,

nr-AutonomousGaps-ENDC-r16                      ENUMERATED {supported}              OPTIONAL,

nr-AutonomousGaps-NEDC-r16                      ENUMERATED {supported}              OPTIONAL,

nr-AutonomousGaps-NRDC-r16                      ENUMERATED {supported}              OPTIONAL,

handoverUTRA-FDD-r16                            ENUMERATED {supported}              OPTIONAL,

cli-RSSI-Meas-r16                               ENUMERATED {supported}              OPTIONAL,

cli-SRS-RSRP-Meas-r16                           ENUMERATED {supported}              OPTIONAL,

condHandoverParametersFRX-Diff-r16              SEQUENCE {

condHandover-r16                                ENUMERATED {supported}          OPTIONAL,

condHandoverFailure-r16                         ENUMERATED {supported}          OPTIONAL,

condHandoverTwoTriggerEvents-r16                ENUMERATED {supported}          OPTIONAL

}                                                                                   OPTIONAL,

pcellT312-r16                                   ENUMERATED {supported}              OPTIONAL,

interFrequencyMeas-Nogap-r16                    ENUMERATED {supported}              OPTIONAL,

simultaneousRxDataSSB-DiffNumerology-Inter-r16  ENUMERATED {supported}              OPTIONAL,

handoverIntraF-IAB-r16                          ENUMERATED {supported}              OPTIONAL,

idleInactiveNR-MeasReport-r16                   ENUMERATED {supported}              OPTIONAL

]]

}

-- TAG-MEASANDMOBPARAMETERS-STOP

-- ASN1STOP

End of changes