3GPP TSG-RAN WG2 Meeting #111-e             	R2-2007041

E-Meeting, 17th Aug - 28th Aug 2020

Source:	vivo

Title:	Protocol stack and service continuity for L2 and L3 relay

Agenda Item:	8.7.3

Document for:	Discussion and Decision

- Introduction

A new SID [1] had been agreed in RAN#86 for study on NR sidelink relay, which has the following objectives:

- Study mechanism(s) with minimum specification impact to support the SA requirements for sidelink-based UE-to-network and UE-to-UE relay, focusing on the following aspects (if applicable) for layer-3 relay and layer-2 relay [RAN2]; - Relay (re-)selection criterion and procedure; - Relay/Remote UE authorization; - QoS for relaying functionality; - Service continuity; - Security of relayed connection after SA3 has provided its conclusions; - Impact on user plane protocol stack and control plane procedure, e.g., connection management of relayed connection; - Study mechanism(s) to support upper layer operations of discovery model/procedure for sidelink relaying, assuming no new physical layer channel / signal [RAN2];

In this contribution, we aim at analysis and comparisons on the above aspect F, C and D for L2 and L3 relay architectures.

- Discussion

  - Protocol stack

    - L3 Architecture

In LTE Release 13, there was a specified L3 sidelink relay function for public safety. And in LTE Release 10, L3 based relay node (RN) was been specified. Hence L3 NR sidelink relay architecture is not new to us. In R17 SA2 TR [2], a typical protocol stack for L3 sidelink relay from the SA2 perspective is shown in as Figure 1:

Figure 1 A typical protocol stack for L3 relay (Figure 6.6.1-2 in [2])

To avoid redundant discussion and clearly confirm the applicability of the above L3 relay protocol stack architecture  from RAN2 perspective, we propose that:

Proposal 1: The above RAN protocal stack can be the baseline for basic NR L3 sidelink relay.

In the above L3 relay architecture, as the remote UE does not have its own NAS connection and PDU sessions. Hence, from 3GPP point of view, it is difficult to guarantee service continuity.

Observation 1: In the basic L3 relay architecture (without N3IWF), there is neither NAS connection nor PDU session between the remote UE and CN, in which case service continuity is difficult from 3GPP point of view.

Beside this basic L3 Relay architecture, another solution for NR L3 sidelink relay architecture had been agreed and captured in [2], which is based on N3IWF (Non-3GPP InterWorking Function).  N3IWF based CP protocol stack and UP protocol stack are shown respectively in Figure 2 and Figure 3.

Figure 2 CP protocol stack for N3IWF based L3 relay (Figure 6.23.2-2 in [2])

Figure 3 UP protocol stack for N3IWF based L3 relay (Figure 6.23.2-3 in [2])

In the above N3IWF based L3 relay architecture, remote UE can establish and maintain its own NAS connection and PDU sessions with its CN nodes. Hence service continuity is possible.

Observation 2: In L3 relay architecture with N3IWF, there are both NAS connection and PDU session between the remote UE and CN, in which case service continuity is possible.

SA2 had also agreed that the N3IWF based solution is transparent to RAN. The NG-RAN (gNB) does not apply any different treatment to the Remote UE's traffic regardless of whether the L3 relay architecture is deployed with N3IWF or not. Thus, from RAN2 perspective, we think that there is common characteristic of these two L3 relay architectures (i.e. with/without N3IWF) that there is no need for Uu RRC connection between the remote UE and gNB.

Proposal 2: In case of L3 relay architecture, there is no need for Uu RRC connection between the remote UE and gNB regardless of whether N3IWF is used or not.

    - L3 Remote UE state and configuration

When a remote UE is transmitting traffic data with network via a L3 relay UE, its RRC state may be:

          - Out of coverage (OOC): when an OOC UE with SL-relay capability wants to trigger UL data transmission, it can find a L3 relay UE via SL-relay discovery procedure to help it establish connection and communicate with network. For DL data triggering, paging forwarding from relay UE to the remote OOC UE may be needed. From our view, this is a reasonable scenario. And even if the remote OOC UE is indirectly transmitting data with network, it is still in OOC state because it has neither RRC connection with network nor SIB acquirement from network. To follow the current NR V2X configuration rules, an OOC remote UE can only use pre-configuration message to decide PC5 configuration. Further optimizations are FFS, e.g. get configuration or SIB forwarding from relay UE via PC5 RRC.

          - RRC Idle state: when an Idle UE with SL-relay capability is in poor link quality, e.g. RSRP is lower than the configured threshold, it can trigger and discover a L3 relay UE for its data forwarding with network. From our view, this is also a reasonable scenario. And even if the remote Idle UE is indirectly transmitting data with network, it is still in Idle state because of no RRC connection with network, but can still acquire SIB & paging. To follow the current NR V2X configuration rules, an Idle remote UE can only use SIB message to decide PC5 configuration. Further optimizations are FFS, e.g. get configuration from relay UE via PC5 RRC.

          - RRC Connected state: for an RRC Connected UE, it is completely under network control. If it moves to the cell edge, it can be handed over to a target cell. If it moves out of the coverage, network will not control it any more. It is not a reasonable scenario that when a UE moves to the cell edge, its serving cell hands it over to a L3 relay UE. From another point of view, for an RRC Idle/Connected UE located in the cell edge or poor link quality, e.g. RSRP lower than a configured threshold, it needs further consideration whether the UE can establish CP-UP/UP-UP split paths, e.g. SRB via direct (i.e. Uu) link and DRB via indirect (i.e. relay) link, or eMBB service via direct link and public safety service via indirect link.

          - RRC Inactive state: for an RRC Inactive state UE, its SRB/DRB(s) are suspended and its CN connection, e.g. NAS connection and PDU session, are kept. In this state if it would establish another L3 relay link for communication with network via Relay UE’s CN connection, it is FFS whether these two connections and tunnels to CN are needed simultaneously, e.g. direct link and indirect link selection based on different service requirements.

From the above analysis, we propose that:

Proposal 3: In L3 relay architecture, the state of remote UE may be OOC (out of coverage) or IDLE, and FFS for Connected and Inactive.

Proposal 4: In L3 relay architecture, remote UE follows the current mechanisms to obtain the PC5 configuration, e.g. via pre-configuration message if OOC and via SIB if IDLE mode.

Furthermore, like current PC5 link configuration acquirement in NR R16 V2X, i.e. via pre-configuration for OOC UE and via SIB for Idle UE, if a remote UE also uses the same way to get PC5 link in a L3 relay architecture, it is necessary to discuss how to differentiate between legacy PC5 link and new PC5 link in L3 relay architecture. Since in legacy PC5 link, traffic data is transmitted between two UEs and QoS profile is based on this one hop PC5 link. But for the traffic data of the remote UE, QoS profile may be end-to-end requirement for the whole two-hops link. For example, for the same end-to-end QoS requirement of 100ms PDB (Packet Delay Budget) and 10^-3 PER (Packet Error Rate), it can choose a proper SLRB configuration for legacy V2X communication. But for PC5 link in a L3 relay architecture, another SLRB configuration may be needed to achieve 50ms PDB and 10^-4 PER in PC5 link (and achieve the rest 50ms PDB and 10^-4 PER in Uu link between relay UE and network) in order to guarantee end-to-end QoS requirements. Or upper layers need to split the end-to-end QoS requirements into two parts, i.e. one for PC5 link between remote to relay and another for Uu link between relay to network.

Proposal 5: In L3 relay architecture, a remote UE may obtain specific PC5 QoS parameters or PC5 configurations used for relayed services, which is different from ones for regular PC5 services, in order to guarantee the end-to-end (i.e. 2 hops) QoS requirement of relayed services.

    - L3 Remote UE mobility

According to the above analysis of remote UE state, remote UE may be OOC or Idle state in L3 relay architecture. Hence remote UE needs to perform Idle mode measurement in its Uu interface. And for its PC5 interface, in order to maintain communication via relay UE, PC5 link unicast measurement, e.g. legacy R16 PC5 unicast measurement or enhanced PC5 unicast measurement, is needed.

Proposal 6: In L3 relay architecture, remote UE may perform PC5 link unicast measurement and Idle mode measurement.

When a remote UE is moving, from the perspective of system resource efficiency, it should immediately hand over to a better link once the departure condition is satisfied, e.g. upon Uu link recovery or other better relay link detection. But from the perspective of service experience, remote UE power saving and even sharing charging, remote UE may except to keep the current relay link till service ends or interrupts as long as the quality of PC5 unicast link between remote UE and relay UE can fulfil basic communication requirement.

Proposal 7: In L3 relay architecture, there may be two methods of mobility for remote UE:

- Method 1: remote UE keeps the current relay link till service ends or interrupts as long as the quality of PC5 unicast link between remote UE and relay UE can fulfil basic communication requirement.

- Method 2: remote UE immediately hands over to a better link once the departure condition is satisfied, e.g. upon Uu link recovery or other better relay link detection.

    - L2 Architecture

In LTE Release 15, there was a study on FeD2D UE-to-NW relays for IoT and wearable, in which a typical L2 relay architecture had been proposed [3]. And in NR Release 16, L2 based IAB wireless backhaul has been specified. Hence, the concept of L2 NR sidelink relay architecture is also not a new one. In R17 SA2 TR [2], a typical protocol stack for L2 sidelink relay is shown from the SA perspective as Figure 4 and Figure 5:

Figure 4 User plane stack for L2 sidelink relay (Figure A.2.1-1 in [2])

Figure 5 Control plane stack for L2 sidelink relay (Figure A.2.2-1 in [2])

The above figures are similar to FeD2D L2 relay architecture. To avoid redundant discussion, we propose that:

Proposal 8: The above RAN protocal stacks can be the baseline for NR L2 sidelink relay.

In L2 relay architecture, SDAP and PDCP anchor of remote UE is located in gNB. And RRC connection between remote UE and gNB is needed, at least, to configure/manage these sublayers and to identify each remote UE. With similar reasons, both NAS connection and PDU session between remote UE and CN are needed.

Observation 3: In L2 relay architecture, there is not only Uu RRC connection between remote UE and gNB, but also NAS connection and PDU sessions between the remote UE and CN.

    - L2 Remote UE state and configuration

In L2 relay architecture, if a remote UE is transmitting data with network via a L2 relay UE, this remote UE is obviously an RRC Connected UE or at least an RRC Connected-like UE because of existence of RRC connection and availability of dedicated RRC signaling.

For a remote UE that does not establish RRC connection with gNB via a L2 relay UE but just needs some SIB or paging message forwarding by a L2 relay UE, e.g. it is OOC, its state is more like an Idle UE because of acquirement of SIB and DL reachability. But in order to support this scenario, registration/attach procedure, Idle mobility, TAU and so on need to be further studied for an OOC remote UE. Hence it is FFS for Idle mode remote UE in the first stage.

Observation 4: In L2 relay architecture, support of remote UE in Idle mode may require further study on how to support registration/attach procedure, Idle mobility, TAU and so on.

Proposal 9: In L2 relay architecture, the state of remote UE may be RRC connected and FFS for IDLE mode (e.g. for paging forwarding scenario).

Regarding on how a remote UE can obtain PC5 configuration for relayed service(s), there are two options:

- Option 1: Following current PC5 procedure, e.g. via sidelink UE information to request PC5 configuration for relayed service(s).

The interface between remote UE and relay UE is also PC5. In NR R16, when an RRC Connected UE wants to trigger PC5 service transmission, it will report its service information via Sidelink UE Information procedure for PC5 configurations. Hence it is a potential choice to reuse Sidelink UE information procedure for PC5 configurations in SL-Relay architecture.

- Option 2: Following Uu procedure, e.g. via NAS and NG procedure and after successful admission control.

In SL-Relay architecture, the final destination of remote UE’s data is to network and not just to Relay UE. In NR Uu, when a UE wants to trigger Uu service, it will report its requirement to CN via NAS procedure, e.g Service Request, and CN node informs gNB via N3 interface for resource reservation. Then after successful admission control, gNB will provide the UE with Uu configuration and resource necessary for the service(s). Since the remote UE triggers a Uu-like service to gNB, it should be configured with PC5 configuration and resources after CN and gNB admission control.

Proposal 10: In L2 relay architecture, there are two options for remote UE to request the PC5 configuration for relayed service(s):

- Option 1: Following current PC5 procedure, e.g. via sidelink UE information to request PC5 configuration.

- Option 2: Following Uu procedure, e.g. via NAS and following NG procedure and configured by gNB after successful admission control.

Furthermore, like a regular Uu service establishment procedure, CN provides end-to-end QoS requirement of remote UE’s service(s) to gNB via NG interface and gNB is responsible for ensuring that QoS requirement of a relayed service is satisfied in two-hop link, i.e. Uu interface and PC5 interface. For example, if a service in Uu interface will require 100ms PDB and 10^-3 PER, Uu interface and PC5 interface may be semi-statically allocated with 50ms PDB and 10^-4 PER respectively or dynamically shared with total 100ms PDB.

Proposal 11: In L2 relay architecture, CN provides end-to-end QoS requirement to gNB and gNB is responsible for ensuring the QoS requirement of a relayed service is satisfied in two-hop link, i.e. Uu interface and PC5 interface.

    - L2 Remote UE mobility

According to the above analysis of remote UE state, remote UE may be in Connected mode after L2 relay architecture establishment. Hence remote UE needs to perform Connected mode measurement in its Uu interface, i.e. configured by dedicated RRC. There are some differences between a regular Connected UE and a remote UE in L2 relay architecture, e.g. serving cell definition, current PC5 relay link measurement reporting and potential PC5 relay link measurement reporting and so on. And for its PC5 interface, in order to maintain communication via relay UE, PC5 link unicast measurement, e.g. legacy R16 PC5 unicast measurement or enhanced PC5 unicast measurement, is needed.

Proposal 12: In L2 relay architecture, remote UE may perform PC5 link unicast measurement and RRC configured measurement.

Since a remote UE is in RRC Connected mode, its mobility should be under control of gNB, e.g. handover from a direct link to a relay link when Uu link quality degrades and vice versa upon Uu link recovery, or handover to a better relay link.

Proposal 13: In L2 relay architecture, the mobility of remote UE is under control of its serving gNB, e.g. handover from the current relay link once the HO condition is satisfied, e.g. upon Uu link recovery or other better relay link detection.

    - Paging/SIB forwarding scenario

Furthermore, there is a special case for the relay scenario: no originated data transfer demand but only paging/SIB forwarding demand for an OOC UE or a UE with very poor link quality. In this scenario, the basic procedure may be as followings:

- The remote UE requests a suitable relay UE for paging/SIB relayed function, e.g. via discovery procedure;

- The relay UE monitors the paging message in the remote UE’s paging occasions and/ or acquires the required SIB(s);

- The relay UE forwards paging message and/or the SIB(s) to the remote UE;

For paging forwarding function, there is an obvious difference between L2 and L3 relay: visibility and reachability for an OOC remote UE. For a remote UE in L2 relay architecture, it can perform registration/attach procedure and TAU procedure via relay UE even if it is OOC. Hence it can always be visible and reachable from the perspective of network. Network may page it with legacy procedure. However, for a remote UE in L3 relay architecture, it can only perform registration/attach procedure and TAU procedure via itself Uu interface. Then it may request relay UE to forward its paging message and SIB(s) due to its poor Uu link quality. For an OOC L3 remote UE, it is not visible and reachable to network.

Proposal 14: RAN2 to discuss and study paging/SIB forwarding scenario for an OOC remote UE.

    - Comparisons and summary

According to the above analysis about protocol stacks and remote UE procedure, baseline comparisons and summary of connection management for a remote UE between L2 and L3 relay are highlighted in below Table 1.

Table 1 Comparisons on connection management of remote UE between L2 and L3 relay

|  | Remote UE in L3 Relay architecture | Remote UE in L2 Relay architecture |
| --- | --- | --- |
| RRC connection of remote UE | Only PC5 RRC | PC5 RRC + Uu RRC |
| Visible to gNB | No | Yes |
| Managed/Configured by gNB | No | Directly |
| PC5 link configuration | Pre-configuration for OOC or SIB for Idle | Uu RRC |
| Remote UE state | OOC or RRC Idle | RRC Connected |
| Measurement of remote UE | PC5 unicast measurement + Uu idle measurement | PC5 unicast measurement + Uu configured measurement |
| Mobility | PC5 link establish/release + Idle cell (re)selection | PC5 link establish/release + Connected HO |

Hence, we propose:

Proposal 15: RAN2 to discuss whether the above Table 1 can be baseline for a remote UE in L2 and L3 relay.

  - Service continuity

    - HO scenarios

There are three typical HO scenarios related to NR sidelink relay:

Scenario 1: Remote UE switching between relay link and direct link;

Scenario 2: Remote UE switching between two relay links;

Scenario 3: Relay UE HO with its remote UE(s) altogether;

The former two scenarios are remote UE mobility as following Figure 6. And scenario 3 is for relay UE mobility with remote UE together. From our point, these three scenarios are all considered.

Figure 6 Typical HO scenarios for remote UE

Hence, we propose:

Proposal 16: RAN2 to study the following three HO scenarios for sidelink relay:

- Scenario 1: Remote UE switching between relay link and direct link;

- Scenario 2: Remote UE switching between two relay links;

- Scenario 3: Relay UE HO with its remote UE(s) altogether;

    - Remote UE mobility in L3

In L3 architecture without N3IWF, service continuity for a remote UE can hardly be guaranteed since different link will have different CN connections, e.g. with remote UE’s AMF/UPF in direct link and with source/target relay UE’s AMF/UPF in indirect link. There are no common UP and CP anchors to achieve service continuity. However, in L3 architecture using N3IWF, remote UE will have its own NAS connection and PDU session with its AMF and UPF. Hence service continuity is possible.

Signalling procedure for remote UE switching from an indirect link to a direct link with N3IWF L3 architecture is illustrated as Figure 7.

Figure 7 Remote UE switching example for L3 using N3IWF

In the above procedures, relay UE acts as a role of source node to achieve service continuity for remote UE. Since the peer PDCP entity of remote UE is located in source relay UE, source relay UE may perform SN Status Transfer and Data Forwarding procedure to achieve strict lossless service.

Observation 5: In scenario 1&2, i.e. remote UE mobility, L3 relay architecture with N3IWF may provide service continuity for remote UE.

    - Remote UE mobility in L2

In L2 relay architecture, remote UE has its own RRC connection and CN connection. Hence the service continuity is similar with current Uu procedure. The following Figure 8 shows an example of signalling procedure for a remote UE switching from an indirect link to a direct link.

Figure 8 Remote UE switching example for L2

In the above procedure, gNB can directly control remote UE to hand over via dedicated RRC signaling. Moreover, the peer PDCP entity of remote UE is located in gNB, current PDCP data recovery procedure can be used for lossless data transfer. Compared to L3, service continuity for remote UE in L2 architecture is easier to guarantee with legacy mechanisms.

Observation 6: In scenario 1&2, i.e. remote UE mobility, L2 relay architecture can provide service continuity for remote UE.

    - Relay UE mobility

Scenario 3 is a typical case for wearable device or personal device, where mobile phone is the relay UE and other portable device(s) are remote UE(s) and they move together shown as the following Figure 9.

Figure 9 Relay UE HO with remote UE together

For this scenario, in L3 architecture, there is only one handover procedure for the relay UE and the remote UE(s), which can be shown in Figure 10 as an example.

Figure 10 Relay UE HO with remote UE together in L3

However, in L2 relay architecture, scenario 3 will trigger simultaneous multiple handover procedures, e.g. one procedure for one UE including not only relay UE but also remote UE(s). Signalling overhead and service interruption time will be higher than L3 architecture. Moreover, it needs further study on how to guarantee multiple UEs synchronous handover to the target.

Proposal 17: In scenario 3, i.e. relay UE HO with remote UE(s) altogether, L3 relay architecture has simpler signaling procedure and lower service interruption &signaling overhead than L2 relay.

- Conclusion

In this contribution, we give analysis and comparisons on protocol stack, QoS and service continuity for L2 and L3 relay. Based on the discussion, we have the following observations and proposals:

Observation 1: In the basic L3 relay architecture (without N3IWF), there is neither NAS connection nor PDU session between the remote UE and CN, in which case service continuity is difficult from 3GPP point of view.

Observation 2: In L3 relay architecture with N3IWF, there are both NAS connection and PDU session between the remote UE and CN, in which case service continuity is possible.

Observation 3: In L2 relay architecture, there is not only Uu RRC connection between remote UE and gNB, but also NAS connection and PDU sessions between the remote UE and CN.

Observation 4: In L2 relay architecture, support of remote UE in Idle mode may require further study on how to support registration/attach procedure, Idle mobility, TAU and so on.

Observation 5: In scenario 1&2, i.e. remote UE mobility, L3 relay architecture with N3IWF may provide service continuity for remote UE.

Observation 6: In scenario 1&2, i.e. remote UE mobility, L2 relay architecture can provide service continuity for remote UE.

And we propose:

Proposal 1: The above RAN protocal stack can be the baseline for basic NR L3 sidelink relay.

Proposal 2: In case of L3 relay architecture, there is no need for Uu RRC connection between the remote UE and gNB regardless of whether N3IWF is used or not.

Proposal 3: In L3 relay architecture, the state of remote UE may be OOC (out of coverage) or IDLE, and FFS for Connected and Inactive.

Proposal 4: In L3 relay architecture, remote UE follows the current mechanisms to obtain the PC5 configuration, e.g. via pre-configuration message if OOC and via SIB if IDLE mode.

Proposal 5: In L3 relay architecture, a remote UE may obtain specific PC5 QoS parameters or PC5 configurations used for relayed services, which is different from ones for regular PC5 services, in order to guarantee the end-to-end (i.e. 2 hops) QoS requirement of relayed services.

Proposal 6: In L3 relay architecture, remote UE may perform PC5 link unicast measurement and Idle mode measurement.

Proposal 7: In L3 relay architecture, there may be two methods of mobility for remote UE:

- Method 1: remote UE keeps the current relay link till service ends or interrupts as long as the quality of PC5 unicast link between remote UE and relay UE can fulfil basic communication requirement.

- Method 2: remote UE immediately hands over to a better link once the departure condition is satisfied, e.g. upon Uu link recovery or other better relay link detection.

Proposal 8: The above RAN protocal stacks can be the baseline for NR L2 sidelink relay.

Proposal 9: In L2 relay architecture, the state of remote UE may be RRC connected and FFS for IDLE mode (e.g. for paging forwarding scenario).

Proposal 10: In L2 relay architecture, there are two options for remote UE to request the PC5 configuration for relayed service(s):

- Option 1: Following current PC5 procedure, e.g. via sidelink UE information to request PC5 configuration.

- Option 2: Following Uu procedure, e.g. via NAS and following NG procedure and configured by gNB after successful admission control.

Proposal 11: In L2 relay architecture, CN provides end-to-end QoS requirement to gNB and gNB is responsible for ensuring the QoS requirement of a relayed service is satisfied in two-hop link, i.e. Uu interface and PC5 interface.

Proposal 12: In L2 relay architecture, remote UE may perform PC5 link unicast measurement and RRC configured measurement.

Proposal 13: In L2 relay architecture, the mobility of remote UE is under control of its serving gNB, e.g. handover from the current relay link once the HO condition is satisfied, e.g. upon Uu link recovery or other better relay link detection.

Proposal 14: RAN2 to discuss and study paging/SIB forwarding scenario for an OOC remote UE.

Proposal 15: RAN2 to discuss whether the above Table 1 can be baseline for a remote UE in L2 and L3 relay.

Proposal 16: RAN2 to study the following three HO scenarios for sidelink relay:

- Scenario 1: Remote UE switching between relay link and direct link;

- Scenario 2: Remote UE switching between two relay links;

- Scenario 3: Relay UE HO with its remote UE(s) altogether;

Proposal 17: In scenario 3, i.e. relay UE HO with remote UE(s) altogether, L3 relay architecture has simpler signaling procedure and lower service interruption &signaling overhead than L2 relay.

- Reference

- RP-193253, New SID: Study on NR sidelink relay, RAN#86, OPPO;

- TR23.752, Study on system enhancement for Proximity based Services (ProSe) in the 5G System (5GS) (Release 17), v0.4.0, 2020-06;

- TR36.746, Study on further enhancements to LTE Device to Device (D2D), User Equipment (UE) to network relays for Internet of Things (IoT) and wearables; (Release 15), v15.1.1, 2018-04;