3GPP TSG RAN WG2 #111-e			R2-2007106

e-Meeting, August 17th – 28th, 2020

Agenda Item:	6.5.3

Source:	Apple

Title:	Clarifications on Intra-UE Prioritization – Capability and Clarification

Document for:	Discussion/Decision

# Introduction

For the intra-UE prioritization feature, in RAN2#109bis-e [1] the following agreement was reached

R2 assumes that PHY-based prioritization and LCH-based prioritization are configured independently and one can be configured without the other (assumption may be modified when LS reply from R1 is received)

In RAN2#110e [2], no consensus was reached on

Proposal 1: Decide on the dependency between PHY -based prioritization and LCH-based prioritization capabilities after confirming whether they can be configured separately and after receiving further RAN1 input on IIOT capabilities.

In this contribution, we request clarifications in terms of capability and configuration of both these features and request RAN2 to revisit this discussion for closure.

# Discussion

LCH based prioritization in 38.306 [3] has the following capability parameters.

| MAC Parameter | Description |
| --- | --- |
| autonomousTransmission-r16 | autonomousTransmission-r16 Indicates whether the UE supports autonomous transmission of the MAC PDU generated for a deprioritized configured uplink grant as specified in TS 38.321 [8]. A UE supporting this feature shall also support lch-priorityBasedPrioritization-r16. |
| lch-PriorityBasedPrioritization-r16 | Indicates whether the UE supports prioritization between overlapping grants and between scheduling request and overlapping grants based on LCH priority as specified in TS 38.321 [8]. |

No reference exists to phy-LayerPrioritization-r16 [3]either exclusively or in relation to lch-PriorityBasedPrioritization-r16.

Observation1: phy-LayerPrioritization-r16 does not exist in the current UE capability list.

Proposal 1: RAN2 to clarify on the capability requirements for phy-LayerPrioritization-r16 and its dependency on lch-PriorityBasedPrioritization-r16.

Current RAN1 Feature on UL intra-UE multiplexing/prioritization [4] of overlapping channel/signals with two priority levels in physical layer.

## Configuration

Observation 2: No explicit configuration indications are available to enable PHY based prioritization.

Proposal 2: RAN2 to confirm implicit or explicit RRC Configuration based enablement and parameter indication for PHY based prioritization.

One potential implicit indication for PHY based prioritization as suggested in R2-2004586 [5] the parameters of which can also be explicitly indicated in a RRC message.

This should ensure the following cases of LCH and PHY prioritization enablement/disablement are taken care of.

| # | LCH Prioritization State | PHY Prioritization State |
| --- | --- | --- |
| 1 | Enabled | Enabled |
| 2 | Enabled | Disabled |
| 3 | Disabled | Enabled |
| 4 | Disabled | Disabled |

# Conclusion

In this contribution, we provided our views on timing advance enhancements for NTN. Our observations and proposals are as follows:

Observation1: phy-LayerPrioritization-r16 does not exist in the current UE capability list.

Proposal 1: RAN2 to clarify on the capability requirements for phy-LayerPrioritization-r16 and its dependency on lch-PriorityBasedPrioritization-r16.

Observation 2: No explicit configuration indications are available to enable PHY based prioritization.

Proposal 2: RAN2 to confirm implicit or explicit RRC Configuration based enablement and parameter indication for PHY based prioritization.

# References

- Chairman’s notes, RAN2#109bis-e, May 2020

- Chairman’s notes, RAN2#110-e, Jul. 2020

- 3GPP TS 38.306, User Equipment (UE) Radio Access Capabilities, v. 16.1.0, Jul. 2020.

- R1-2003073, RAN1 UE features list for Rel-16 NR after RAN1#100bis-E, AT&T, NTT DOCOMO, INC.

- R2-2004586, Impacts of independent configuration of intra-UE prioritization in PHY and MAC Layers, RAN2#110-e, CATT