3GPP TSG-RAN WG2 Meeting #111-e	R2-2007494

Online, August 17th - 28th, 2020

Agenda Item:	8.12.3

Souce:	MediaTek Inc.

Title:	eDRX for reduced capability UEs

Document for:	Discussion and decision

# 1 Introduction

The SID on Support of Reduced Capability NR Devices [1] includes the following objectives on power saving:

Study UE power saving and battery lifetime enhancement for reduced capability UEs in applicable use cases (e.g. delay tolerant) [RAN2, RAN1]: - Reduced PDCCH monitoring by smaller numbers of blind decodes and CCE limits [RAN1]. - Extended DRX for RRC Inactive and/or Idle [RAN2] - RRM relaxation for stationary devices [RAN2]

In this contribution, we discuss the motivation and principles for applying the extended DRX to RedCap UEs in NR and provide our views on the way forward.

# 2 Discussion

In the RedCap SID, the following use cases were identified for the study item:

- Industrial wireless sensors

- Video Surveillance

- Wearables

Battery life of industrial wireless sensors is an important consideration, with the expectation that the battery should last at least a few years to avoid the need for frequent maintenance. These devices are expected to be stationary or with little movement, and its data is mostly uplink-centric and delay tolerant. Normally sensors would not be paged by the network frequently. Moreover, the amount of data to be transmitted or received is expected to be small, therefore the devices are not expected to stay in RRC Connected mode for a long period of time. The dominant contribution to the power consumption comes from monitoring paging occasions in Idle and/or Inactive modes.

For wearables, the battery is expected to last multiple days (up to a few weeks). This target is already difficult to achieve for most smartphone devices. Given that these devices have a smaller form factor that smartphones, with smaller battery capacities, achieving a longer battery life requires enhancements beyond those available for smartphones today.

In order to achieve the battery life targets for these devices, enhancements for Idle/Inactive mode DRX are necessary. In Release 16, the maximum paging cycle that can be configured for NR is 256 radio frames, i.e. 2.56 seconds. On the other hand, with eDRX in LTE/NB-IoT, the paging cycle has been extended beyond 2.56, up to 10485.76 seconds in LTE/NB-IoT. For use-cases that have limited RRC connected mode operation and are delay-tolerant or are uplink-centric, a reduction in Idle mode power consumption by reducing paging monitoring frequency will result in significant battery life improvements.

Observation 1: Introducing eDRX to NR helps improve battery life for industrial wireless sensors and wearables use cases.

In LTE MTC and NB-IoT, extended DRX (eDRX) has been introduced as a power saving mechanism since Release 13. eDRX works by extending the time interval during which the UE does not monitor the paging messages. The cost of reduced monitoring is a delay in reaching the UE in Idle or Inactive mode, when there is pending downlink data. For devices and use cases that expect delay-tolerant downlink traffic, this delay can be acceptable.

In eDRX, the UE monitors for paging during a time interval called paging time window (PTW). The length of the PTW can be up to 40.96 sec for NB-IoT. During the PTW, the UE monitors the paging according to its legacy idle mode DRX (I-DRX) configuration. The PTW is repeated in cycles of TeDRX. TeDRX can span multiple hyper-frames (HFNs), each consisting of 1024 radio frames. For the Inactive mode, the maximum value for the RAN paging cycle has been extended from 256 to 1024 radio frames with eDRX. A typical eDRX timeline for Idle mode is illustrated in Figure 1 below.

Figure 1 Example eDRX timeline

Unlike I-DRX, there is no common (default) eDRX configuration that is broadcast by the network and shared by multiple UEs; only UE specific eDRX configuration is possible. The UE specific eDRX configuration can be signalled to the UE during the registration procedure for initial registration or the registration procedure for mobility and periodic registration update [7].

In order to evaluate the additional power savings that could be achieved by introducing eDRX in NR compared to legacy I-DRX, we use a model based on [8], scaled to 20MHz for Idle mode operation. We consider two scenarios: 1) High SINR, and 2) Low SINR, as illustrated below:

Figure 2 Timeline for I-DRX with high SINR

Figure 3 Timeline for I-DRX with low SINR

Relative power during various states can be modelled as below:

Table 1 Power state modelling for I-DRX

| Component Description | Power notation | Relative power | Time notation | Time (ms) |
| --- | --- | --- | --- | --- |
| SSB processing | PSSB | 50 | TSSB | 2/4 |
| Intra-frequency neighbor cell measurement | PIntraF | 60 | TIntraF | 2 |
| Paging occasion reception | PPO | 50/120 (without/ with PDSCH) | TPO | 1/4 (high/ low SINR) |
| Inter-frequency neighbor cell measurement | PInter | 60 | TInterF | 5 |
| Micro sleep | PMS | 45 | TMS | (*) |
| Light sleep | PLS | 20 | TLS | (*) |
| Deep sleep | PDS | 1 | TDS | (*) |

(*) The value depends on the power saving scenario adopted

We also define the following energy consumption for the state transition.

| Component Description | Energy notation | Energy | Occurrence notation |
| --- | --- | --- | --- |
| Micro sleep transition | EMST | 0 | NMST |
| Light sleep transition | ELST | 100 | NLST |
| Deep sleep transition | EDST | 450 | NDST |

Based on the above timeline and power model, the power consumption for I-DRX with high SINR is given by:

PIDRX, High SINR=PSSB×TSSB+PPO×TPO+PDS×(TIDRX-TSSB-TPO)+EDST×NDSTTIDRX

For low SINR, it is given by:

PIDRX,  Low SINR=PSSB×TSSB+PIntraF×TIntraF+PPO×TPO+PInterF×TInterF+PLS×TLS+ELST×NLST+PDS×TIDRX-TSSB-TIntraF-TPO-TInterF-TLS+EDST×NDSTTIDRX

For eDRX, if we consider the UE to be in deep sleep outside of PTW and consuming power PIDRX (in the formula above) during PTW, the formula for eDRX power consumption becomes:

PeDRX=PIDRX×LPTW+PDS×(TeDRX-LPTW)TeDRX

Where, LPTW is the PTW length.

Some example power savings by introducing eDRX, with different eDRX/I-DRX configurations are summarised in the table below:

Table 2 Example power savings that can be achieved with eDRX

| Scenario | TI-DRX (ms) | TeDRX (ms) | PTW length (ms) | % Savings with eDRX compared to I-DRX |
| --- | --- | --- | --- | --- |
| High SINR | 2560 | 10,485,760 | 2560 | 29.02 |
| High SINR | 1280 | 10,485,760 | 1280 | 44.99 |
| High SINR | 320 | 10,485,760 | 320 | 76.59 |
| Low SINR | 2560 | 10,485,760 | 2560 | 51.35 |
| Low SINR | 1280 | 10,485,760 | 1280 | 67.86 |
| Low SINR | 320 | 10,485,760 | 320 | 89.41 |

The above figures are based on the assumption that the deep sleep power factor PDS used in I-DRX (= 1) also applies when the UE does not monitor the paging outside the PTW in eDRX (in Idle mode). We expect that the actual power factor when the UE goes to sleep in eDRX to be considerably smaller than PDS in I-DRX (i.e. << 1) due to the longer sleep period. Therefore the gains shown above should be seen as extremely conservative estimates, with real life power saving gains expected to be significantly higher.

Given the power savings gains achievable, eDRX concepts and mechanisms such as PTW and extension of paging cycles to hyper-frames that were introduced for LTE/NB-IoT should be re-used in RedCap. Further details such as the exact formulas for determining the PTW and any potential impacts on other procedures such as system information modification can be discussed during the work item phase.

Note that 5G core network (5GC) already supports the required NAS procedures and signalling for eDRX, in Registration Request (in Requested extended DRX parameters) and Registration Accept (in Negotiated extended DRX parameters) messages [7]. Therefore introducing eDRX for RedCap in NR is expected to have little or no impact on the 5GC core network specifications. However to be on the safe side, we think that RAN2 should send an LS to SA2 to inform them of the recommendation to support eDRX in NR for RedCap UEs and to check if there are any potential concerns.

Proposal 1: eDRX mechanisms in LTE/NB-IoT such as PTW and HFN are used as the baseline for introducing eDRX in RedCap. The exact formulas and impact on other procedures can be discussed during the work item phase.

Proposal 2: RAN2 to send an LS to SA2 with recommendation to introduce eDRX, to check if there are any potential issues for RedCap from 5GC perspective.

# 3 Conclusion

We have the following observation and proposals for introducing eDRX in NR for RedCap:

Observation 1: Introducing eDRX UEs can be beneficial for industrial wireless sensors and wearables use cases in RedCap.

Proposal 1: eDRX mechanisms in LTE/NB-IoT such as PTW and HFN are used as the baseline for introducing eDRX in RedCap. The exact formulas and impact on other procedures can be discussed during the work item phase.

Proposal 2: RAN2 to send an LS to SA2 with recommendation to introduce eDRX, to check if there are any potential issues for RedCap from 5GC perspective.

# 4 References

- RP-193238 New SID on support of reduced capability NR devices

- RP-192160 Summary of email discussion on NR-Light, Ericsson

- TS 36.304 User Equipment (UE) procedures in idle mode

- TS 36.331 Radio Resource Control (RRC)

- TS 38.304 User Equipment (UE) procedures in Idle mode and RRC Inactive state

- TS 38.331 Radio Resource Control (RRC) protocol specification

- TS 24.501 Non-Access-Stratum (NAS) protocol for 5G System (5GS)

- TR 38.840 Study on User Equipment (UE) power saving in NR