3GPP TSG-WG2 Meeting #111e	R2-2007535

# e-Meeting, 17th August – 28th August 2020

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.300 | CR | 1303 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to cell selection for IAB in NSA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB-Core |  |  |  |  |  |  | Date: |  | 2020-08-03 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | iab-support indicator is introduced in eNB system information broadcasting for IAB-node working in ENDC. However there is no description of IAB-MT specific cell selection/reselection operation. This explanation should be different with the 38.331 from the following aspects: - since iab-support indicator is also applicable to cell reselection, this cell reselection aspect also need to be added in this section. - There is no consideration of SNPN in 36.300. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the same contents as the IAB-MT specific cell selection in 9.2.1.1 of TS 38.300 with the following differences: - Add (re-) into cell selection description. - Remove SNPN part. Impact analysis Architecture options EN-DC Impacted functionality: Operations in cell (re)selection of IAB node |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is no way to understand cell selection/reselection of IAB-MT in stage 2 level. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

CHANGE START

10.1.1.1	Cell selection

The principles of PLMN selection in E-UTRA are based on the 3GPP PLMN selection principles. Cell selection is required on transition from EMM_DETACHED to EMM-REGISTERED and from ECM-IDLE or ECM-CONNECTED.

Cell selection:

-	The UE NAS layer identifies a selected PLMN and equivalent PLMNs;

-	The UE searches the E-UTRA frequency bands and for each carrier frequency identifies the strongest cell. It reads cell system information broadcast to identify its PLMN(s):

-	The UE may search each carrier in turn ("initial cell selection") or make use of stored information to shorten the search ("stored information cell selection").

-	The UE seeks to identify a suitable cell; if it is not able to identify a suitable cell it seeks to identify an acceptable cell. When a suitable cell is found or if only an acceptable cell is found it camps on that cell and commence the cell reselection procedure:

-	A suitable cell is one for which the measured cell attributes satisfy the cell selection criteria; the cell PLMN is the selected PLMN, registered or an equivalent PLMN; the cell is not barred or reserved and the cell is not part of a tracking area which is in the list of "forbidden tracking areas for roaming";

-	An acceptable cell is one for which the measured cell attributes satisfy the cell selection criteria and the cell is not barred.

-	The IAB-MT applies the cell selection procedure as described for the UE with the following differences:

-	The IAB-MT ignores cell-barring or cell-reservation indications contained in cell system information broadcast;

-	The IAB-MT only considers a cell as a candidate for cell (re-)selection if the cell system information broadcast indicates IAB support for the selected PLMN.

Transition to RRC_IDLE:

On transition from RRC_CONNECTED to RRC_IDLE, a UE should camp on the last cell for which it was in RRC_CONNECTED or a cell/any cell of set of cells or frequency be assigned by RRC in the state transition message.

Recovery from out of coverage:

The UE should attempt to find a suitable cell in the manner described for stored information or initial cell selection above. If no suitable cell is found on any frequency or RAT the UE should attempt to find an acceptable cell.

CHANGE END