3GPP TSG-WG2 Meeting #111-e	R2-2007567

# 17 – 28 August 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.304 | CR | 0810 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Group WUS corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eMTC5-Core, NB_IOTenh3-Core |  |  |  |  |  | Date: |  |  | 2020-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Some aspects of group WUS description are difficult to follow. Structure of the conditional text is difficult to follow as well as text duplication. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change ‘time offset’ to ‘timeoffset’ to be consistent with legacy text. Remove unnecessary intermediate variable. Change ‘WUS resource’ to ‘WUS Resource’ when referring to a specific WUS resource. Change WG to WG’ to make it easier to distinguish from wg. Clean-up bullet text layout. Impact analysis Impacted functionality: Group WUS Inter-operability: If the network is implemented according to the CR but UE is not, no interoperability problems are foreseen. If the UE is implemented according to the CR but UE is not, no interoperability problems are foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Description of group WUS remains unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5.1, 7.5.2, 7.5.3, 7.5.4, 7.5.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of first change

### 7.5.1	General

When all of the following conditions are meet then the UE shall monitor GWUS using the GWUS parameters provided in system information.

-	the UE supports GWUS and GWUS configuration (gwus-Config) is provided in system information;

-	(groupAlternation is present in gwus-Config and UE supports groupWakeUpSignalAlternation) or groupAlternation is not present in gwus-Config.

A UE supporting GWUS can be configured to monitor a WUS group and a common WUS. Upon detecting either of them, UE shall monitor POs as defined in clause 7.4.

For NB-IoT, E-UTRAN may configure up to 2 WUS resources (numbered 0 and 1). The timeoffset, g0, from the end of WUS Resource 0 to the start of corresponding PO is determined as defined in clause 7.4. When both wus-Config and gwus-Config are present, WUS Resource 0 shares radio resources with wus-Config.The timeoffset from the end of WUS Resource 1 to the start of corresponding PO is sum of the timeoffset g0 and the maximum WUS duration.

After the UE has determined the applicable gap between end of WUS resource and associated PO as specified in clause 7.4, UE selects the WUS group set for the corresponding gap as specified in clause 7.5.2. From the selected WUS group set, UE selects one WUS group as defined in clause 7.5.3. If groupAlternation is not present in gwus-Config, the UE monitors the selected WUS group with the corresponding timeoffset for each PO. If groupAlternation is present in gwus-Config and UE supports groupWakeUpSignalAlternation, the UE determines the WUS group to monitor for each PO and the corresponding timeoffset as specified in clause 7.5.4.

For BL UEs and UEs in enhanced coverage, E-UTRAN may configure up to 4 WUS resources. The resource number, time and frequency location of these resources is determined as specified in clause 7.5.5.

Start of next change

### 7.5.2	WUS group sets selection

The total number of WUS groups, maxWG, configured for a gap is determined with the following equation:

maxWG= i=0maxWR-1maxWGnumGroupsListi

where:

- maxWR is the total number of WUS resources configured for the gap.

- numGroupsList[i] is the number of WUS groups configured for WUS Resource i, provided in gwus-Config, for the gap.

Using numGroupsList for the gap, the UE builds the list of WUS groups as an ordered list of pairs (NIDresource, NgroupWUS) where the first entry corresponds to the first WUS group on the first configured WUS resource and the last entry corresponds to the last WUS group on the last configured WUS resource.

For a NB-IoT UE, if resourcePosition provided in gwus-Config is set to secondary, NIDresource = 0 is not used and the first entry in the list corresponds to NIDresource = 1. Otherwise, NIDresource is the index of the WUS resources in numGroupsList.

For a BL UE or UE in enhanced coverage, UE determines NIDresourceof the configured resources as specified in clause 7.5.4.

If probThreshList is present in gwus-Config, UE determines the WUS group sets as defined in Table 7.5.2.1. The total number of WUS group sets is equal to the number of entries in probThreshList + 1. The WUS groups are first assigned to WUS group set 1, followed by WUS group set 2, and so on. The UE determines the WUS group set corresponding to its probability PNAS, if configured, as defined in Table 7.5.2-1. If PNAS is not configured UE selects the WUS group set with highest index that has at least one WUS group (i.e. Upper bound – Lower bound > 0).

Table 7.5.2-1: WUS group set definition when probThreshList is configured

| WUS group set | probThreshList | WUS group index in WUS groups list |  |
| --- | --- | --- | --- |
|  |  | Lower bound | Upper bound |
| 1 | PNAS ≤ Thresh1 | 0 | Nth1 -1 |
| 2 | Thresh1 < PNAS ≤ Thresh2 (Note) | Nth1 | Nth1 + Nth2 -1 |
| 3 | Thresh2 < PNAS ≤ Thresh3 (Note) | Nth1 + Nth2 | Nth1 +Nth2 + Nth3 -1 |
| 4 | PNAS > Thresh3 | Nth1 +Nth2 + Nth3 | maxWG-1 |
| where Threshi is the value signalled in the ith entry of probThreshList Nthi is the value signalled in the ith entry of groupsForServiceList Note: When the total number of WUS group sets is less than 4, the upper bound for the WUS group set with highest index is maxWG-1. |  |  |  |

If probThreshList is not present in gwus-Config, there is only one WUS group set containing all the WUS groups configured in numGroupsList. The total number of WUS groups is maxWG.

Start of next change

### 7.5.3	WUS group selection

After selection of the WUS group set as specified in clause 7.5.2, the UE selects the WUS group to monitor as below.

For BL UE or UE in enhanced coverage, the UE determines wg with following equation:

wg=floorfloorUE_IDN×NsNn mod Nw

For NB-IoT, the UE determines wg with following equation:

wg=floorUE_IDN×Ns×W mod Nw

where:

- UE_ID, N, Ns, Nn and W are defined in clause 7.1.

- Nw is the number of WUS groups in the selected WUS group set.

- wg is the index of the WUS group in the selected WUS group set.

If probThreshList is not present (i.e. in this case all WUS groups belong to the same WUS group set) then:

WG’ = wg

If probThreshList is present, the UE determines WG’, the index of the corresponding WUS group within the WUS groups list of the selected WUS group set, as defined in Table 7.5.3-1.

Table 7.5.3-1: Index of the WUS group to monitor

| Selected WUS group set | WG’ |
| --- | --- |
| 1 | wg |
| 2 | wg + Nth1 |
| 3 | wg + Nth1 + Nth2 |
| 4 | wg + Nth1 + Nth2 + Nth3 |
| Where Nthi is defined in table 7.5.1 |  |

The entry corresponding to WG’ in the WUS groups list defined in clause 7.5.2 provides (NIDresource, NgroupWUS) as specified in TS 36.213 [6].

Start of next change

### 7.5.4	WUS Group Alternation

When WUS group alternation is used in the cell then one of two schemes for WUS group alternation is used depending on gwus-Config configuration. In first scheme a set of  WUS groups alternate togher at each PO; while in the second scheme entire WUS groups in a WUS resource alternate at each PO.

If groupAlternation is present in gwus-Config:

-	if probThreshList is not present in gwus-Config and commonSequence is set to g0, the UE determines the WUS group to monitor for the current PO as follows:

WGcurrent=WGinitial+ Gmin·divSFN+1024HSFNTcellmod maxWG

NIDresourceNgroupWUS)

-	else, the UE determines the WUS group to monitor for the current PO as follows:

mcurrent=minitial+divSFN+1024HSFNTcellmod maxWR

where:

-	Tcell is the default DRX cycle for the cell.

-	maxWR is the total number of WUS resources configured in numGroupsList for the gap.

-	Gmin is the lowest number of WUS groups configured amongst all WUS resources for the gap.

-	WGcurrent is the index of the WUS group to monitor for the current PO. The entry corresponding to WGcurrent in the WUS groups list defined in clause 7.5.2 provides (NIDresource, NgroupWUS) as specified in TS 36.213 [6].

-	WGinitial is the index, WG’, of the WUS group determined in clause 7.5.3.

-	minitial is defined based on NIDresource given in the entry corresponding to the index WG’ determined in clause 7.5.3:

-	For a NB-IoT UE : minitial =NIDresource

-	For a BL UE or UE in enhanced coverage:

-	if NIDresource= 0 is configured to be used for GWUS:

-	minitial =NIDresource - 1, where NIDresource

-	else:

-	minitial =NIDresource , where NIDresource

-	mcurrent is used to determine NIDresource of the WUS group to monitor for the current PO as follows:

-	For a NB-IoT UE : NIDresource = mcurrent

-	For a BL UE or UE in enhanced coverage:

-	if NIDresource=0 is configured to be used for GWUS:

-	NIDresource = mcurrent

-	else:

-	NIDresource = mcurrent +1

NgroupWUS of the WUS group to monitor for the current PO is given in the entry corresponding to the index WG’ determined in clause 7.5.3.

Start of next change

### 7.5.5	WUS Resource Location for BL UEs and UEs in Enhanced coverage

A BL UE or UE in enhanced coverage determines the time/frequency location of WUS resources based on the number of configured WUS resources and the frequency location of WUS Resource 0 (NIDresource=0). If wus-Config is present, frequency location for WUS Resource 0 is defined by frequencyLocation parameter in wus-Config. Otherwise, frequency location for WUS Resource 0 is defined by resourceLocationWithoutWUS in gwus-Config. The frequency location of other WUS resources (i.e., WUS Resource 1, 2, 3), based on frequency location of WUS Resource 0, is given in Table 7.5.5-1.

Table 7.5.5-1: WUS Resource frequency location

| WUS Resource (NIDresource) | Frequency location of WUS Resource ID 0 |  |  |  |
| --- | --- | --- | --- | --- |
|  | n0 | n2 |  | n4 (NOTE 1) |
|  |  | NB frequency < centre frequency | NB frequency > centre frequency |  |
| WUS Resource 1,3 | n2 | n4 | n0 | n2 |
| WUS Resource 2 | n0 | n2 | n2 | n4 |
| WUS Resource 2 (NOTE 2) | n4 | n0 | n4 | n0 |
| NOTE 1: This column is applicable if wus-Config is present. NOTE 2: This row is applicable if resourceLocationWithWUS is primary3FDM. |  |  |  |  |

The timeoffset, g0, from the end of WUS Resource 0 and WUS Resource 1 to the start of corresponding PO is determined as defined in clause 7.4. Except when resourceLocationWithWUS is set to primary3FDM , the timeoffset from the end of WUS Resource 2 and WUS Resource 3 to the start of corresponding PO is sum of the timeoffset g0 and the maximum WUS duration. When resourceLocationWithWUS is set to primary3FDM, the timeoffset for WUS Resource 2 is same as WUS Resource 0 and 1.

The resource pattern ID (rp-ID) which indicates the WUS Resources applicable for WUS Groups is derived based on resourceMappingPattern and the configured number of WUS resources as follows:

If resourceLocationWithWUS is configured:

rp-ID = 2*(maxWR – 1) if resourceLocationWithWUS is set to primary

rp-ID= 2*maxWR - 1 if resourceLocationWithWUS is set to secondary

rp-ID=7 if resourceLocationWithWUS is set to primary3FDM.

If resourceLocationWithoutWUS is configured:

rp-ID = 2*(maxWR-1)

where maxWR is the total number of WUS resources configured in numGroupsList for the gap.

The WUS resource IDs corresponding to the resource pattern ID are determined as defined in Table 7.5.5-2.

Table 7.5.5-2: WUS Resources applicable for Resource Pattern

|  |  | Resource Pattern ID |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| WUS Resource (NIDresource) | 0 | X |  | X |  | X |  | X | X |
|  | 1 |  | X | X | X | X | X | X | X |
|  | 2 |  |  |  | X | X | X | X | X |
|  | 3 |  |  |  |  |  | X | X |  |

If NIDresource = 0 is not used, the first entry in the numGroupsList corresponds to NIDresource = 1. Otherwise, NIDresource is the index of the WUS resources in numGroupsList.

End of change