3GPP TSG-RAN WG2 Meeting #111 Electronic	R2-2007640

# Elbonia, 17 – 28 August 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 0831 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Sidelink type 2 groupcast reception upon member ID |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2020-08-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Give the reasons for change: - In RAN2#109b-e it was agreed that groupcast option 2 can only be selected when, amongst others, the member ID has been passed from the V2X to the AS layer. This is reflected correctly in the transmitting HARQ entity section 5.22.1.3, but the agreement should also be reflected for the receiving entity. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Explain the changes: - When determining whether to send HARQ ACK if type 1 groupcast is not indicated by the SCI, it is now also ensured that the UE has been provided the member ID by the V2X layer in case type 2 groupcast is selected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE may send HARQ ACK even though member ID has not been provided, and the group membership may not be known |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.22.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Modified Subclause

##### 5.22.2.2.2	Sidelink process

For each PSSCH duration where a transmission takes place for the Sidelink process, one TB and the associated HARQ information is received from the Sidelink HARQ Entity.

For each received TB and associated Sidelink transmission information, the Sidelink process shall:

1>	if this is a new transmission:

2>	attempt to decode the received data.

1>	else if this is a retransmission:

2>	if the data for this TB has not yet been successfully decoded:

3>	instruct the physical layer to combine the received data with the data currently in the soft buffer for this TB and attempt to decode the combined data.

1>	if the data which the MAC entity attempted to decode was successfully decoded for this TB; or

1>	if the data for this TB was successfully decoded before:

2>	if this is the first successful decoding of the data for this TB:

3>	if this TB is associated to unicast, the DST field of the decoded MAC PDU subheader is equal to the 8 MSB of any of the Source Layer-2 ID(s) of the UE for which the 16 LSB are equal to the Destination ID in the corresponding SCI, and the SRC field of the decoded MAC PDU subheader is equal to the 16 MSB of any of the Destination Layer-2 ID(s) of the UE for which the 8 LSB are equal to the Source ID in the corresponding SCI; or

3>	if this TB is associated to groupcast or broadcast and the DST field of the decoded MAC PDU subheader is equal to the 8 MSB of any of the Destination Layer-2 ID(s) of the UE for which the 16 LSB are equal to the Destination ID in the corresponding SCI:

4>	deliver the decoded MAC PDU to the disassembly and demultiplexing entity;

2>	consider the Sidelink process as unoccupied.

1>	else:

2>	instruct the physical layer to replace the data in the soft buffer for this TB with the data which the MAC entity attempted to decode.

1>	if HARQ feedback is enabled by the SCI:

2>	if type 1 groupcast is indicated by the SCI according to clause 8.4.1 of TS 38.212 [9]:

3>	if UE’s location information is available and distance beteween UE’s location and the central location of the nearest zone indicated by the Zone_id in the SCI is smaller or equal to the communication range requirement in the SCI; or

3>	if UE’s location information is not available:

4>	if the data which the MAC entity attempted to decode was not successfully decoded for this TB or the data for this TB was not successfully decoded before:

5>	instruct the physical layer to generate a negative acknowledgement of the data in this TB.

2> if type 1 groupcast is not indicated by the SCI according to clause 8.4.1 of TS 38.212 [9]:

3> if the data which the MAC entity attempted to decode was successfully decoded for this TB or the data for this TB was successfully decoded before; and

3>	if type 2 groupcast is indicated by the SCI according to clause 8.4.1 of TS 38.212 [9] and the group member ID is provided by upper layer

4>	instruct the physical layer to generate a positive acknowledgement of the data in this TB.

3>	else:

4>	instruct the physical layer to generate a negative acknowledgement of the data in this TB.

Next Modified Subclause