3GPP TSG-RAN WG2 Meeting #111-e	R2-2007709

# Electronic Meeting, 17th – 28th Aug 2020

| CR--Form--v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | 4405 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Compliance check failure for CPC configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_feMob-Core |  |  |  |  |  | Date: |  |  | 2020-07-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to the current RRC spec, all stored CPC configuration will be released upon PCell change. For the PCell change without SN involved, since the SN is not aware of the PCell change, the SN has no idea whether the stored CPC configuration has been released or not on the UE side, which may cause the configuration mismatching between the SN side and the UE side. After PCell change without SN involved, if the SN wants to modify the previous CPC configuration, a compliance check failure may happen due to the CPC configuration mismatching between the SN side and the UE side, which shall cause the UE to trigger the undesired recovery procedure (e.g. SCG failure information procedure or RRC re-establishment procedure). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a note in 5.3.5.5 to specify that the UE ignores the compliance check failure for an RRCReconfiguration message received as part of ConditionalReconfiguration for CPC after successful completion of PCell change if CPC was configured, i.e. the UE does not initiate the SCG failure information procedure or the connection re-establishment procedure. Impact analysis Impacted architecture options: LTE, (NG)EN-DC Impacted functionality: CHO Inter-operability: - If the network is implemented according to the CR and the UE is not, in case the SN sends a RRCReconfiguration message to modify the previous CPC configuration after PCell change, the UE might initiate unnecessary SCG failure information procedure or RRC re-establishment procedure due to the compliance check failure for CPC configuration, which shall introduce additional interruption delay. - If the UE is implemented according to the CR and the network is not, there is no inter-operability problem. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The unnecessary recovery procedure may be triggered due to the compliance check failure for CPC configuration, which shall introduce additional interruption delay. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.5.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

CHANGE START

#### 5.3.5.5	Reconfiguration failure

The UE shall:

1>	if the UE is unable to comply with (part of) the configuration included in the RRCConnectionReconfiguration message or if the upper layers indicate that the nas-Container is invalid:

2>	continue using the configuration used prior to the reception of RRCConnectionReconfiguration message;

2>	if the UE is in NE-DC:

3>	perform the actions as specified in TS 38.331 [82], clause 5.3.7;

2>	else if security has not been activated:

3>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause other;

2>	else:

3>	initiate the connection re-establishment procedure as specified in 5.3.7, upon which the connection reconfiguration procedure ends;

NOTE 1:	The UE may apply above failure handling also in case the RRCConnectionReconfiguration message causes a protocol error for which the generic error handling as defined in 5.7 specifies that the UE shall ignore the message.

NOTE 2:	If the UE is unable to comply with part of the configuration, it does not apply any part of the configuration, i.e. there is no partial success/ failure.

NOTE 3:	The compliance also covers the NR configuration carried within octet strings e.g. field nr-SecondaryCellGroupConfig. I.e. the failure behaviour defined also applies in case the UE cannot comply with the NR configuration or with the combination of (parts of) the LTE and NR configurations.

NOTE 4:	The UE ignores the compliance check failure for an RRCConnectionReconfiguration message received as part of conditionalReconfiguration for CPC after successful completion of PCell change if CPC was configured, i.e. the UE does not initiate the SCG failure information procedure or the connection re-establishment procedure.

CHANGE END