3GPP TSG-RAN WG2 Meeting #111 electronic	R2-2007734

# Online, August 17th - 28th, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.324 | CR | 0018 | rev |  | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on PC5 QoS flow remapping |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ASUSTeK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2020-08-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | REL-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - In RAN2#108 meeting, the following agreement was made: “SDAP Rx behaviour upon remapping is left to UE implementation for insequence delivery. And it will not be captured in the specification.”. However, according to the current specification in subclause 5.2.4, a UE will always deliver the retrieved SDAP SDUs to the upper layer, which would cause reordering issue if PC5 QoS flow remapping occurs. We think it is RAN2’s responsibility to make sure people would implement a UE correctly when following the 3GPP specification. Therefore, in our opinion, at least a NOTE should be added in subclause 5.2.4 to indicate the above agreement (Option 1). Preferably, Rx UE behavior upon remapping is specified for completeness (Option 2). RAN2 is kindly requested to discuss and make the decision. - The text “control” is missing in term of End-Marker control PDU in sublause 6.1.2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Change for PC5 QoS flow remapping in subclause 5.2.4: Option 1: A NOTE is added in subclause 5.2.4 to indicate the agreement, i.e. “NOTE: SDAP Rx behaviour upon remapping is left to UE implementation for insequence delivery.” Option 2: After a PC5 QoS flow is remapped from the previously-mapped SL-DRB to the current SL-DRB, the UE shall not deliver the retrieved SDAP SDU to the upper layer until an End-Marker control PDU for the PC5 QoS flow is received. - The missing text “control” is added in sub-clause 6.1.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Reordering issue would occur if the UE is implemented based on the current procedural text. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.4, 6.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First Change

### Option 1:

### 5.2.4	SL reception

At the reception of an SDAP data PDU from lower layers for a PC5 QoS flow, the receiving SDAP entity shall:

-	if the SL-DRB from which this SDAP data PDU is received is configured by RRC (TS 38.331 [3]) with the presence of SDAP header:

-	retrieve the SDAP SDU from the SL SDAP data PDU as specified in the clause 6.2.2.4;

-	else:

-	retrieve the SDAP SDU from the SL SDAP data PDU as specified in the clause 6.2.2.1;

-	deliver the retrieved SDAP SDU to the upper layer.

NOTE:	SDAP Rx behaviour upon remapping is left to UE implementation for insequence delivery.

### Option 2:

### 5.2.4	SL reception

At the reception of an SDAP data PDU from lower layers for a PC5 QoS flow, the receiving SDAP entity shall:

-	if the SL-DRB from which this SDAP data PDU is received is configured by RRC (TS 38.331 [3]) with the presence of SDAP header:

-	retrieve the SDAP SDU from the SL SDAP data PDU as specified in the clause 6.2.2.4;

-	else:

-	retrieve the SDAP SDU from the SL SDAP data PDU as specified in the clause 6.2.2.1;

-	if the SL-DRB is configured by RRC (TS 38.331 [3]) with the presence of SDAP header and the End-Marker control PDU of the PC5 QoS flow has not been received yet on the previously-mapped SL-DRB after the PC5 QoS flow is remapped from the previously-mapped SL-DRB to the SL-DRB:

-	store the retrieved SDAP SDU.

-	else:

-	deliver the retrieved SDAP SDU to the upper layer.

At the reception of an End-Marker control PDU from lower layers for a PC5 QoS flow, the receiving SDAP entity shall:

-	deliver the stored SDAP SDU(s) for the PC5 QoS flow to the upper layer, if any.

Start of Second Change

### 6.1.2	Control PDU

a) End-Marker Control PDU

End-Marker control PDU is used by the SDAP entity at UE to indicate that it stops the mapping of the SDAP SDU of the QoS flow indicated by the QFI/PFI to the DRB/SL-DRB on which the End-Marker control PDU is transmitted.

End of Changes