3GPP TSG-RAN WG2 # 111- e 	R2-2007781

# Electronic meeting, 17th August - 28th August, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | 0385 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on inter-RAT MRO feature |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-Core |  |  |  |  |  | Date: |  |  | 2020-08-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - It is agreed in email discussion [AT110-e][802] that one optional feature is used to indicate the reconnected RLF and RLF report for inter-RAT MRO feature, which is aligned with the procedure part specified in TS 38.331 while in TS 38.306 they are separated into two optional features. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Reconnected RLF report for inter-RAT MRO EUTRA and Radio link report for inter-RAT MRO features are merged into one feature. Impact analysis Impacted 5G architecture options: Standalone, EN-DC Impacted functionality: The CR has impact on UE capability reporting. Inter-operability: - There will be misunderstanding between NW and UE on UE’s capability to support inter-RAT MRO feature. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The inter-RAT MRO feature descried in TS 38.306 is not align with the procedure part described in TS 38.331. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change:

------------------------------- [Start of change] -------------------------

## 5.2	UE receiver features

Definitions for feature

SU-MIMO Interference Mitigation advanced receiver - R-ML (reduced complexity ML) receivers with enhanced inter-stream interference suppression for SU-MIMO transmissions with rank 2 with 2 RX antennas - R-ML (reduced complexity ML) receivers with enhanced inter-stream interference suppression for SU-MIMO transmissions with rank 2, 3, and 4 with 4 RX antennas UE supporting the feature is required to meet the Enhanced Receiver Type requirements in TS 38.101-4 [18].

Relaxed measurement Indicates whether the UE supports relaxed RRM measurements of neighbour cells in RRC_IDLE/RRC_INACTIVE as specified in TS 38.304 [21].

Mobility history information storage It is optional for UE to support the storage of mobility history information and the reporting in UEInformationResponse message as specified in TS 38.331 [9]. UE is not required to report this capability.

Cross RAT RLF Report Indicates whether the UE supports delivery of EUTRA RLF report to an NR node upon request from the network. UE is not required to report this capability.

Radio Link Failure Report for inter-RAT MRO EUTRA Indicates whether the UE supports: - include EUTRA CGI, if available, or otherwise include the physical cell identity and carrier frequency of the target PCell of the failed handover, and associated TAC as failedPCellId in RLF-Report as specified in TS 38.331 [9]. - include EUTRA CGI and associated TAC as previousPCellId in RLF-Report as specified in TS 38.331 [9]. - include eutra-CellIdentity in reconnectionCellIdentity in the VarRLF-Report upon UE has radio link failure or handover failure and successfully re-connected to an E-UTRA cell as specified in TS 38.331 [9].

------------------------------- [End of change] --------------------------