3GPP TSG-RAN WG2 Meeting #111-e	R2-2007858

Electronic, 17th – 28th Aug, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 0842 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the HARQ-based Sidelink RLF detection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2020-08-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - maxNumConsecutiveDTX needs to be aligned with ASN.1. - The initialization of counter numConsecutiveDTX should be performed upon establishement of each PC5-RRC connection, which is missing in the current specification. - The increment of numConsecutiveDTX is ambiguious in the current specification, which should be clarified as ‘increment by 1’ rather than ‘by 2’ or others. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In 5.22.1.3.3, change numConsecutiveDTX into sl-MaxNumConsecutiveDTX - In 5.22.1.3.3, clarify that numConsecutiveDTX, which is maintained for each PC5-RRC connection is initiated as zero once the PC5-RRC connection is established. - In 5.22.1.3.3, add ‘by 1’ after ‘increment numConsecutiveDTX’ Impact analysis Impacted functionality: HARQ-based Sidelink RLF Inter-operability: - If the UE is implemented according to this CR but the network is not, there is no inter-operability issue foreseen, because the CR does not impact the Uu signalling. - If the network is implemented according to this CR but the UE is not, there is no inter-operability issue foreseen, because the CR does not impact the Uu signalling. - If one UE is implemented according to this CR but the other UE is not, there is no inter-operability issue foreseen, because the CR does not impact the PC5-RRC signalling. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NumConsecutiveDTX cannot be initiated and incremented propoerly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.22.1.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--------------------- [Start of change] ----------------------------------

##### 5.22.1.3.3	HARQ-based Sidelink RLF detection

The HARQ-based Sidelink RLF detection procedure is used to detect Sidelink RLF based on a number of consecutive DTX on PSFCH reception occasions for a PC5-RRC connection.

RRC configures the following parameter to control HARQ-based Sidelink RLF detection:

-	sl-MaxNumConsecutiveDTX.

The following UE variable is used for HARQ-based Sidelink RLF detection.

-	numConsecutiveDTX, which is maintained for each PC5-RRC connection and initiated as zero once the PC5-RRC connection is established.

The Sidelink HARQ Entity shall (re-)initialize numConsecutiveDTX to zero for each PC5-RRC connection which has been established by upper layers, if any, upon (re)configuration of sl-MaxNumConsecutiveDTX.

The Sidelink HARQ Entity shall for each PSFCH reception occasion associated to the PSSCH transmission:

1>	if PSFCH reception is absent on the PSFCH reception occasion:

2>	increment numConsecutiveDTX by 1;

2>	if numConsecutiveDTX reaches sl-MaxNumConsecutiveDTX:

3>	indicate HARQ-based Sidelink RLF detection to upper layers.

1>	else:

2>	re-initialize numConsecutiveDTX to zero.

--------------------------[End of change] ------------------------------