3GPP TSG-RAN WG2 Meeting #111 Electronic	R2-2007945

# Elbonia, 17 – 28 August 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | 0397 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | FR2 inter-RAT measurement gap requirement indication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core, TEI16 |  |  |  |  |  | Date: |  |  | 2020-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | When UE operating on the E-UTRA, the FR2 inter-RAT measurement gap requirement can be indicated via two fields (i.e. interRAT-NeedForGapsNR and independentGapConfig). interRAT-NeedForGapsNR-r16 This field defines for each supported E-UTRA band or band combination whether measurement gaps are required to perform SSB based inter-RAT measurements on each supported NR band.. Or independentGapConfig This field indicates whether the UE supports two independent measurement gap configurations for FR1 and FR2 specified in clause 9.1.2 of TS 38.133 [5]. The field also indicates whether the UE supports the FR2 inter-RAT measurement without gaps when (NG)EN-DC is not configured.. However, if interRAT-NeedForGapsNR is present for measured FR2 band on top of independentGapConfig, it is not clear for network which field should be referred in order to determine the inter-RAT measurement gap requirement. Two alternative solutions are proposed: Option1: If gaps requirement for measured FR2 bands are indicated both in independentGapConfig and interRAT-NeedForGapsNR, the UE shall set the same gap requirement for FR2 bands in both fields. Option2: If gaps requirement for measured FR2 bands are indicated both in independentGapConfig and interRAT-NeedForGapsNR, the network should refer to interRAT-NeedForGapsNR instead of independentGapConfig to determine the gap requirements for the indicated measured band per operating E-UTRA band or band combinations. Considering interRAT-NeedForGapsNR can be reported on each meausured FR2 band, it is reasonable to support Option2 to enable gap requirement report with finer granularity. |
| Summary of change: |  | Clarify indication of FR2 inter-RAT measurement gap requirement if interRAT-NeedForGapsNR is present for measured FR2 band on top of independentGapConfig. The network should refer to interRAT-NeedForGapsNR instead of independentGapConfig to determine the gap requirements for the indicated measured band per operating E-UTRA band or band combinations. Impact analysis Impacted functionality: FR2 Measurement Gap requirement. Inter-operability: If the network is implemented according to the CR and the UE is not or vice versa, the measurement gap requirement may not aligned in NW and UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The FR2 inter-RAT measurement gap requirement is ambiguous between network and UE, which will cause system performance downgrade. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Modified Subclause

### 4.2.9	MeasAndMobParameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| independentGapConfig This field indicates whether the UE supports two independent measurement gap configurations for FR1 and FR2 specified in clause 9.1.2 of TS 38.133 [5]. The field also indicates whether the UE supports the FR2 inter-RAT measurement without gaps when (NG)EN-DC is not configured. If interRAT-NeedForGapsNR is present for measured FR2 band per operating E-UTRA band or band combinations, the network should refer to interRAT-NeedForGapsNR instead of this field to determine the gap requirements for the indicated measured band per operating E-UTRA band or band combinations. | UE | No | No | No |

Next Modified Subclause